3GPP TSG-RAN WG4 Meeting #107	R4-2308673

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on RRM requirements for Rel-18 NTN

Source: 	Huawei, HiSilicon

Agenda item:	8.27.5

Document for:	Discussion

Introduction

RRM impacts for NTN enhancements are discussed in RAN4#106 and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Coverage enhancement

- NTN in above 10GHz

- NW verified UE location

- Cell reselection enhancements

- HO enhancements

In this paper we will provide our views on RRM impacts for NTN enhancements.

Discussion

Coverage enhancement

RAN4 received LS [1] from RAN1 related to coverage enhancement.

Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4

Based on RAN1 WA, UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. This may have some impact on the UE timing requirements because in Rel-17 timing pre-compensation is assumed for every UL transmission. If UE does not perform timing pre-compensation in a window, UE may not meet the timing requirements for all slots in the window. This would impact the applicability of the timing requirements.

In our view, the situation is similar to segment based UL transmission in IoT NTN. The difference is that in IoT NTN UE would always not perform timing pre-compensation during a segment, while in NR NTN may or may not perform timing pre-compensation depending on whether it causes phase discontinuity that may violate the phase difference limit. We are open to how to capture this condition in the requirements.

Proposal 1: RAN4 to discuss the possible impact of “not performing TA pre-compensation update within an TDW” on UE timing requirements.

NTN in above 10GHz

Issue 2-1: Numerologies in NR-NTN above 10 GHz bands Agreement: - RAN4 to consider the following numerologies for RRM requirement definition for NR-NTN above 10 GHz bands: - Consider SSB SCS larger than 30kHz, i.e. 120kHz and 240kHz - Consider UL SCS larger than 30kHz, i.e. 60kHz and 120kHz - RAN4 to send an LS to RAN1 and RAN2 Issue 2-2: UE UL Timing Accuracy Requirements for higher UL SCS than 30kHz in NR-NTN above 10 GHz bands Agreement: - The assumption of the maximum total positioning error due to UE location and Satellite position estimation error shall be tightened compared to the assumption of the existing Rel-17 NR NTN. - The exact values and required conditions will be further discussed/determined in the future meetings. - Alternatives can be further discussed, e.g. ECP (only for 60kHz SCS), limiting UL SCS, limiting SSB SCS, limiting UE mobility, etc. - Note: the above “maximum total positioning error due to UE location and Satellite position estimation error” will not be specified in the requirement but it’s only used as an assumption to derive the UE Tx timing requirement.

RAN4 agrees to tighten the assumption on UE location and satellite position estimation error for defining the timing requirements for 60kHz and 120kHz UL SCS.

On GNSS error, we suggest to re-use the requirements in 38.171. In clause 6.2.1 the nominal accuracy under ideal GNSS condition is 15m, which is for non-mobility scenario. For mobility scenario, in clause 6.5.1 the accuracy is defined as 50m, and we suggest to take it as baseline.

On satellite position estimation error, it depends on the accuracy of the ephemeris information itself and how frequent gNB/UE would update the estimation (validity timer of the ephemeris information). Assuming ephemeris information update period <= 10s, a satellite position estimation error of 25m can be considered.

Proposal 2: RAN4 to consider the following assumptions for timing accuracy for NTN in Ka band

- GNSS error: [15]m for non-mobility scenario, [50]m for mobility scenario

- Satellite location error: [25]m assuming ephemeris information update period <= [10]s

With Proposal 2, the timing error and the ratio compared to CP length is shown in Table 1.

Table 1: Timing error with tightened assumption and ratio compared to CP

Considering the channel delay spread and TA adjustment resolution, it would be preferable if Te can be well less than 50% of CP. It can be seen that the resulted Te with

- 60kHz SCS with NCP/ECP in non-mobility scenario and

- 60kHz SCS with ECP in mobility scenario

can meet such criterion, while for other cases Te is larger than desired.

One alternative proposed is to limit the UL SCS (to 60kHz). While this is beneficial from timing point of view, the maximum residual frequency error that can be handled with 60kHz SCS is smaller compared to 120kHz SCS. Also, 60kHz is less robust to phase noise compared to 120kHz which may limit the achievable throughout.

Therefore, we suggest RAN4 to further discuss the feasibility of 60kHz and 120kHz SCS considering the impacts of timing/frequency accuracy and UE mobility. Another alternative RAN4 may consider is to introduce ECP for 120kHz SCS which provide good balance between all consideration factors.

Proposal 3: RAN4 to further discuss the feasibility of 60kHz and 120kHz SCS for NTN operation in Ka band considering the impacts of timing/frequency accuracy and UE mobility.

Proposal 4: RAN4 considers to introduce ECP for 120kHz SCS.

Comparing TN RRM requirements for FR1 and FR2, another outstanding difference is the use of Rx beam and more specifically for RRM, the consideration on Rx beam sweeping in measurements and mobility requirements. We think Rx beam sweeping also needs to be assumed for NTN in Ka bands. In last meeting some companies mentioned that for NGSO scenario the sweeping factor of 8 as in TN may result too long measurement delay and cause mobility issue. We think the concern is valid and we are open to discuss the how to account for Rx beamforming in RRM requirements, also considering different implementation options, e.g. phase antenna array or parabolic antenna.

Proposal 5: RAN4 to discuss how to account for Rx beamforming in the measurement and mobility requirements for NTN in Ka band, considering both parabolic antenna and phase array antenna.

In last meeting the following was agreed in main session [3].

Issue 1-1: UE types - Encourage companies to provide the regulation differences leading to different RF/RRM requirements between the mobile and fixed NTN UEs, and further discuss whether RAN4 needs to define the different UE types. - Discuss possible differentiation of NTN UE types in above 10 GHz, based on RF parameters (noise figure, antenna types, antenna gain and transmission power, or alternatively minimum EIRP in the peak direction, Effective Isotropic Sensitivity EIS, etc.). - The impact on RF and RRM requirements will be investigated, and analysis on RF requirement impact is prioritized.

While RF is discussing possible differentiation of UE types for mobile and fixed UEs based on RF parameters, RRM also needs to discuss whether and how to differentiate RRM requirements for mobile and fixed UEs. In our view, such differentiation may be meaningful, e.g. as discussed above, the timing accuracy and the mobility requirements (UE Rx beam tracking) may be different for mobile and fixed UEs.

Proposal 6: RAN4 to discuss possible differentiation of RRM requirements for mobile and fixed UEs.

NW verified UE location

RAN#98 approved inclusion of NW verified UE location as an additional objective. The objective is to enable multi-RTT positioning with a single satellite, and in our view, from RAN4 RRM perspective, RAN4 needs to define requirements for delay and accuracy requirements for UE Rx-Tx measurement.

Existing TN requirements can be used as baseline. Of course, some specifics for NTN can be discussed and may need to be based on RAN1 inputs, e.g. the positioning is based on a single moving satellite, so the interval between two measurements may need to be controlled. The measurement sample and processing capability also need to be discussed in RAN1 first. In last meeting, some companies raised the issue of HO, and we think it is a valid issue and are open to discuss further.

RAN4 may also need to discuss the side condition for NTN positioning. Based on our evaluation, the Es/Iot for DL measurement will be above 0dB, but we are open to further discussions.

Proposal 7: RAN4 to define measurement period and accuracy requirements for UE Rx-Tx measurement.

- Existing TN requirements can be used as baseline.

- NTN specific aspects can be further discussed based on RAN1 progress

- Side condition is FFS

Another area RAN4 needs to pay attention is the report mapping for UE and gNB Rx-Tx. In current spec, the reporting range is from -0.5ms to +0.5ms. This is sufficient for TN positioning, but for NTN the distance between UE and TRP (satellite) is quite large. RAN1 is discussing how to define the measurement report to enable LMF to know the distance between UE and satellite. RAN4 should wait for RAN1 conclusion before discussing whether to update UE/gNB Rx-Tx report mapping.

Proposal 8: RAN4 waits for RAN1 conclusion on whether updates to UE/gNB Rx-Tx report mapping.

Cell reselection enhancements

NTN-NTN cell reselection

Issue 4-1-A: Time-based cell reselection in earth moving cell NTN deployments - No agreement.

RAN2 agreed the following related to time based measurement triggering.

- For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case.

- Time-based cell reselection criteria is not pursued in R18.

As the details including the signalling and the how the measurement is to be triggered have not been agreed in RAN2, we suggest RAN4 to wait further progress in RAN2 before discussing the requirements.

Proposal 9: RAN4 to discuss whether and how to define requirements for time triggered cell reselection measurement based on RAN2 progress.

Issue 4-1-B: For location-based cell reselection in earth moving cell NTN deployments Agreement: - For location-based cell reselection in earth moving cell NTN deployments: - Whether the coverage information of serving cell is (absolutely) necessary: - No consensus in the group on whether serving cell coverage information is absolutely necessary. - Consider further progress and conclusion from RAN2, if any - Whether and to what extent restrict the use of the values of DRX cycle: - FFS: Do not further restrict DRX cycle beyond Rel-17 NR NTN.

RAN2 agreed the following related to location based measurement triggering.

- RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements.

- For earth-moving cell, new IE is introduced to indicate the reference location of serving cell.

- For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19.

In Rel-17 RAN4 has defined location based cell reselection measurement requirements for earth fixed cell:

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

Since RAN2 agreed that the location-based cell measurement rules of quasi-fixed cell is reused for earth moving cell, the requirements of earth fixed cell can also be re-used as baseline. The applicability or margin of the location based triggering (50m for fixed cell) may need further discussion because the reference location is now also moving and UE needs to estimate the motion trajectory.

Proposal 10: Location based cell reselection measurement requirements for earth fixed cell are re-used for earth moving cell as baseline. FFS the margin for applying the distance threshold.

In last meeting some companies suggest to further restrict applicable DRX cycle for moving cell.

In Rel-17 RAN4 already defined that 2.56s DRX cycle is not applicable for moving cell scenario. We do not support to define further restriction on top of Rel-17 one. The coverage duration in LEO moving cell can vary depending on the NW deployment and the covered area on earth (e.g. low or high latitude), and can be up to several mins. Further restricting the applicable DRX cycle will reduce the NW flexibility, which is not preferred. In any case, it is up to NW to decide which DRX cycle to use.

Proposal 11: Do not further restrict DRX cycle beyond Rel-17 NR NTN for cell reselection in earth moving cell.

Issue 4-1-C: whether and how to manipulate Kcarrier in Ttrigger - No agreement. Further discussion in May meeting for “earth moving cell” and “quasi-earth fixed cell.”

In last meeting some companies suggest to define additional requirements related to Ttrigger.

In our understanding, the proposal is to define prioritization of measurements when the time from t0 to t-Service is shorter than current Ttrigger, where t0 is the time when UE receives SI related to t-Serivce. For example, UE should prioritize measuring the detected cells or performing cell search on the intra-frequency layer in such a case.

The proposed prioritization or optimization can already be done by UE implementation without spec change. With the additional requirements, such prioritization will be mandated. Although we can understand the motivation, we suggest to de-prioritize this optimization. On one hand, this optimization is not included in the WI scope, and there are already a lot of mobility related enhancements being discussed in RAN2 and RAN4, in addition to other objectives which may have large RAN4 impacts. On the other hand, it is preferred to leave the flexibility to UE implementation to decide which cells or layers are prioritized in the concerned scenario.

Proposal 12: De-prioritize additional requirements related to Ttrigger.

NTN-TN cell reselection

Issue 4-2: NTN-TN Cell reselection enhancements (to reduce UE power consumption) - No agreement. Wait for further progress from RAN2. Companies can provide further analyses in the next meeting as usual. The analyses and discussions may depend on the following aspects: - The direction of cell reselection, e.g. from TN to NTN, from NTN to TN - The type of satellites, e.g. GEO, GSO, NGSO - The type of NTN cell deployment, e.g. earth fixed cell vs. earth moving cell - Accuracy of TN coverage information provided by NTN cell. - Whether assistance information of neighbour NTN cells ( e.g. ephemeris data, frequency layer and PCI) provided by serving TN cell is mandatory in terms of RRM requirement definition/applicability

RAN2 has agreed the framework of TN coverage information.

1.	For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful

RAN4 should define NTN-TN cell reselection requirements considering the availability and inaccuracy of the TN coverage. As agreed in RAN2, UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage

In last meeting some companies proposed to study the measurement behaviour to deal with inaccuracy of coverage information. We think this is a valid issue. The intention to introduce TN coverage information is to enable UE power saving, so some fallback mechanism should be considered when the TN coverage information is not accurate which is not a corner case considering the complexity to describe the actual TN coverage and the fact that RAN2 signaling is only to provide center location + radius.

Proposal 13: If necessary, RAN4 to define NTN-TN cell reselection requirements considering

- UE is not required to measure TN frequency in an area there is no TN coverage

- UE is allowed to relax or stop measuring the TN frequency if no TN neighbor cell is detected for a time period, details are FFS.

In last meeting, some companies propose to revisit the applicability condition related to availability of NTN neighbour cell assistance information. In our view, the existing cell reselection requirements are defined assuming assistance information of neighbour cell are available from serving cell. Otherwise, UE may not be able to track the time or frequency change during the cell search or measurement. Therefore, we do not support to the applicability.

Proposal 14: RAN4 not to change the applicability condition related to availability of NTN neighbour cell assistance information in Rel-18.

Issue 4-3: NTN-NTN Cell reselection enhancements (to reduce UE power consumption) - No agreement. In the future meetings, companies can provide further analyses on whether and to what extent UE measurement for LEO NTN-NTN cell reselection can be relaxed to reduce power consumption. The analyses should take into account, e.g. LEO cell type, cell deployment type (earth moving and quasi-earth fixed cell), UE implementation vs. RAN2 spec support based approach, feasibility, quantitative analyses in terms of gain-loss, etc.

In last meeting some companies propose to study relaxed cell measurements for NTN-NTN reselection.

For GEO and LEO fixed cell, we already have location triggered measurement in Rel-17, where UE is not required to measure when it is close to a reference location. Also, we have the relaxed measurement from Rel-16 TN. We assume there are enough tools to reduce power consumption.

For LEO moving cell, the coverage time would be short compared to fixed cell, and there are proposals to further restrict the DRX cycles. In this sense, we do not see the motivation to further relax the measurement since the mobility budget is already tight.

Proposal 15: De-prioritize additional requirements for relaxed measurement for NTN-NTN cell reselection.

HO enhancements

Issue 5-1: RACH-less (C)HO Agreement: - RAN4 to define RACH-less NTN HO requirements based on the RACH-less LTE HO requirements. NR and NTN specific adjustments shall be made. The adjustments to investigate may include: - FFS on known/unknown condition - FFS on necessity of fine time tracking if target cell is known - FFS on others if identified - Note: some aspects would need to wait for further RAN2 progress.

RAN2 agreed that LTE RACH-less HO procedure is used as baseline. RAN4 also agreed that the LTE RACH-less HO requirements can also be re-used as baseline. The related requirements from 36.133 are copied below.

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PUSCH channel within Dhandover seconds from the end of the last TTI containing the RRC command when UE is configured with RACH-less or combination of RACH-less and make-before-break handover. Dhandover equals the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2] plus the interruption time stated in clause 5.1.2.1.2.2 when UE is configured with RACH-less handover. 5.1.2.1.2.2 Interruption time for RACH-less handover When intra-frequency or inter-frequency RACH-less handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + 20 ms Where: Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. TIU is the interruption uncertainty in acquiring the first PUSCH transmission occasion when UE is configured with RACH-less handover in the new cell. - TIU can be up to 10 ms if UL grant is configured in RRC command. NOTE: The actual value of TIU shall depend upon the UL grant configuration in RRC command. - TIU can be up to TUL_grant if UL grant is not configured in RRC command. NOTE: TUL_grant is the time required to acquire and process uplink grant from the target Pcell. In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.1 for intra-frequency handover and Clause 8.1.2.3.1 for inter-frequency handover.

Compared to normal LTE HO, the change is in the definition of TIU. In this sense, the time for fine time tracking should be also included in the delay requirements.

Technically, fine time tracking is also needed because the first UL in the target cell is PUSCH which is less tolerant to timing error compared to PRACH. Also, for the known case, the last measurement may be performed several seconds ago, while for UE to meet the timing requirements an SSB should be available in the 160ms before the UL.

With the time for fine time tracking, we do not see strong need to update the known condition.

Proposal 16: For NTN RACH-less HO,

- time for fine time tracking should be included in the delay requirements.

- Re-use the known condition from normal NTN HO as baseline

Issue 5-2: HO for satellite switching without PCI change - No agreement. Wait for further progress from RAN2.

RAN2 agreed to support satellite switching without PCI change.

- In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18)

- Remove the part in brackets “as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18” in the LS to RAN1. The action to RAN1 will also ask for feedback for the hard satellite switch (not only the soft satellite switch case), e.g. action to RAN1 is to see if there are any major technical issues (as in the agreement).

RAN4 should define requirements for this enhancement. In our view, the mechanism is similar to HO. UE still needs to perform internal processing (which can be shortened), cell search, fine time tracking and wait for the PRACH resource. In this sense, the HO requirements can be re-used as starting point, and RAN4 can further discuss if any adaptation is needed.

In last meeting, some companies commented that the procedure may be different from normal HO in terms of absence of HO command and RACH procedure. On HO command, we agree with the comment and the RRC procedure delay may need to be updated if the indication of the satellite switching is not via RRC. On the RACH procedure, we understand as baseline it is needed for UL sync, and RAN2 will further discus whether it can be combined with RACH-less HO.

FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

Proposal 17: RAN4 to define requirements for satellite switching without PCI change. HO requirements can be re-used as starting point, FFS components to be adapted.

Issue 5-3: Group-based HO for signalling overhead reduction - No agreement. Wait for further progress from RAN2. Issue 5-4: Location based CHO enhancements for earth moving NTN cell - No agreement. Wait for further progress from RAN2.

RAN2 has not made further progress related to group-based HO and location based CHO for moving cell, so we suggest RAN4 to wait before discussing the requirements.

Proposal 18: RAN4 to wait for further progress from RAN2 on group-based HO and location based CHO for moving cell before discussing the requirements.

Conclusions

In this paper we provided our views on RRM impacts for NTN enhancements.

Proposal 1: RAN4 to discuss the possible impact of “not performing TA pre-compensation update within an TDW” on UE timing requirements.

Proposal 2: RAN4 to consider the following assumptions for timing accuracy for NTN in Ka band

- GNSS error: [15]m for non-mobility scenario, [50]m for mobility scenario

- Satellite location error: [25]m assuming ephemeris information update period <= [10]s

Proposal 3: RAN4 to further discuss the feasibility of 60kHz and 120kHz SCS for NTN operation in Ka band considering the impacts of timing/frequency accuracy and UE mobility.

Proposal 4: RAN4 considers to introduce ECP for 120kHz SCS.

Proposal 5: RAN4 to discuss how to account for Rx beamforming in the measurement and mobility requirements for NTN in Ka band, considering both parabolic antenna and phase array antenna.

Proposal 6: RAN4 to discuss possible differentiation of RRM requirements for mobile and fixed UEs.

Proposal 7: RAN4 to define measurement period and accuracy requirements for UE Rx-Tx measurement.

- Existing TN requirements can be used as baseline.

- NTN specific aspects can be further discussed based on RAN1 progress

- Side condition is FFS

Proposal 8: RAN4 waits for RAN1 conclusion on whether updates to UE/gNB Rx-Tx report mapping.

Proposal 9: RAN4 to discuss whether and how to define requirements for time triggered cell reselection measurement based on RAN2 progress.

Proposal 10: Location based cell reselection measurement requirements for earth fixed cell are re-used for earth moving cell as baseline. FFS the margin for applying the distance threshold.

Proposal 11: Do not further restrict DRX cycle beyond Rel-17 NR NTN for cell reselection in earth moving cell.

Proposal 12: De-prioritize additional requirements related to Ttrigger.

Proposal 13: If necessary, RAN4 to define NTN-TN cell reselection requirements considering

- UE is not required to measure TN frequency in an area there is no TN coverage

- UE is allowed to relax or stop measuring the TN frequency if no TN neighbor cell is detected for a time period, details are FFS.

Proposal 14: RAN4 not to change the applicability condition related to availability of NTN neighbour cell assistance information in Rel-18.

Proposal 15: De-prioritize additional requirements for relaxed measurement for NTN-NTN cell reselection.

Proposal 16: For NTN RACH-less HO,

- time for fine time tracking should be included in the delay requirements.

- Re-use the known condition from normal NTN HO as baseline

Proposal 17: RAN4 to define requirements for satellite switching without PCI change. HO requirements can be re-used as starting point, FFS components to be adapted.

Proposal 18: RAN4 to wait for further progress from RAN2 on group-based HO and location based CHO for moving cell before discussing the requirements.

Reference

- R4-2306358, WF on NR NTN enhancement RRM requirements, Moderator (Qualcomm Incorporated)

- R1-2304094, LS on PUSCH DMRS bundling for NR NTN coverage enhancement, RAN1

- R4-2306624, WF on NTN UE RF requirements in Ka-band, ZTE