3GPP TSG-RAN WG4 Meeting #107	R4-2308675

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on option B-1-1 for BWP without restriction

Source: 	Huawei, HiSilicon

Agenda item:	8.11.2.2

Document for:	Discussion

Introduction

RRM requirements for option B-1-1 for BWP without restriction are discussed in RAN4#106-bis-e and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Applicability condition of RLM/BFD/BM requirements

- Clarification on existing timing requirements

- L3 enhancement

- Requirements for supporting multiple options

- Additional requirements

In this paper we will provide our views on RRM requirements for option B-1-1 for BWP without restriction.

Discussion

Applicability condition of RLM/BFD/BM requirements

Issue 3-1: Applicable of existing RLM/BFD/BM requirements for supporting Option B-1-1 <Agreement >: - When CD-SSB is outside the active BWP, the existing requirements for SSB-based RLM/BFD/BM are applicable for UE supporting option B-1-1. - FFS details of applicable condition

In our understanding, UE capability is being discussed in RAN1, and the applicability of the L1 requirements are also involved. For example, restriction on the frequency domain location of the CD-SSB and UE specific BWP are discussed.

| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured.] 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | [Per band] | No | No | n/a | [UE indicates at most one of FG 53-1 and FG 53-2.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In order to get alignment among WGs and to avoid duplicated discussions, we suggest RAN4 to wait for RAN1 conclusion before discussing the applicable condition.

Proposal 1: RAN4 waits for RAN1 conclusion before discussing the applicable condition for the L1 requirements for option B-1-1.

Clarification on existing timing requirements

Issue 3-3: Clarification on existing timing requirements for supporting Option B-1-1 <Way forward >: - Proposals - Option 1: (vivo, Intel) - It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not. - Option 2: (Ericsson, Apple, Nokia, Intel, Huawei, Spreadtrum, CATT, MediaTek, OPPO) - No clarification on existing timing requirements

We support option 2.

Similar as for option A, we think the current timing requirements and its applicability are generic, and it does not preclude the case where SSB is outside BWP, so no clarification is needed.

Proposal 2: RAN4 not to make further clarification on existing timing requirements for option B-1-1.

L3 enhancement

<Agreement >: - It is a common RAN4 understanding that L3 measurements requirements impact for options B-1-1 and B-1-2 are not explicitly included in the WI objectives. - RAN4 work will focus on enabling L1 measurements for options B-1-1 and B-1-2 in Q2’2023. - In RAN4#107 RAN4 can continue discussion on whether any further clarifications are needed on intra-frequency measurements for UEs supporting options B-1-1 and B-1-2.

First, in our view it is beneficial to support intra-frequency measurement without MG for option B-1-1. Else, NW has to configure MG for intra-frequency L3 measurement which is very essential for UE mobility, and the benefit of being able to do L1 measurement without MG nor interruption is lost.

Second, intra-frequency measurement without MG is not supported for option B-1-1 based on current spec. Some clarification on the applicability condition for intra-frequency measurement without MG is needed. As copied below, the applicability condition for intra-frequency measurement without MG is defined in cl. 9.2.1 and it is reasonable to add supporting option B-1-1 as one applicability condition as another condition.

The UE can perform intra-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3]. For UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for intra-frequency measurement, - An intra-frequency SSB measurement is defined as measurement without gap if - the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the intra-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE, and - the active downlink BWP is not an initial BWP [3] The delay requirements are specified in clause 9.2.5

As L3 measurements requirements impact is not explicitly included in the WI scope, supporting intra-frequency measurement without MG based on option B-1-1 would be subject to RANP approval, like supporting intra-frequency measurement based on NCD-SSB in option C.

In last meeting, some companies commented that supporting intra-frequency measurement without MG may require a separate capability. We think it is a valid concern because there are some differences in L1 and L3 measurements, e.g. cell search is required in L3 but not in L1, and we are open to further discussions.

Besides, RAN4 also needs to discuss the relation with NFG and NCSG capability. For example, if UE supports option B-1-1 for a band, it is unreasonable that it reports it needs interruption/NCSG or MG for the intra-frequency measurement and it will cause confusion to NW.

Another issue raised up in last meeting is the inter-frequency measurement, e.g. whether UE supporting option B-1-1 should be able to measure inter-frequency MO without MG when the SSB is outside active BWP but within UE CBW. This seems to us an extension, but we are open to further discussion.

Proposal 3: Subject to RANP approval, RAN4 to define supporting option B-1-1 as one applicability condition for intra-frequency measurement without MG.

- FFS whether new UE capability is needed for L3 enhancements

- FFS relation with NFG and NCSG capability

- FFS on inter-frequency measurement without MG

Requirements for supporting multiple options

Issue 3-6: Whether to define requirements for supporting multiple options <Way forward >: - Proposals - Option 1: - Requirements/UE behaviour for UE supporting both option B-1-1 and C - Option 2: - Requirements for UE supporting both option B-1-1 and A.

We do not think additional requirements are needed.

For option 1, NCD-SSB is configured per BWP basis, and in 38.331 it is specified that NCD-SSB must be within the corresponding BWP.

absoluteFrequencySSB Frequency of the NCD-SSB. The network configures this field so that the SSB is within the bandwidth of the BWP configured in BWP-DownlinkCommon.

In addition, in 38.331 it is specified that when NCD-SSB is configured, it will be used for SSB related operations.

nonCellDefiningSSB If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the “SSB” configured in the QCL-Info IE; the “ssb-Index” configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

Therefore, for UE supporting both option B-1-1 and C, it should use NCD-SSB within the active BWP for L1 measurement. If UE is in a BWP without CD- nor NCD-SSB, then we understand it should use CD-SSB.

For option 2, it is up to NW to configure whether to use SSB, CSI-RS or both for L1 measurement. UE supporting both option B-1-1 and A should meet the corresponding requirements just like when SSB is within BWP.

Proposal 4: RAN4 not to define additional requirements for UE supporting multiple options.

Additional requirements

Issue 3-7: Whether to define additional requirements for supporting option B-1-1 <Way forward >: - Proposals - Option 1: - Do not introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported. - Do not introduce a new section for RRM requirements for option B-1-1. - Option 2: - Introduce a section for RRM requirement for Option B-1-1 and describe the behaviour of UE and gNB like as below. - “The Option B-1-1 supporting UE can autonomously change carrier freq. and RF BW to access CD-SSB for BM/RLM/BFD without causing any interruption. The gNB is expected to configure UE specific active BWP considering the max. supportable CHBW of the UE in the specific band.”

We do not see to specify UE and gNB behaviour as in option 2. What RAN4 needs to do is to define which requirements are applicable for UE supporting option B-1-1 and the corresponding applicability condition. UE should meet the requirements without causing any interruption when the gNB configurations satisfy the applicability condition. This has been the way how RAN4 requirements are defined.

Proposal 5: RAN4 not to introduce new UE and gNB behavior for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported.

Conclusions

In this paper we provided our views on RRM requirements for option B-1-1 for BWP without restriction.

Proposal 1: RAN4 waits for RAN1 conclusion before discussing the applicable condition for the L1 requirements for option B-1-1.

Proposal 2: RAN4 not to make further clarification on existing timing requirements for option B-1-1.

Proposal 3: Subject to RANP approval, RAN4 to define supporting option B-1-1 as one applicability condition for intra-frequency measurement without MG.

- FFS whether new UE capability is needed for L3 enhancements

- FFS relation with NFG and NCSG capability

- FFS on inter-frequency measurement without MG

Proposal 4: RAN4 not to define additional requirements for UE supporting multiple options.

Proposal 5: RAN4 not to introduce new UE and gNB behavior for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported.

Reference

- R4-2306397, WF on completion of specification support for BWP operation without restriction, vivo