3GPP TSG-RAN WG4 Meeting #107	R4-2308676

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on option C for BWP without restriction

Source: 	Huawei, HiSilicon

Agenda item:	8.11.2.3

Document for:	Discussion

Introduction

RRM requirements for option C for BWP without restriction are discussed in RAN4#106-bis-e and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Applicability condition of RLM/BFD/BM requirements

- Clarification on existing timing requirements

- L3 enhancement

- Additional requirements

In this paper we will provide our views on RRM requirements for option C for BWP without restriction.

Discussion

Applicability condition of RLM/BFD/BM requirements

Issue 4-1: Applicable of existing RLM/BFD/BM requirements for supporting Option C <Agreement >: - The existing requirements for SSB-based RLM/BFD/BM for non-RedCap UE are applicable to option C. - FFS details or wording of applicable condition.

In our understanding, UE capability is being discussed in RAN1, and the applicability of the L1 requirements are also involved. For example, the NCD-SSB should be within active BWP, and other restriction on the frequency domain location of the CD-SSB and UE specific BWP are discussed.

| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | [Per band] | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In order to get alignment among WGs and to avoid duplicated discussions, we suggest RAN4 to wait for RAN1 conclusion before discussing the applicable condition.

Proposal 1: RAN4 waits for RAN1 conclusion before discussing the applicable condition for the L1 requirements for option C.

Clarification on existing timing requirements

Issue 4-3: Clarification on existing timing requirements for supporting Option C <Way forward >: - Proposals - Option 1: (Ericsson, Nokia, Intel, CATT, MediaTek, OPPO) - The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support. - Option 2: (vivo, Huawei, Spreadtrum) - It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can also be NCD-SSB within active BWP if UE supports option C.

For RedCap, RAN4 updated the applicability condition of UE timing requirements to include both CD- and NCD-SSB. Basically, both can be used for time tracking.

- 7.1A.2 Requirements The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms on the condition that: - the SSB is within the UE’s active BWP, or - the SSB is not within the UE’s active BWP, and the measurement gap is configured.

In our view, the applicability condition can be re-used for non-RedCap UE with option C.

Proposal 2: RAN4 to include NCD-SSB in the applicability condition for timing requirements, same as timing requirement for RedCap.

L3 enhancement

Issue 4-4: Impact to intra-frequency measurement requirements for supporting option C During GTW on 17th April, following agreements were reached. <Agreement >: - It is a common RAN4 understanding that support of L3 measurements based on NCD-SSB for Option C can be beneficial, but it is not explicitly included in the WI objectives. Whether to support L3 measurements based on NCD-SSB is up to RAN decision. - RAN4 work will focus on enabling L1 measurements based on NCD-SSB in Q2’2023. Issue 4-5: Impact to inter-frequency measurement requirements for supporting option C <Way forward >: - The issue is postponed.

For RedCap, RAN4 updated the definition of intra- and inter-frequency measurements. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2]. Accordingly, RAN4 also defined requirements for transition of intra-frequency between CD-SSB NCD-SSB, transition of a measurement between intra- and inter-frequency.

### 9.2B.1 Introduction A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2]. ### 9.1A.6 Minimum requirement at transitions When the intra-frequency measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching during one cell identification period, the UE shall use a cell identification period that is the maximum of the cell identification periods corresponding to the first SSB type and the second SSB type after the BWP switching. When the intra-frequency measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching during one measurement period, the UE shall use a measurement period that is the maximum of the measurement periods corresponding to the first SSB type and the second SSB type after the BWP switching. When the measurement on one measurement object transitions from inter-frequency measurement to intra-frequency measurement or vice versa due to BWP switching during one cell identification period, the UE shall use a cell identification period that is the maximum of the cell identification periods corresponding to the first SSB type and the second SSB type after the BWP switching. When the measurement on one measurement object transitions from inter-frequency measurement to intra-frequency measurement or vice versa due to BWP switching during one measurement period, the UE shall use a measurement period that is the maximum of the measurement periods corresponding to the first SSB type and the second SSB type after the BWP switching . Subsequent to the above-mentioned measurement periods and/or cell identification periods, the cell identification and measurement period requirements on each measurement object are based on the SSB type after the BWP switch.

It was agreed last meeting that support L3 measurement based on NCD-SSB is beneficial but it is not explicitly included in the WI scope. The benefit of supporting L3 measurement based on NCD-SSB is that intra-frequency measurement can be based on the NCD-SSB within active BWP and UE can thus perform intra-frequency measurement without MG. Else, NW has to configure MG for intra-frequency L3 measurement which is very essential for UE mobility, and the benefit of being able to do L1 measurement without MG nor interruption is lost. In our view, the RedCap requirements can be re-used for non-RedCap UE with option C.

In last meeting, some companies suggested that whether inter-frequency measurement (based on NCD-SSB) needs MG or not depends on whether UE supports Rel-16 inter-frequency without MG. In our view this is reasonable but we do not see any spec change needed, i.e. the current requirements for inter-frequency without MG do not differentiate whether the inter-frequency SSB is CD- or NCD-SSB.

Proposal 3: Subject to RANP approval, RAN4 to re-use the intra-/inter-frequency definition and requirements for transition between CD-SSB and NCD-SSB for L3 measurement from RedCap for UE supporting option C.

Additional requirements

Issue 4-7: Whether to define requirements in addition to L1/L3 measurements for supporting option C <Way forward >: - The issue is postponed.

In HO requirements for RedCap, the applicable scenarios are clarified and HO to target cell’s specific RedCap BWP associated with NCD-SSB is supported in addition to HO to target cell’s initial BWP.

#### 6.1D.1.1 Introduction The purpose of NR handover is to change the NR PCell to another NR cell for RedCap UE. The requirements in this clause are applicable to SA NR. The requirements in this clause apply for the following handover scenarios: Handover to a target cell’s initial BWP associated with CD-SSB; Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP).

This means UE can perform RACH on the BWP containing either CD-SSB or NCD-SSB. In addition, for RedCap RAN4 agreed to support the scenario where the measured SSB and the target SSB for HO are different. In our view, those HO scenarios involving NCD-SSB can also be supported for non-RedCap UE with option C and same requirements from RedCap can be re-used.

In last meeting, some companies commented that whether to introduce these additional requirements for option C is related to handling of L3 measurement. In our view they are separate issues, e.g. UE may still consider the NCD-SSB within active BWP as inter-frequency but it can still support HO to target cell based on a BWP with NCD-SSB. However, we agree that introduction of these additional requirements is subject to RANP approval as it is not explicitly included in the WI scope now.

Another comment from last meeting is that HO to a BWP without CD-SSB is not very relevant to non-RedCap UE. We have a different view because during HO, UE is not required to read SIB1 from the initial BWP with CD-SSB, but the SIB1 is sent to UE via HO command. In this case, it would be beneficial if UE could support HO to target cell based on a BWP with NCD-SSB as it will alleviate the resource congestion on initial BWP.

Proposal 4: Subject to RANP approval, RAN4 to support HO scenarios involving NCD-SSB and re-use the same requirements from RedCap.

Conclusions

In this paper we provided our views on RRM requirements for option C for BWP without restriction.

Proposal 1: RAN4 waits for RAN1 conclusion before discussing the applicable condition for the L1 requirements for option C.

Proposal 2: RAN4 to include NCD-SSB in the applicability condition for timing requirements, same as timing requirement for RedCap.

Proposal 3: Subject to RANP approval, RAN4 to re-use the intra-/inter-frequency definition and requirements for transition between CD-SSB and NCD-SSB for L3 measurement from RedCap for UE supporting option C.

Proposal 4: Subject to RANP approval, RAN4 to support HO scenarios involving NCD-SSB and re-use the same requirements from RedCap.

Reference

- R4-2306397, WF on completion of specification support for BWP operation without restriction, vivo