3GPP TSG-RAN WG4 Meeting #107	R4-2308697

Incheon, KR, 22th – 26th May, 2023

Title: 	Discussion on maintaining issues for RLM/BFD relaxation requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.2.10.3

Document for:	Discussion

Introduction

In this contribution, we provide discussion on the maintaining issues for RLM/BFD relaxation requirements.

Discussion

Based on RAN4 agreements in WF [1], RLM/BFD relaxation is applicable for DRX cycle <=80ms. Then, RLM/BFD relaxation is not applicable in non-DRX mode or not applicable for DRX cycle >80ms. The existing non-applicable conditions for RLM/BFD relaxation in TS38.133 are defined as follows:

The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met: - The UE sends out-of sync indications to the higher layers, - The timer T310 is running. - No DRX is used or DRX cycle is longer than 80ms … The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met: - The timer beamFailureDetectionTimer is running. - No DRX is used or DRX cycle is longer than 80ms

The definition of “no DRX is used” in TS38.133 is described in section 3.6.1.

- 3.6.1 RRC connected state requirements in DRX For the requirements in RRC connected state specified in this version of the specification, the UE shall assume that no DRX is used provided the following conditions are met: - DRX parameters are not configured or - DRX parameters are configured and - drx-InactivityTimer is running or - drx-RetransmissionTimerDL is running or - drx-RetransmissionTimerUL is running or - ra-ContentionResolutionTimer is running or - a Scheduling Request sent on PUCCH is pending or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the preamble not selected by the MAC entity Otherwise the UE shall assume that DRX is used.

Even DRX is configured, the time period when drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer is running is still considered as the state “no DRX is used”, which means that the active time period due to some timers are running in DRX mode is still considered as “no DRX is used”. In DRX mode, UE could switch between the states “DRX is used” and “no DRX is used” within one DRX cycle. The state “no DRX is used” is not equivalent to the non-DRX mode.

Observation 1: According to the definition in section 3.6.1 in TS38.133, the active time period due to some timers are running in DRX mode is still considered as “no DRX is used”, which means “no DRX is used” is not equivalent to the non-DRX mode.

Whether or not to apply RLM/BFD relaxation is evaluated from a long-term perspective rather than a short-term perspective. Hence, the wording “No DRX is used” is not suitable for defining the non-applicable condition of RLM/BFD relaxation. We suggest to use “no DRX is configured” to replace it.

Proposal 1: For RLM/BFD relaxation requirements, the conditions for DRX cycle applicability need to be updated and can be defined as follow:

-	No DRX is configured or DRX cycle is longer than 80ms

Conclusions

This contribution provides the discussion on maintaining issues for RLM/BFD relaxation requirements.. The following are provided:

Observation 1: According to the definition in section 3.6.1 in TS38.133, the active time period due to some timers are running in DRX mode is still considered as “no DRX is used”, which means “no DRX is used” is not equivalent to the non-DRX mode.

Proposal 1: For RLM/BFD relaxation requirements, the conditions for DRX cycle applicability need to be updated and can be defined as follow:

-	No DRX is configured or DRX cycle is longer than 80ms

Reference

- R4-2105797, “WF on NR UE Power Saving Enhancements”, MediaTek, vivo