3GPP TSG-RAN WG4 Meeting #107	R4-2308703

Incheon, KR, 22th – 26th May, 2023

Title: 	Discussion on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment

Source: 	Huawei, HiSilicon

Agenda item:	8.12.3

Document for:	Discussion

Introduction

In this contribution, we will provide the discussion on RRM remaining issues for intra-band non-collocated EN-DC/NR-CA.

Discussion

For supporting intra-band NR-CA in non-collocated deployment, the UE capabilities were separately discussed for 2-layer MIMO and 4-layer MIMO cases.

Two-layer MIMO case

For intra-band non-collocated EN-DC/NR-CA, RAN4 has achieved agreements on the impacted RRM requirements for Type 2 UE. The RRM progress in last RAN4 meeting were captured in WF [1]. RAN4 has agreed that the existing SCell activation delay requirements for FR1 SCell for non-contiguous intra-band CA can be re-used for Type 2 UE. RAN4 need to further study whether any CRs are needed to clarify the requirements applicability.

The existing SCell activation delay requirements in FR1 are defined as follows:

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms. If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise. otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - the SCell is contiguous to an active serving cell in the same band, and - A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell - its RTD with contiguous FR1 active serving cell is larger than 260ns with respect to the to-be-activated SCell’s SSB numerology, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB; If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band. … where, TSMTC_MAX: - In FR1, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. … TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling: - In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst. …

It can be observed that the values of TSMTC_MAX and TFirstSSB_MAX used for defining SCell activation delay requirements in FR1 are separately defined for intra-band case and inter-band case.

Observation 1: In current spec, the values of TSMTC_MAX and TFirstSSB_MAX used for defining SCell activation delay requirements in FR1 are separately defined for intra-band case and inter-band case.

Since R15, the SCell activation requirements are defined based on the assumption that UE has shared RF chain for intra-band CA and separate RF chains for inter-band CA. However, Type 2 UE is assumed to have separate RF chains for intra-band non-contiguous CA operation, which is similar to inter-band CA operation. Hence, the values of TSMTC_MAX and TFirstSSB_MAX defined for inter-band case can be reused for intra-band non-contiguous CA case for Type 2 UE.

Proposal 1: For Type 2 UE, the values of TSMTC_MAX and TFirstSSB_MAX used for defining SCell activation delay requirements for FR1 inter-band CA case can be reused for FR1 intra-band non-contiguous CA.

Four-layer MIMO case

For 4-layer MIMO case, RAN4 suggests to postpone Type 3a/3b and Type 4a/4b to future release, which has been captured in the WF [2] and the RF discussions on both Type 3a/3b and Type 4a/4b are postponed. Hence, we suggest to postpone the discussions on Type 3a/3b and Type 4a/4b in RRM session accordingly.

Proposal 2: For 4-layer MIMO, it is suggested to postpone the RRM discussions on both Type 3a/3b and Type 4a/4b.

Conclusions

In this paper we provided our analysis on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment. The followings are provided.

Observation 1: In current spec, the values of TSMTC_MAX and TFirstSSB_MAX used for defining SCell activation delay requirements in FR1 are separately defined for intra-band case and inter-band case.

Proposal 1: For Type 2 UE, the values of TSMTC_MAX and TFirstSSB_MAX used for defining SCell activation delay requirements for FR1 inter-band CA case can be reused for FR1 intra-band non-contiguous CA.

Proposal 2: For 4-layer MIMO, it is suggested to postpone the RRM discussions on both Type 3a/3b and Type 4a/4b.

Reference

- R4-2306335, “WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA”, Huawei, HiSilicon

- R4-2306605, “WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment”, KDDI