3GPP TSG-RAN WG4 Meeting #107	R4-2308706

Incheon, KR, 22th – 26th May, 2023

Title: 	Discussion on RRM requirements for NR network-controlled repeaters

Source: 	Huawei, HiSilicon

Agenda item:	8.29.5

Document for:	Discussion

Introduction

In RAN4#106bis meeting, RAN4 achieved some agreements on RRM impacts for NCR-MT and the progress has been captured in WF [1]. In this contribution, we will provide further discussion on the remaining open issues for NCR MT.

Discussion

RAN4 achieved the agreements on most of necessary RRM requirements for NCR-MT. However, the following open issues still need to be further studied for NCR-MT.

Issue 4-2: adaptive beamforming for NCR access link - Agreements - FFS whether to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency. Issue 5-1: BFD/BFR/RLM for NCR-MT - Agreements - For LA NCR-MT - Define RLM/BFD/BFR requirements - For WA NCR-MT - Do not define BFD/BFR requirements - FFS for RLM requirements.

The NCR-MT is to communicate with the gNB to indicate the side control information for NCR-Fwd. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB. The NCR-Fwd is the function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE, which is transparent for the UE. Besides, according to the scope of this WID, RAN4 needs to study the necessary RRM requirements for NCR-MT rather than NCR-Fwd. So, there is no need to define RRM requirements for NCR-Fwd access link for beam configuration and switching latency.

Proposal 1: There is no need to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

For legacy RLM/BFD/CBD, UE is required to perform regularly measurements in order to monitor cell/beam level link quality of serving cell. In RAN1, it has been agreed that R15 legacy BFD/BFR/RLM mechanisms are supported as optional functionalities for NCR-MT. Besides, the beam for C-link at NCT MT almost can be assumed as fixed beam. We suggest that it is up to UE how to perform RLM/BFD/CBD measurements if supports. RAN4 does not need to define RRM requirements to require UE to perform regularly RLM/BFD/CBD measurements.

Proposal 2: There is no need to define RLM requirements for WA NCR-MT.

Conclusions

In this paper we provided our analysis on RRM impacts for NCR MT. The followings are provided.

Proposal 1: There is no need to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

Proposal 2: There is no need to define RLM requirements for WA NCR-MT.

Reference

- R4-2306360, “WF on NCR-MT RRM requirements”, ZTE