3GPP TSG-RAN WG4 Meeting #107	R4-2308724

Inchon Korea, 22 May – 26 May, 2023

Source:	ZTE Corporation

Title:	Discussion on inter-RAT measurement without gaps

Agenda Item:	8.10.3.2

Document for:	Approval

- Introduction

In the last RAN4 106 bis-e meeting, some meaningful agreements on the discussion on inter-RAT measurement without gaps have been achieved [1]. In this paper, we continue to give our views on the inter-RAT no-gap measurement in the aspects of using scenarios, UE capabilities, general principle to define measurement requirements and measurement reporting period requirements.

- Discussion

Up to now, all agreed using scenarios for inter-RAT NR/LTE measurements without gap is summarized as follows:

- the inter-RAT NR measurements without gap in Rel-18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

- the inter-RAT LTE measurements without gap in Rel-18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

2.1 Using scenarios

Issue 2-1-1: Differentiate scenarios for Case b-2

In the last meeting, some companies proposed that the scenario for Case b-2 could be further divided into the following two scenarios:

- Scenario 1: Inter-RAT LTE measurement for LTE CRS rate-matching feature - Scenario 2: Inter-RAT LTE measurement for CRS-IM receiver

However, in our view, we don’t see the necessity. In the previous meetings, we have separated inter-RAT LTE measurement into case b-1 and case b-2. There is no need to further differentiate case b-2, which would complex our work.

Proposal 1: No need to further differentiate the scenario for case b-2.

2.2 UE capabilities

Issue 2-2-2: UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP (Case b-2)

In the previous meeting, a new per UE capability has been agreed to introduce to support case b-2. In our view, we prefer to follow similar capability as Rel-16 inter-frequency without gap capability, as follows:

interFrequencyMeas-NoGap-r16 Indicates whether the UE can perform inter-frequency SSB based measurements without measurement gaps if the SSB is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of cells to be measured.

Proposal 2: For UE capability to support case b-2, we prefer to follow similar capability as Rel-16 inter-freq without gap capability.

2.3 General principle to define measurement requirements

Issue 2-3-2: Searcher limitation

For search limitation, the options proposed in the last meeting are given as below:

- Option 1: Inter-RAT LTE measurement without gap(case b-2) can be performed in parallel with NR measurement without searcher limitation - Option 2: Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported

Regarding to search limitation, in NR, there are at most two parallel measurements allowed. Specifically, one searcher is used by PCell, and the other searcher is shared by all other MOs without gap. While in LTE, there is no concept of searcher. In our view, inter-RAT LTE measurement without gap can be performed in parallel with NR measurement without searcher limitation.

Proposal 3: Inter-RAT LTE measurement without gap can be performed in parallel with NR measurement without searcher limitation.

2.4 Measurement reporting period requirements

In the last meeting, the measurement reporting period requirements were separated into the following issues:

Issue 2-4-1: All inter-RAT cases with “gap”

Issue 2-4-2: case a-1 with “no gap but interruption allowed”

Issue 2-4-3: case a-1 with “no gap no interruption”

Issue 2-4-4: case b-1 with “no gap but interruption allowed”

Issue 2-4-5: case b-1 with “no gap no interruption”

Issue 2-4-6: case b-2 with “no gap but interruption allowed”

Issue 2-4-7: case b-2 with “no gap no interruption”

For issue 2-4-1 and Issue 2-4-4, it has been agreed that the existing requirements can be reused. In this subject, we’d like to share our views on the the left issues.

Issue 2-4-2: Measurement period requirements for case a-1 with “no gap but interruption allowed”

Issue 2-4-3: Measurement period requirements for case a-1 with “no gap no interruption”

For the measurement period for case a-1 with ‘no gap but interruption allowed’, in our view, there are two options:

- Option 1: On the top of measurement period requirement for inter-RAT NR measurement in the TS36133 clause 8.1.2.4;

- Option 2: On the top of measurement period requirement for NR inter-freq no-gap measurement with interruption.

Compared with option 1, option 2 takes the impact of interruption into consideration. Thus, we prefer to take the measurement period requirements for inter-freq no-gap measurement with interruption as base line.

Proposal 4: For measurement period requirement for case a-1 with ‘no gap but interruption allowed’, we could take the measurement period requirements for inter-freq no-gap measurement with interruption as base line.

Similarly, in our view, for measurement period requirement for case a-1 with‘no gap no interruption’, we could take the measurement period requirements for inter-freq no-gap measurement without interruption as base line.

Proposal 5: For measurement period requirement for case a-1 with‘no gap no interruption’, we could take the measurement period requirements for inter-freq no-gap measurement without interruption as base line.

Issue 2-4-5: Measurement period requirements for case b-1 no gap no interruption allowed

Issue 2-4-7: Measurement period requirements for case b-2 no gap no interruption allowed

According to the discussion in the last meeting, the measurement requirement for case b-1 and case b-2 can be defined by the following framework:

TIdentify,  interRAT_EUTRAN_gapless=TBasicIdentify∗480TInter1∗CSSFinterRAT      ms

And there are some concern on the definition of TBasicIdentify, Tinter1 and CSSFinterRAT. In our understanding, TBasicIdentify is the time period used in the inter-RAT LTE equation where the maximum allowed time for the UE to identity a new inter-RAT LTE Cell is defined. According to the existing specification, TBasicIdentify = 480 ms for MG and NCSG. And there is no definition of TInter1 and CSSFinterRAT in the inter-RAT LTE no-gap measurement. For CSSFinterRAT, it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i for measurement conducted outside measurement gap. In this equation, we shall take no gap LTE MO into consideration.

Proposal 6: For measurement requirement for case b-1 and case b-2, TBasicIdentify  keeps 480 ms.

Proposal 7: For CSSFinterRAT, take gap-less LTE MO into consideration.

Issue 2-4-6: Measurement period requirements for case b-2 no gap but interruption allowed

It is still under discussion that whether the indication ‘no gap with interruption’ is allowed. We suggest to postpone this issue until we conclude issue 2-2-2.

Proposal 8: Postpone the issue 2-4-6 until we conclude issue 2-2-2.

- Conclusion

In this contribution, we have the following observations and proposals for inter-RAT measurement without gaps:

Proposal 1: No need to further differentiate the scenario for case b-2.

Proposal 2: For UE capability to support case b-2, we prefer to follow similar capability as Rel-16 inter-freq without gap capability.

Proposal 3: Inter-RAT LTE measurement without gap can be performed in parallel with NR measurement without searcher limitation.

Proposal 4: For measurement period requirement for case a-1 with ‘no gap but interruption allowed’, we could take the measurement period requirements for inter-freq no-gap measurement with interruption as base line.

Proposal 5: For measurement period requirement for case a-1 with‘no gap no interruption’, we could take the measurement period requirements for inter-freq no-gap measurement without interruption as base line.

Proposal 6: For measurement requirement for case b-1 and case b-2, TBasicIdentify  keeps 480 ms.

Proposal 7: For CSSFinterRAT, take gap-less LTE MO into consideration.

Proposal 8: Postpone the issue 2-4-6 after we conclude issue 2-2-2.

- Reference

- [1]  R4-2306331 “WF on NR_MG_enh2 Part 2”, Intel Corporation