3GPP TSG-RAN WG4 Meeting #107	R4- 2308744

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title:	Discussion on NS_50 PC1.5 A-MPR for band n39

Agenda Item:	7.19.2

WI Code:	HPUE_NR_FR1_TDD_R18

Document for:	Approval

# 1	Introduction

The NS_50 PC1.5 A-MPR is the remaining issue for the WID [1] as per WF [2]. Some measurement results have been provided by Skyworks [3] and Qualcomm [4]. In the sequel, we share our view on the A-MPR requirements based on our measurement and study.

# 2	Discussion

The measurement conditions follow the agreement in [2]. The first test case is for CBW=5MHz. It’s found that the same conclusion can be drawn as in [4].

Observation 1: No PC1.5 A-MPR is required for CBW=5MHz.

Test cases for other CBWs are investigated, and similar observations as those in [3] are obtained.

Observation 2: For CBWs larger than 20MHz, the A-MPR regions assigned with A1 and A2 contain the most demanding RB allocations.

When examining the PC2 A-MPR requirements, it’s noticed that the A-MPR for 2Tx architecture are not specified yet. For NS_48/49, the following note is specified to account for the additional RIMD effect due to 2Tx, which should also be applicable for NS_50.

NOTE 7: 	The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable.

Observation 3: NOTE 7 of the A-MPR Table 6.2.3.1-1 is also applicable to PC2 A-MPR for NS_50. Otherwise, the PC2 A-MPR for 2Tx architecture is incomplete.

PC1.5 uses dual-Tx by default. Hence, the RIMD effect should be included in the A-MPR requirements. Based on our measurements and considering the impact of APT/ET implementations, we propose the following for PC1.5:

Proposal 1: Based on the existing A-MPR for PC2 (i.e. Table 6.2.3.19-4), set A1=A1+3dB and A2=A2+[2.5~3]dB for PC1.5.

Due to time limit, we have not performed an exhaustive checking on all A-MPR regions. It might be possible that the existing A-MPR regions for PC2 need expansion for PC1.5.

Proposal 2: Use the existing A-MPR region definitions for PC2 as baseline for PC1.5. Modifications or new A-MPR regions can be allowed if found necessary.

# 3	Conclusion

The PC1.5 A-MPR for NS_50 has been discussed. Based on our study, the following observations and proposals are made:

Observation 1: No PC1.5 A-MPR is required for CBW=5MHz.

Observation 2: For CBWs larger than 20MHz, the A-MPR regions assigned with A1 and A2 contain the most demanding RB allocations.

Observation 3: NOTE 7 of the A-MPR Table 6.2.3.1-1 is also applicable to PC2 A-MPR for NS_50. Otherwise, the PC2 A-MPR for 2Tx architecture is incomplete.

Proposal 1: Based on the existing A-MPR for PC2 (i.e. Table 6.2.3.19-4), set A1=A1+3dB and A2=A2+[2.5~3]dB for PC1.5.

Proposal 2: Use the existing A-MPR region definitions for PC2 as baseline for PC1.5. Modifications or new A-MPR regions can be allowed if found necessary.

# References

- RP-222351 New WID: High power UE (power class 1.5) for NR TDD bands, CMCC, RP#97e

- R4-2217119 WF on HPUE_Basket_Intra-CA_TDD, Huawei, HiSilicon, RAN4#104bis-e

- R4-2219981 Preliminary measured back-off results for NS_50 PC1.5 A-MPR for n39, Skyworks, RAN4#105

- R4-2305505 NS_50 measurements and A-MPR, Qualcomm, RAN4#106bis-e