3GPP TSG-RAN WG4 Meeting #107									                          R4- 2308745

Incheon, South Korea, May 22 – 26, 2023

Agenda item:			8.14.2.2

Source:	Ericsson

Title:	Discussion on ATG UE RF Tx requirements

Document for:	Discussion

# Introduction

This paper discusses the remaining open issues of ATG UE RF Tx requirements.

# Discussion

Open issues

## The maximum output power

Agreement:

- Introduce two UE types to distinguish the antenna types for ATG UE

WF: FFS on accuracy/ tolerance.

WF: FFS whether and/or how to improve the term of [the minimum of maximum output power].

The ATG UE output power depends on the aircraft type and deployment scenario. Limiting the upper end of MOP to the maximum output power modeled in the co-existence simulations is necessary to ensure co-existence. Furthermore, an upper limit is needed as the signaling needs to be defined. The co-existence simulation assumed 43 dBm EIRP with a certain antenna array size which lead to 26 dBm TRP. The tolerance could be ±2 dB.

- The tolerance could be ±2 dB

- Suggest using [lower bound of maximum power range]

## Minimum output power for ATG

WF: FFS the assumption of minimum distance between ATG BS and ATG UE to derive the minimum output power.

To estimate the minimum output power for ATG UE, one can consider the scenario where there is the minimum pathloss towards the BS and the maximum antenna gain.

In this case, the minimum altitude considered for ATG is 3 km, which sets the minimum possible distance between the aircraft to an ATG BS at 3 km. In this case, the BS antenna would be pointing straight up towards the UE and the UE antenna would be pointing straight down. Although this scenario may not be very likely, it does represent the theoretical minimum pathloss that could occur. Also, we go beyond the assumptions for the co-existence simulations that some margin is embedded into the minimum power. This is the lowest ever needed in any conceivable scenario. Assuming a minimum SNR target of 0 dB, the estimated minimum UE power in this scenario is overly pessimistic, since a higher SNR target is likely to be used in reality, and the UE power could also be higher.

With the above assumption, an omnidirectional UE at 2 GHz experiences a coupling loss of 84 dB.

Coupling loss at 2 GHz = 108 dB (pathloss) – 24 dB (BS antenna gain).

With a 20 MHz bandwidth, the noise floor of the BS is -96 dBm, leading to a required UE power of -12 dBm.

In the case of 4GHz UE with a 20 dB gain, the coupling loss can be calculated as 73 dB, which is derived as below:

Coupling loss at 4 GHz = 114 dB (pathloss) – 24 dB (BS antenna gain) – 17 dB (UE antenna gain).

With a 100 MHz bandwidth, the BS noise floor is -89 dBm which requires a UE output power of -16 dBm. Based on these calculations, a minimum output power of -20 dBm should be sufficient. However, it is worth noting that this analysis relies on an unlikely scenario in which an aircraft is directly above a BS at a minimum altitude. If a more realistic scenario would be considered, then an even higher UE minimum output power could be considered.

The requirement for minimum output power may not be significantly impacted by the scaling of bandwidth, as the above estimations showed that it has already much more relaxed compared to the current requirement specified for the UE in TS 38.101-1[18]. Therefore, our proposal for the minimum output power is:

- The minimum output power for ATG UE is -25 dBm for 2 GHz and -20 dBm for 4 GHz.

## UL 256 QAM for ATG UE

WF: FFS whether 256 QAM should be supported for ATG UL and the existing transmit modulation quality requirements can be reused.

For the ATG UE, a high-capacity link to the aircraft is required and it is expected that with LoS a high SNR can be achieved, and the same modulation order will be utilized most of the time. As a result, a high modulation order, i.e., 256 QAM is essential for the ATG UE. It is worth noting that this MCS referred is not applicable to every UE. UEs located at the cell edge do not necessarily have the link budget to support 256 QAM, but it does not imply that 256 QAM support should be excluded altogether. A simplified link budget calculation with the agreed ATG BS and UE assumption is delivered below.

The distance between ATG UE and BS could be 10km ~ 300km. The typical propagation channel is single tap channel, so free space propagation loss could be used. Other parameters for ATG BS and ATG UE are captured in Table 1 which refers to TS38.876 6.2.2. Subarray antenna model is used for both FDD and TDD ATG BS in following calculation.

Table 1 System and antenna parameters for ATG BS and UE

| Centre frequency [GHz] |  | 2 | 4 |
| --- | --- | --- | --- |
| CBW [MHz] |  | 20 | 100 |
| PRB number |  | 106 | 273 |
| ATG BS system | Tx max power [dBm] | 43 | 50 |
|  | noise figure [dB] | 5 | 5 |
| ATG BS antenna | Element gain [dBi] | 6.4 |  |
|  | Array configuration (Row × Column) | 4x8 |  |
|  | Sub-array elements | 3 |  |
| ATG UE system | TRP or EIRP [dBm] | 40 | 43 |
|  | noise figure [dB] | 9 | 9 |
| ATG UE antenna | element gain [dBi] | 0 | 5 |
|  | Array configuration (Row × Column) | \ | (8x2) or (16x1) |

The intention is to show the possibility of using 256 QAM in an ATG network, so the largest antenna gain is assumed for all propagation distances. The SNR at the receiver side for 2GHz and 4GHz are shown in Figure 1. When the aircraft is closer to the BS, the SNR above 25 dB can be seen.

Figure 1 Received SNR vs. propagation distance.

- UL 256QAM should be supported for ATG UE.

## Configured transmitted power

It is still important for the network to be able to configure a Pmax in case there are regulatory limits. It is unsure of the applicable regulation at present, it may be beneficial to have the functionality/capability to limit the power to below the UEs indicated maximum if required.

- It is proposed to define the Pcmax following the definition in TS 38101-1

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The tolerance could be ±2 dB

Proposal 2	Suggest using [lower bound of maximum power range]

Proposal 3	The minimum output power for ATG UE is -25 dBm for 2 GHz and -20 dBm for 4 GHz.

Proposal 4	UL 256QAM should be supported for ATG UE.

Proposal 5	It is proposed to define the Pcmax following the definition in TS 38101-1

# References

[1] R4-2306609 WF on ATG UE RF requirements