3GPP TSG-RAN WG4 Meeting #107									                          R4- 2308749

Incheon, South Korea, May 22 – 26, 2023

Agenda item:			8.20.2.2.4

Source:	Ericsson

Title:	TP to TR 38.858 on Feasibility of FR2-1 UE aspects

Document for:	Approval

# Introduction

In the last RAN4#106bis-e meeting, TPs from different companies addressing the feasibility of UE aspects were consolidated and approved in [1]. This paper captured the agreements in the WF [2] and proposed some updates and comments for TR 38.858 on the Feasibility of UE aspects, i.e., 10.7.

# References

- R4-2305971 TP to TR 38.858 feasibility of and impact on FR2 UE RF requirements.

- R4-2305969 WF for SBFD feasibility study and requirements impact from UE aspect.

# Text Proposal

## <<Start of Change for TR 38.858>>

10.7 FR2-1 Feasibility of UE aspects

10.7.1    Interference analysis

10.7.1.1  UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.1.1 Receiver aspects

Existing co-channel UE RX performance requirements

For legacy UEs, the current UE RF architecture can be assumed without any RF architecture modification.

Currently there are no RF requirements for UE co-channel Rx performance.

Sub-band filtering and legacy UEs

For legacy UEs, no sub-band filtering is implemented, and therefore RAN4 has not assumed any subband filtering.

Sub-band filtering for a SBFD-aware UEs – UEs with a new feature

SBFD-aware UEs support half duplex operation while receiving the sub-band configuration from the network. For this kind of UE, the RF architecture and applicable RF requirements need further study for future releases. In the present study, legacy UEs are the main focus.

.

Thermal self-noise aspects (both adjacent channel and co-channel)

RAN4 decided on a simple fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was considered to be sufficient for the purpose of system studies for SBFD. RAN4 decided on a NF of 10 dB.

Subband in-channel selectivity

It is worth noting that the RF degradations can cause inter-subband interference . An analysis of the FR2-1 receiver’s design was conducted. Various factors, such as residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions, were considered. The effect of all these distortions is lumped into a single parameter called selectivity. Based on the discussion and analysis from the meeting, contributions suggestedthe sub-band selectivity value being  23 dB. The receiver performance is simply modelled as being 23 dB below the total input power level Pin. Pin the total power into the receiver, whether it is signal or jammer.

An analysis of the FR2-1 receiver’s design was conducted. Various factors, such as residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions, were considered.

The definition of Sub-band/In-channel selectivity is introduced for clarity in the SBFD feasibility study:

- Sub-band/In channel selectivity is the ratio of the interference power in the desired DL to the total input power at the receiver input. The total input power includes both desired signal and the undesired jammers. The selectivity includes the entire receiver all the way through the output of the FFT.

FFT leakage and selectivity

In an ideal scenario, the UL transmission of the aggressor UE should not impact the DL reception of the victim UE due to the OFDM wave orthogonality. However, non-ideal FFT suppression can cause interference to the victim UE, particularly when the UL sub-band has frequency errors and is not time-synchronized with the DL sub-band. The analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity, and frequency and time offset are not significant factors influencing UE-UE interference. It is worth noting that the RF degradations can cause inter-subband interference as well and the impact will depend on the targeted Rx IM and EVM performance. Nonetheless, this interference will not be any worse than the selectivity value. For this reason, the 23 dB was agreed upon for modeling the inter-sub-band selectivity.

10.7.1.1.2 Transmitter aspects

Inband emissions (co-channel)

RAN4 has decided to use the IBE requirements from 38.101-2 clause 6.4.2.3.4 (power class 3 UE). It is understood these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters,   Since typical values for the UE parameters were not determined, the formulation from the MPS is being used.

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.

Analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity,

Apart from the selectivity, the degradation can be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel case, the leakage was agreed to be modelled using IBE based model. Additionally, the IQ image contribution for the IBE model for co-channel CLI can be ignored for the DUD configuration.

For UE co-channel Tx model, UE IBE in TS 38.101-2 can be used in the feasibility study as shown in Table 10.7.1.1-1. This model consists of three parts, General, IQ image, Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored in the feasibility study. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation.

Table 10.7.1.1-1: Requirements for in-band emissions in TS 38.101-2 (For Power class 3)

| Parameter description | Unit | Limit (NOTE 1) |  |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | dB |  |  |  | Any non-allocated (NOTE 2) |
|  |  |  | Output power for FR2-1 | Output Power for FR2-2 |  |
| IQ Image | dB | -25 | > 10 dBm | > 8.1 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | ≤ 10 dBm | ≤ 8.1 dBm |  |
| Carrier leakage | dBc | -25 | > 0 dBm | > -1.9dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm | -14.9 dBm ≤ Output power ≤ -1.9 dBm |  |

10.7.1.2  UE-UE adjacent channel CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.2.1 Receiver aspects

Effect of adjacent channel aggressor UE jammer

An analysis of the FR2-1 receiver’s design was conducted. Various factors, such as residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions, were considered. The receiver’s performance is simply represented as 34 dB lower than the input power level (Pin), which includes both signal and jammer power.

10.7.1.2.2 Transmitter aspects

ACLR (adjacent channel)

ACLR is one aspect modelled as an interference aspect from a nearby aggressor UE transmitting in an adjacent UL subband. UE ACLR is modeled as [24 dB] at max power, improving 1 dB/dB with backoff up to a maximum of 10 dB of improvement. Therefore, when the backoff is 10 dB, the ACLR is [34] dB. This model is an approximation of the performance of a typical UE.

10.7.2	Summary

Editor's note: This section captures the conclusion of feasibility.

## <<End of Change>>