3GPP TSG-WG4 Meeting #107 	R4-2308756

# Incheon, Korea, May 22 - May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3301 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for definition of PL-RS maintained and time tracking for unified TCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Definition of PL-RS maintained and clarification to the UE time tracking requirements for unified TCI |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN4 has been discussing for a long time the issue of what ‘Pl-RS maintained’ mens and its definition. Such definition is missing and is needed to make the UE requirements applicable. Additional, it needs to be clear what the UE requirements for time tracking is for the active UL TCI state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN4 requirements are broken |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.3, 8.16.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

### 8.16.3	MAC-CE based uplink TCI state switch delay

The requirements in this clause shall apply for UL TCI state switch using separate UL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in UL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in 8.16.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch.

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell,

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

-	If target TCI state is unknown,

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + (TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

Where,

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

-	PL-RS is maintained provided:

-	There are no more than 4 different RS configured as PL-RS per serving cell among all active UL TCI states (UL TCI state or joint) for PUSCH/PUCCH/SRS transmissions

-	The target pathloss reference signal remains detectable during TCI state switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the TCI state switching period

-	SNR of the associated SSB ≥-3dB

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after L1-RSRP measurement when target TCI state is unknown.

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after MAC CE command is decoded by the UE for known TCI State.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS when PL-RS is associated with serving cell

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB when PL-RS is associated with PCI different from serving cell

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	The UE shall be able to transmit with the old UL TCI state until slot n+ THARQ + 3Nslotsubframe,µ.

# <Start of change 2>

### 8.16.5	Active Uplink TCI state list update delay

The requirements specified in this clause are applicable if

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, and a MAC CE activates more than one target separate TCIs, and at least one UL TCI is included, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, and a MAC CE activates more than one target joint TCI.

If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to transmit PUCCH, PUSCH or SRS with the new target TCI states at the first slot that is after

n + THARQ + 3Nslotsubframe,µ + NM * (Tfirst_target-PL-RS_List + 4 * Ttarget_PL-RS_List + 2ms) / NR slot length,

If a subset of target TCI states in the active TCI state list are unknown, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to transmit PUCCH, PUSCH or SRS with the new target TCI states at the first slot that is after

n+THARQ + 3Nslotsubframe,µ+ (TL1-RSRP_List + Tfirst_target-PL-RS_List + 4*Ttarget_PL-RS_List + 2ms) / NR slot length

If all target TCI states in the active TCI state list are unknown, the requirements specified in this clause are not applicable.

Where

-	If all TCIs are known,

-	if the target PL-RS associated with or included in any UL TCI is not maintained, NM = 1;  Where maintained PL-RS is defined in clause 8.16.3

-	if a subset of target TCI states in the active TCI state list are unknown,

-	TL1-RSRP_List = is the longest L1 measurement time (TL1-RSRP) of the source RS among the unknown target TCI states , where TL1-RSRP is specified in clause 8.16.3

-	if the number of cells associated with the target TCI states in the active TCI list is 2, and time to first PL-RS associated to the TCIs are overlapped in FR2,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS_SC + min(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP) ;

-	Ttarget-PL-RS_List = 2 * Ttarget-PL-RS_SC, if Ttarget-PL-RS_SC = Ttarget-PL-RS_CDP

-	Ttarget-PL-RS_List = max(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP), if  Ttarget-PL-RS_SC  ≠ Ttarget-PL-RS_CDP

-	if the number of cells associated with the target TCI states in the active TCI list is 2, and time to first PL-RS associated to the TCIs are not overlapped in FR2,

-	Tfirst_target-PL-RS_List = max (Tfirst_target-PL-RS_SC, Tfirst_target-PL-RS_SC);

-	Ttarget-PL-RS_List = max(Ttarget-PL-RS_SC , Ttarget-PL-RS_CDP)

-	Otherwise,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS_SC

-	Ttarget-PL-RS_List = Ttarget-PL-RS_SC.

-	Tfirst_target-PL-RS_SC is Tfirst_target-PL-RS for serving cell

-	Tfirst_target-PL-RS_CDP is Tfirst_target-PL-RS for cell with different PCI from serving cell

-	Ttarget-PL-RS_SC is Ttarget-PL-RS for serving cell

-	Ttarget-PL-RS_CDP is Ttarget-PL-RS for cell with different PCI from serving cell-	THARQ, Tfirst_target-PL-RS, Ttarget-PL-RS are defined in clause 8.16.3.

Note:	The requirements specified in this clause do not apply if more than 4 different RSs are maintained as PL-RS per serving cell among all active UL (or joint) TCI states.

When UE receives PDSCH carrying MAC-CE for active TCI state list update, and

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘joint’, or

-	higher layer configuration ‘unifiedTCI-StateType-r17’ is set to ‘separate’, while the target TCI list comprises at least one DL TCIs and at least one UL TCIs,

UE is not expected to transmit on UL based on the target TCI before UE completes the DL and UL TCI list update.

The UE is required to perform accurate tracking of tim and frequency derived from DL-RS associated with an active UL TCI state or active joint TCI state.

<End of changes>