3GPP TSG-RAN WG4 Meeting #107 	R4-2308769

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Bandwidth part operation without restriction in NR – Option B-1-1

Agenda item:	8.11.2.2

Document for:	Discussion

Introduction

RAN#99 approved a new WID to address the issues on BWP without restriction in [1]. Objectives can be found below:

The WI was discussed first time in RAN4#106bis where some agreements were reached which are captured in [2]. [2] also captures a number of open aspects to be discussed further. In this paper we give our input to the discussion related Option B-1-1 and the possible UE, network, and system impacts.

Discussion

The solution related to option B-1-1 is based on following assumptions:

- Option B) Perform BM/RLM/BFD based on SSB outside active BWP

  - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

    - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions

Additionally, the objective is:

- For Option B-1-1

  - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1)

Following agreements were made in last meeting:

Issue 3-1: When CD-SSB is outside the active BWP, the existing requirements for SSB-based RLM/BFD/BM are applicable for UE supporting option B-1-1.

Issue 3-2: Existing SSB based RLM/BFD/CBD/BM measurement requirements in clause 8.1, 8.5, 9.5 and 9.8 are to be made applicable for UE supporting option B-1-1.

Issue 3-8: Do not specify assumptions on RF BW and BB BW for UE supporting option B-1-1.

While following issues are open for further discussion:

Issue 3-1: Applicable of existing RLM/BFD/BM requirements for supporting Option B-1-1:

details of applicable condition

Issue 3.3: Clarification on existing timing requirements for supporting Option B-1-1

Issue 3-5: Clarification/modification on inter-frequency measurement requirements for supporting option B-1-1

Issue 3-6: Whether to define requirements for supporting multiple options

Issue 3-7: Whether to define additional requirements for supporting option B-1-1

In the following we shortly address each issue.

Issues 3-1 & 3-2:

As it was agreed that when CD-SSB is outside the active BWP, the existing requirements for SSB-based RLM/BFD/BM are applicable for UE supporting option B-1-1, RAN4 need to define the applicability for the requirements in the related section for Option B-1-1.

Currently, RAN4 has defined following condition for these measurements (using RLM as example):

- UE is not required to perform RLM outside the active DL BWP

Hence, an exception from this condition needs to be defined for UEs supporting Option B-1-1.

One Example TP could be:

- Except for a UE supporting ‘Option B-1-1’, a UE is not required to perform RLM outside the active DL BWP

Similar text could be used in the sections 8.1A.1, 8.1B.1 and 8.1C.1. For BFD/link recovery procedures sections 8.5.1, 8.5A.1, 8.5B.1 and 8.18 would need to be updated.

For Beam Management (BM), L1-RSRP measurements, the TP update would need to be slightly different although with the same meaning/intention. We suggest:

- When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP. A UE supporting ‘Option B-1-1’ shall be able to perform measurements for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements outside the active BWP.

Similar wording ‘within the active BWP’ has been used also other places in 38.133 and similar update might be needed there.

As the changes are needed quite many places in 38.133, we suggest firstly agree on the text before going to CR phase.

We also expect that RAN4 need to define or know how to refer to ‘Option B-1-1’ and which basic assumption a UE supporting Option B-1-1 supports. For example, capturing:

- Option B) Perform BM/RLM/BFD based on SSB outside active BWP

  - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

    - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions

RAN4 to have a definition of ‘Option B-1-1’.

RAN4 to firstly agree on how to refer to ‘Option B-1-1 capable UE’ and the text before going to CR phase.

Issue 3-3:

For option B-1-1 the UE is assumed using a larger BW to cover SSB outside active BWP without interruptions. Hence, when needed, the UE will use wider BW to ensure that SSB based measurements can be performed. The existing timing requirements applicable since Rel-15 account option B-1-1. Hence, there is no reason to clarify the existing timing requirements.

No clarification on existing timing requirements when CD-SSB is outside active BWP is needed.

It is on the other hand important that RAN4 defines which basic assumption a UE supporting Option B-1-1 supports.

Issue 3-5:

We do not see a need to clarify inter-frequency measurement requirements for a UE supporting option B-1-1. We expect that the existing procedures and signalling already can account when CD-SSB is outside the active BWP.

No clarification on existing inter-frequency measurement requirements when CD-SSB is outside active BWP is needed.

Issue 3-6:

Whether RAN4 should define requirements for a UE supporting multiple options is not only related to option B-1-1, but would be a general issue to be discussed – addressing all options.

However, our preference is to wait until any possible capabilities have been clarified and the detailed UE requirements for each option have been defined (when needed).

Whether RAN4 should define requirements for a UE supporting multiple options should be discussed later.

Issue 3-7:

For this solution it is assumed that the CD-SSB is not within the active BWP. Additionally, it is assumed that there will not be any NCD-SSB allocated within the active BWP, and there is no additional CSI-RS allocated for RLM, BFD or BM within the active BWP.

The UE must use the CD-SSB for RLM, BFD and BM even when CD-SSB is not within the active BWP. Additionally, the UE must perform RLM, BFD and BM using the CD-SSB without causing interruption. Hence, the behavior is the same as if the CD-SSB would be within the active BWP.

For this solution we do not see a need to define any new RAN4 requirements.

No new additional UE requirements would be needed for Option B-1-1.

RAN4 would have to update the current conditions clarifying that a UE supporting Option B-1-1 can perform SSB based RLM, BFD and BM, on the CD-SSB when the CD-SSB is not within the active BWP.

RAN4 must update the conditions clarifying that a UE supporting Option B-1-1 can perform SSB-based RLM, BFD and BM, when the CD-SSB is not within the active BWP.

Additionally, it must be clarified that these measurements can be performed without causing interruptions.

RAN4 to clarify that a UE supporting Option B-1-1 can perform SSB-based RLM, BFD and BM, on the CD-SSB not within the active BWP without causing interruptions.

Same conditions would have to be clarified also for SSB-based L3 measurements. Hence, UEs supporting Option B-1-1 must be able to perform SSB-based non-gap assisted intra-frequency measurements on CD-SSB even when not within the active BWP.

RAN4 to clarify that a UE supporting Option B-1-1 can perform SSB-based intra-frequency L3 measurements without gaps when the CD-SSB is not within the active BWP.

Such conditions would need to capture that L3 SSB-based measurements can be performed without gaps and without causing interruptions. Our understanding is based on that a UE supporting B-1-1 can (is a prerequisite) perform intra-frequency measurements without gaps (and interruptions) even if the CD-SSB is outside the active BWP.

Conclusion

In this paper we take a further look at bandwidth part operation without restriction in NR – Option B-1-1. We propose following related to option B-1-1:

- RAN4 to have a definition of ‘Option B-1-1’.

RAN4 to firstly agree on how to refer to ‘Option B-1-1 capable UE’ and the text before going to CR phase.

No clarification on existing timing requirements when CD-SSB is outside active BWP is needed.

No clarification on existing inter-frequency measurement requirements when CD-SSB is outside active BWP is needed.

Whether RAN4 should define requirements for a UE supporting multiple options should be discussed later.

No new additional UE requirements would be needed for Option B-1-1.

RAN4 must update the conditions clarifying that a UE supporting Option B-1-1 can perform SSB-based RLM, BFD and BM, when the CD-SSB is not within the active BWP.

RAN4 to clarify that a UE supporting Option B-1-1 can perform SSB-based RLM, BFD and BM, on the CD-SSB not within the active BWP without causing interruptions.

RAN4 to clarify that a UE supporting Option B-1-1 can perform SSB-based intra-frequency L3 measurements without gaps when the CD-SSB is not within the active BWP.

- References

- RP-230805, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Moderator (Vodafone), vivo, Mar. 2023.

- R4-2306397, WF on completion of specification support for BWP operation without restriction, vivo