3GPP TSG-RAN WG4 Meeting # 107		R4-2308784

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.27.4

Source:	Samsung,

Title:	Discussions on NTN UE RF

Document for:	Discussion

# Introduction

In last #106-bis-e meeting, R4-2306624 concluded RAN4 should further discuss the differences of NTN UE based on the parameters and requirements in terms of regulation, RF and RRM. Hence, in this paper, we try to list the general regulations, implementations of the NTN UE to the group to further discuss the possible way to categorize NTN UE.

# Regulations on NTN UE

From the latest WID, RP-230809, the following services were considered in NR-NTN-enh:

The following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered

  - ESIM scenarios for NGSO in Ka band are not considered in this WI.

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

  - Regarding mobile VSAT, three types of terminal and scenario exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study.

Observation 1: In the WID RP-230809, we initially should consider 1) GSO: Fixed VSAT, and Airborne, Maritime, Land VSAT (i.e. ESIM); 2) NGSO: Fixed VSAT.

Considering the exemplary 27GHz band, from Radio Regulations 2020, the following recommendations and regulations should be applied to the NTN UE:

| SAN | UE | Related ITU-R recommendations and regulations: | Possible impact on RAN4 requirements |
| --- | --- | --- | --- |
| GSO | Fixed VSAT | Recommendations: ITU-R S.524: off-axis eirp limit ITU-R S.1594: Maximum emission level. | - Output power requirement - Antenna characteristics: - parabolic antenna size, efficiency, etc; - phased array: element configuration, beamforming characteristics, etc. |
|  | Airborne VSAT (Aero-ESIM) | Resolves in Resolution 156 for ESIM in FSS in 29.5-30GHz band 1.1 with respect to satellite networks of other administrations, the earth station shall remain within the envelope of the coordination agreements of the satellite networks with which this earth station is associated or, in the absence of such agreements, comply with the off-axis e.i.r.p. density levels given in the Annex; 1.4 in case of interference, the administration responsible for the satellite network shall, upon receipt of a report of harmful interference with respect to any terrestrial systems operating in the countries listed in No. 5.542, immediately cease or reduce the interference to the acceptable level; 1.6 that these earth stations be subject to permanent monitoring and control by a Network Control and Monitoring Centre (NCMC) or equivalent facility and be capable of receiving and acting upon at least “enable transmission” and “disable transmission” commands from the NCMC; Resolves in Resolution 169 for ESIM in FSS in 27.5~29.5GHz 1.1.6 for the protection of non-GSO FSS systems operating in the frequency band 27.5-28.6 GHz, ESIMs communicating with GSO FSS networks shall comply with the provisions contained in Annex 1 to this Resolution; 1.1.7 for the protection of non-GSO MSS feeder links of non-GSO systems for which complete coordination information was received before, and for which feeder-link earth stations were in service as of, 28 October 2019 in the frequency band 29.1-29.5 GHz, ESIMs communicating with GSO FSS networks should consider Annex 2 to this Resolution; 1.2.2 transmitting aeronautical and maritime ESIMs in the frequency band 27.5-29.5 GHz shall not cause unacceptable interference to terrestrial services to which the frequency band is allocated and operating in accordance with the Radio Regulations, and Annex 3 to this Resolution shall apply; 5.1 for the operation of ESIMs, techniques to maintain pointing accuracy with the associated GSO FSS satellite, without inadvertently tracking adjacent GSO satellites, are employed; 5.2 all necessary measures are taken so that ESIMs are subject to permanent monitoring and control by a network control and monitoring centre (NCMC) or equivalent facility in order to comply with the provisions in this Resolution, and are capable of receiving and acting upon at least “enable transmission” and “disable transmission” commands from the NCMC or equivalent facility; | Resolution 156: Resolves 1.1 off-axis e.i.r.p. may impact: - Output power requirement - Antenna characteristics: - parabolic antenna size, efficiency, etc; - phased array: element configuration, beamforming characteristics, etc. Resolves 1.4, 1.6 may impact: - Uplink power control Resolution 169: Resolves 1.1.6 leads to off-axis e.i.r.p may impact: - Output power requirement - Antenna characteristics: - parabolic antenna size, efficiency, etc; - phased array: element configuration, beamforming characteristics, etc. Resolves 1.1.7 leads to e.i.r.p. density limit, off-axis e.i.r.p. mask, carrier burst duty cycle, and number of transmitting ESIMs in a single satellite beam in 15MHz channel, these may impact: - Uplink scheduling and channel allocation; - Output power requirement; - Antenna characteristics: - parabolic antenna size, efficiency, etc; - phased array: element configuration, beamforming characteristics, etc. Resolves 1.2.2 leads to horizontal e.i.r.p. limit, power flux density (pfd) limit on earth surface, and these may impact: - Elevation angle requirement; - Output power requirement; - Antenna characteristics: - parabolic antenna size, efficiency, etc; - phased array: element configuration, beamforming characteristics, etc. |
|  | Maritime VSAT (M-ESIM) |  |  |
|  | Land VSAT (L-ESIM) |  |  |

And Article 21 of Radio Regulation should also be taken into account for the output power limit, horizontal e.i.r.p. limits, etc.

Proposal 1: RAN4 to discuss whether and how to categorize UE with above information in terms of regulation, possible impact on RAN4 spec can be power limit, off-axis pattern, tracking accuracy, schemes for uplink transmission, like cease transmission when tracking failed, or reported interference, etc.

# Mechanical and electrical scanning

The Off-axis e.i.r.p. regulation is majorly given in ITU-R S.524, where it is defined as following:

4	that earth stations operating in GSO networks in the FSS transmitting in the 27.5-30 GHz frequency band be designed in such a manner that at any angle, , which is 2 or more off the main lobe axis of the earth station antenna, the e.i.r.p. density in any direction within 3 of the GSO should not exceed the following values:

Angle off-axis	Maximum e.i.r.p. per 40 kHz

2	    	7	(19 – 25 log ) dB(W/40 kHz)

0	 7	    	9.2	–2 dB(W/40 kHz)

0	 9.2	    	48	(22 – 25 log ) dB(W/40 kHz)

48	    	180	–10 dB(W/40 kHz).

In NTN co-ex study, there’re two types of UE antennas considered initially, one is parabolic antenna, and the other is phased-array antenna.

| Parabolic antenna | Phased array antenna |
| --- | --- |
| R4-2305930: Parabolic antenna pattern for NTN UE: - Use the following parabolic antenna model for NTN UE Fθ=2 J1uu With: - Jix is the Bessel function of first type and ith order with argument x - is the angle in a θ;φ spherical coordinates system, - u=πDλsin⁡(θ) - D : Antenna diameter - : Wavelength Characteristics; fixed VSAT; L-ESIM. Frequency band; 27GHz; 27GHz. Antenna type and configuration; Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter; Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg). Polarisation; circular; circular. Rx Antenna gain; 39.7 dBi; TBD dBi per element. Antenna temperature; 150 K; TBD K. Noise figure; 1.2 dB; TBD dB. Tx transmit power; 2 W (33 dBm); [TBD W (TBD dBm)]. Tx antenna gain; 43.2 dBi; TBD dBi per element. - For calibration, the number of active NTN UE (UL) is 10 UEs with divided RBs per UE by channel bandwidth. - For calibration, NTN UE antenna elevation angle is 30 and 90 degree. - For calibration, NTN UE NF is [TBD]. NOTE 1: FFS whether and how to consider phased array antenna pattern, not preclude to consider phased array antenna pattern in the future if needed.. | R4-2305383: Table 2-1 UE phased array antenna characteristics Ka band UE phased array antenna. 1; Ka band UE phased array antenna Characteristics. 1.1; Antenna pattern; TR 38.921. 1.2; Element gain (dBi) (Note 2); 5.2. 1.3; Horizontal/vertical 3 dB beam width of single element (degree); 90° for H 90° for V. 1.4; Horizontal/vertical front-to-back ratio (dB); 30 dB. 1.5; Antenna polarization; Circular（RHCP or LHCP）. 1.6; Antenna array configuration (Row × Column)(Note 4); 40*40 elements. 1.7; Number of supported polarizations, P; 1. 1.8; Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.5 of wavelength for V. 1.9; Array Ohmic loss (dB) (Note 2); 2. 1.10; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); 8 dBm. 1.11; maximum coverage angle in the horizontal plane (degrees); 0~360 degrees. 1.12; vertical coverage range (degrees) between beam direction and normal direction; 0~60 degrees. 1.13; normal direction; Toward Z+ axis. |

Observation 2: For the parabolic antenna, the mechanical scanning of the beam will not impact the antenna or beam pattern, it will only shift the antenna pattern to the mechanical-tilted directions.

Observation 3: For phased-array antenna, after electrical tilting, the off-axis eirp will exceed the S-524 mask because of the sidelobe distortion due to phased-shift nature of the AAS antenna. In this case, in order to meet S-524 off-axis eirp mask, the NTN UE power had to be reduced to lower its side lobe.

Following figures shows the 40x40 phased array referring to R4-2305383 with adjusted power may results in failure to meet compliance of ITU-R S.524 off-axis eirp mask when it electronically tilting its beam with certain degrees, e.g. 30, 60 in figures.

Proposal 2: RAN4 to discuss whether or how to build requirements for NTN UE with phased array antenna operation to guarantee its compliance to the off-axis eirp mask.

- The output power limit may be reduced when the phased array antenna perform electronical tilting to point its beam to the desired SAN.

- If use same power level, the NTN UE with phased array may also be required to perform some mechanical rotation for its antenna to reduce the electrical tilting range, and to reduce the effect of the side lobe distortion.

- If mechanical rotation is required for both phased array and parabolic antenna NTN UE, discuss whether or how to define requirements for the rotation speed and accuracy.

# Tracking accuracy

Observation 4: The NTN UE, especially those mobile VSAT (Land, maritime, aeroplane ESIM), may operate with turbulence circumstances. Under such situations, the NTN UE may point towards an undesired direction which may cause issue. In order to protect satellite in other orbits, power reduction or fast electronical or mechanical beam adjustment may be requested for NTN UE.

Proposal 3: RAN4 to discuss whether or how to build requirements for tracking accuracy to guarantee its compliance during turbulence circumstances, and how fast should it adjust its beam back to desired direction.

# Interference monitoring

Observation 5: When received interference report, the mobile VSAT (Land, maritime, aeroplane ESIM) is required to perform interference mitigation or stop transmission.

Proposal 4: RAN4 to discuss whether or how to build requirements for interference mitigation, for example to update closed-loop power control based on the interference reporting.

# Conclusion

Observation 1: In the WID RP-230809, we initially should consider 1) GSO: Fixed VSAT, and Airborne, Maritime, Land VSAT (i.e. ESIM); 2) NGSO: Fixed VSAT.

Proposal 1: RAN4 to discuss whether and how to categorize UE with above information in terms of regulation, possible impact on RAN4 spec can be power limit, off-axis pattern, tracking accuracy, schemes for uplink transmission, like cease transmission when tracking failed, or reported interference, etc.

Observation 2: For the parabolic antenna, the mechanical scanning of the beam will not impact the antenna or beam pattern, it will only shift the antenna pattern to the mechanical-tilted directions.

Observation 3: For phased-array antenna, after electrical tilting, the off-axis eirp will exceed the S-524 mask because of the sidelobe distortion due to phased-shift nature of the AAS antenna. In this case, in order to meet S-524 off-axis eirp mask, the NTN UE power had to be reduced to lower its side lobe.

Proposal 2: RAN4 to discuss whether or how to build requirements for NTN UE with phased array antenna operation to guarantee its compliance to the off-axis eirp mask.

- The output power limit may be reduced when the phased array antenna perform electronical tilting to point its beam to the desired SAN.

- If use same power level, the NTN UE with phased array may also required to perform some mechanical rotation for its antenna to reduce the electrical tilting range, and to reduce the effect of the side lobe distortion.

- If mechanical rotation is required for both phased array and parabolic antenna NTN UE, discuss whether or how to define requirements for the rotation speed and accuracy.

Observation 4: The NTN UE, especially those mobile VSAT (Land, maritime, aeroplane ESIM), may operate with turbulence circumstances. Under such situations, the NTN UE may point towards an undesired direction which may cause issue. In order to protect satellite in other orbits, power reduction or fast electronical or mechanical beam adjustment may be requested for NTN UE.

Proposal 3: RAN4 to discuss whether or how to build requirements for tracking accuracy to guarantee its compliance during turbulence circumstances, and how fast should it adjust its beam back to desired direction.

Observation 5: When received interference report, the mobile VSAT (Land, maritime, aeroplane ESIM) is required to perform interference mitigation or stop transmission.

Proposal 4: RAN4 to discuss whether or how to build requirements for interference mitigation, for example to update closed-loop power control based on the interference reporting.

# Reference

[1] R4-2306624, “WF on NTN UE RF requirements in Ka-band”, ZTE

[2] RP-230809, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales

[3] R4-2305930, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung

[4] R4-2305383, “Discussion on Rel-18 NTN coexistence study assumption”, Huawei, HiSilicon