# 3GPP TSG-RAN WG4 Meeting # 107                                  R4-2308825

# Incheon, KR, May 22 – May 26, 2023

# Source: 	ZTE Corporation

# Title: 	Views on HPUE RedCap in FR1

# Agenda item:	11.1

# Document for:	Approval

# 1 Introduction

In last RAN #99 meeting, a WID was proposed[1] on NR power class 2 RedCap (Reduced Capability) UE in FR1. Due to some concerns were raised. Consequently, RAN task RAN4 to provide some initial studies on the two aspects [2] before a dedicated WID is confirm.

Option 3: RAN task RAN4 the following work in the next quarter and come back to RAN#100

To study how to enable HPUE including PC2 for RedCap in the band agnostic way

To study the applicable form factor, e.g., sensor, camera

In last meeting, there were some initial discussions on HPUE RedCap in FR1, and it was summarized in [3], in which the majority companies think it is feasible to enable PC2 for RedCap HPUE, while few of companies showed concern on the applicable form factor.

Meanwhile, the companion WF was agreed in [4]. In this contribution, we give some further discussions.

# 2	Discussion

In [5], we have already recalled some agreements on the FR1 RedCap UE in Rel-17, including the situation for PC2 FR1 RedCap UE in Rel-17.

In the WF, the first issue is form factor for PC2 Redcap HPUE, which are:

### Issue 1: Form factor for PC2 RedCap HPUE Candidate options for further discussion: - Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE in the specification - Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification

In our understanding, whether or not support PC2 RedCap HPUE depends on UE implementation. Actually, for PC3 RedCap UE in Rel-17, although the applicable use cases were identified, there was no restriction on the applicable form factor for FR1 RedCap UE in the specification. For FR2 RedCap UE, except the 3 identified use cases (i.e. Industry sensor,video surveillance, wearables), the other non-RedCap UE types are not precluded to support RedCap (i.e.RedCap variants), which is shown as below:

Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |
| Note: RedCap variants of non-RedCap UEs are not precluded |  |

Actually, RAN4 spent several meeting cycles to discuss the applicable use cases for RedCap UE in Rel-17, while in the end, no restriction on the applicable form factor in the specification. In addition, PC2 power class is optional reported by RedCap UE. So we think there is no need to restrict the applicable form factor for PC2 RedCap HPUE in the specification. Therefore, we have the following proposal:

Proposal 1. Not restrict the applicable form factor for PC2 RedCap HPUE in the specification

The next issue is how to specify PC2 RedCap UE, which are:

### Issue 2: How to specify HPUE including PC2 for RedCap Way forward: - PC2 RedCap HPUE requirements can be specified in the band agnostic way at least for TDD and HD-FDD. - The band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE. - Further discuss whether the band agnostic way can be applied for PC2 FD-FDD bands with 1Rx and 2Rx, based on more analysis on the RF requirements including A-MPR and RSD. - Note 1: 1Tx architecture is assumed for RedCap PC2. Note 2: PC2 for Rel-17 Redcap is considered under this RAN task.

The reasons for band agnostic way have already been elaborated in companies’ contribution [3] in last meeting. For TDD and HD-FDD RedCap UE, due to there are no additional requirements like A-MPR/RSD, there are no problems to specify TDD and HD-FDD PC2 RedCap UE in the band agnostic way.

For FD-FDD band, there is a ongoing basket WID [6] to introduce some PC2 FDD band, in which A-MPR and RSD requirements are discussed including 1Tx and 2Tx architecture. Currently, there are FDD bands of n8, n5, n28, n26, n25, n66, n71, n85, n2, n13 included in basket WID, that is not all the FDD band supports PC2.

Observation 1. Several (Not all) FDD bands support PC2 which are included in HPUE_NR_FR1_FDD_R18 basket WID

As similar as PC3 RedCap UE, the PC2 FDD non-RedCap UE requirements with 1Tx architectures and channel bandwidth up to 20MHz can be applied to PC2 FDD RedCap UE, we think there is no need to re-discuss the A-MPR and RSD requirements for PC2 FDD RedCap UE in the case of the supported operating bands PC2 for FDD RedCap are the same with PC2 FDD non-RedCap bands, and in this case band agnostic way can also be applied for PC2 FD-FDD RedCap UE. Otherwise, non-band agnostic way should be applied and A-MPR/RSD requirements should be discussed.

Observation 2. Band agnostic way can also be applied for PC2 FD-FDD RedCap UE in the case of the supported operating bands for PC2 FD-FDD RedCap are the same with PC2 FDD non-RedCap bands. Otherwise, band agnostic way cannot be applied, and A-MPR/RSD requirements should be discussed.

Currently, there is no sub-clause to define the supported operating band for PC3 RedCap UE. Instead general texts were used in sub-clause 5.3I and sub-clause 6.2I. Considering there were already two FDD band, i.e. band n1 and n3, defined in the spec, so by modifying the wordings a bit in sub-clause 6.2I to apply the same PC2 FDD non-RedCap bands for PC2 RedCap, which was proposed in the draft CR [7]:

    - 6.2I.1	Maximum output power for RedCap

For Redcap UE, the requirements for power class 3 and power class 2 specified in clause 6.2.1 apply.

The above modifications imply the supported operating bands for PC2 FDD RedCap are the same with PC2 FDD non-RedCap bands. So the A-MPR and RSD(1Tx) with the channel bandwidth up to 20MHz of PC2 FDD non-RedCap can be applied for PC2 FDD RedCap UE.

Proposal 2. By applying same PC2 FDD non-RedCap bands for PC2 FD-FDD RedCap UE, band agnostic way can be applied for PC2 FD-FDD RedCap.

# 3 Conclusion

In this contribution, we give some discussion on PC2 RedCap UE, the conclusions are summarized below:

Observation 1. Several (Not all) FDD bands support PC2 which are included in HPUE_NR_FR1_FDD_R18 basket WID.

Observation 2. Band agnostic way can also be applied for PC2 FD-FDD RedCap UE in the case of the supported operating bands for PC2 FD-FDD RedCap are the same with PC2 FDD non-RedCap bands. Otherwise, band agnostic way cannot be applied, and A-MPR/RSD requirements should be discussed.

Proposal 1. Not restrict the applicable form factor for PC2 RedCap HPUE in the specification.

Proposal 2. By applying same PC2 FDD non-RedCap bands for PC2 FD-FDD RedCap UE, band agnostic way can be applied for PC2 FD-FDD RedCap.

# 4	Reference

[1] RP-230312, New WID: NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, MediaTek

Replaces

[2] RP-230763, Summary of offline discussions about new spectrum related WIs, RAN4 chair (Huawei)

[3] R4-2306314, Email discussion summary for [106bis-e][152] RAN_task_UERFWF on the RedCap RF, Ericsson, Moderator (China Telecom)

[4] R4-2306650, WF on RedCap HPUE, China Telecom

[5] R4-2305127, Views on HPUE RedCap in FR1, ZTE

[6] Revised WID: High power UE (power class 2) for NR FR1 FDD single band, China Unicom

[7] Draft CR for enabling PC2 for Redcap UE, China Unicom