3GPP TSG-RAN WG4 Meeting #107             	R4-2308830

Incheon, South Korea, 22th May-26th May, 2023

Agenda item:	8.13.5.2

Source:	Samsung

Title:	View on PDSCH requirements with CA

Document for:	Discussion

- Introduction

In RAN#97-e meeting, the updated WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2) was approved in [1]. In the last RAN4 meeting, the deployment, test scenario, channel model and test scope for demodulation requirements were under discussion. The related agreements were captured in the WF [2].  Regarding the test scope and test setup for PDSCH requirement with CA, the related agreement was made as following

- Test cope of intra-band CA requirement - RAN4 to only introduce new HST FR2 CA requirements for PDSCH - Test cases - Specify two sets of PDSCH requirements for FR2 HST intra-band CA scenario - HST-DPS-FR2-BI-B: Uni-directional scenario A with DPS scheme 1b - HST-DPS-FR2-BI-A: Bi-directional scenario B with DPS scheme 1a - Apply the Rel-17 FR2 HST channel model parameters to PDSCH CA demodulation requirements with DPS transmission schemes - SCS & BW - Specify CA PDSCH requirements for FR2 HST with component carrier configuration as - {50, 100, 200, 400} MHz for 120 KHz SCS - Companies are encouraged to check whether existing SNR requirement for 200MHz CBW defined in single carrier can be reused for CA case with 200MHz in the next meeting pending on the collected impairment results - FFS on the carrier combinations considered for requirement - Other test parameters - Reuse the exiting test parameters of FR2 HST with DPS 1a and DPS 1b in single carrier for FR2 HST PDSCH intra-band CA requirements - SCS: 120KHz only - TDD pattern: FR2.120-1 (DDDSU) - MCS 17 and Rank2 - Other parameters such as number of HARQ process, K1 values for different CCs can be used from NR Rel-16 FR2 normal CA - Test Applicability rule - Applicability rule for different CA configuration and bandwidth combination sets can be reused from NR Rel-16 normal CA as baseline - Others - FFS on the necessity of considering additional margin for CA requirement based on single carrier simulation with considering imperfection model - FFS on how to consider the imperfection model if considered

In this contribution, the view on remaining issue of test setup for PDSCH requirement with CA is provided.

# 2	Discussion

Carrier Combination

Based on the agreements in Rel-16 WI on NR performance requirement enhancement, only 1 case include 120KHz+120KHz for FR2 NR CA. Since only 120KHz SCS requirement is introduced for Rel-17 FR2 HST, considering it is not feasible to support large doppler in 60KHz SCS. Therefore, only TDD with 120KHz SCS+TDD with 120KHz SCS are feasible for specifying PDSCH requirement with CA.

Proposal 1: Consider TDD 120KHz SCS + TDD 120KHz SCS for carrier combination

Release independent

Rel-17 FR2 HST is mainly focusing on single carrier scenario with new PC6 UE introduced to support FR2 HST scenario. Considering carrier aggregation is necessary to improve the throughput, it is proposed to introduce Rel-18 FR2 HST PDSCH requirement with CA from Rel-17.

Proposal 2: it is proposed that Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

Other

In Rel-16 WI on NR performance requirement enhancement, FR2 CA PDSCH requirement was introduced, with any assumption that additional margin is considered, where CA requirements can be directly derived by single carrier cases.  Similar as Rel-17 FR1 HST, PDSCH requirement with CA has also introduced with similar rule applied. Therefore, for FR2 HST CA requirement, we prefer to apply the same rule for specifying the FR2 CA PDSCH requirement.

Proposal 3: Not consider additional margin for CA requirement based on single carrier. The performance requirements specified in the section of CA requirement do not apply for UE single carrier test.

# 3	Conclusion

In this contribution, the view on the remaining issue of test setup for PDSCH requirement with CA is provided.

Proposal 1: Consider TDD 120KHz SCS + TDD 120KHz SCS for carrier combination

Proposal 2: it is proposed that Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

Proposal 3: Not consider additional margin for CA requirement based on single carrier. The performance requirements specified in the section of CA requirement do not apply for UE single carrier test.

# References

RP- 222272, Revised WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2)

R4-2305892	WF on NR FR2 HST Enhancements RRM requirements		Samsung