3GPP TSG-RAN WG4 Meeting # 107	R4-2308862

# Incheon, KR, May 22 – May 26, 2023

Title: 	Discussions on UE demodulation requirements for intra-band non collocated EN-DC/NR-CA deployment scenario

Source: 	Huawei, HiSilicon

Agenda item:	8.12.4

Document for:	Discussion

Background

As per [1], some open issues on demodulation requirements for this WI were left. This paper provides our views on related open issues

Discussions

UE type assumption

Based on the latest progress from RF session, both type 3a/3b and type 4a/4b UEs have been precluded in Rel-18. Therefore, demodulation session can only focus on type 2 UE discussion.

Proposal 1: only focus on demodulation requirements discussion for type 2 UE.

Requirements definition for Type 2 UE

Based on our understanding, RF session agreed to consider type 1 UE as baseline and type 2 as optional feature specified for non-collocated scenario. Type 2 UE has two independent RF chains but no special updates to baseband processing. Hence we propose to not define requirements for type 2 UE.

Observation 1: Type 2 UE has no updates to the baseband processing compared to baseline architecture.

Proposal 2: Don’t define demodulation requirements for type 2 UE

Conclusion

This paper provide our views on UE requirements for intra-band non collocated EN-DC/NR-CA deployment scenario. The proposals and observations are:

Proposal 1: Preclude type 3a/3b and type 4a/4b UE in performance requirements definition and only focus on type 2 UE.

Observation 1: Type 2 UE has no updates to the baseband processing compared to baseline architecture.

Proposal 2: Don’t define requirements for type 2 UE

- 4   Reference

- R4-2305891 WF on UE demodulation requirements for intra-band non-collocated EN-DC/NR-CA deployment scenario