3GPP TSG-RAN WG4 Meeting # 107	R4-2308874

# Incheon, KR, May 22 – May 26, 2023

Title: 	Discussions on 4Tx PUSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.5.3.2

Document for:	Discussion

Background

As per [1], some open issues were left for 4Tx PUSCH performance requirements. In this paper, we provide our views

Simulation results

The simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions for 4Tx PUSCH requirements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

The simulation results are captured in Table 2-2:

Table 2-2: Summary of simulation results for 4Tx PUSCH requirements

| SCS | Bandwidth | PUSCH mapping type | Propagation conditions | MCS | Antenna | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 5MHz | Type A | TDLB100-400 | 2 | 4T4R | -0.3 |
|  |  |  |  |  | 4T8R | -3.7 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.2 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.0 |
|  |  |  |  |  | 4T8R | 9.0 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | -0.3 |
|  |  |  |  |  | 4T8R | -3.7 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.3 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 16.9 |
|  |  |  |  |  | 4T8R | 9.0 |
|  | 20MHz | Type A | TDLB100-400 | 2 | 4T4R | 0.1 |
|  |  |  |  |  | 4T8R | -3.4 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.5 |
|  |  |  |  |  | 4T8R | 5.5 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.7 |
|  |  |  |  |  | 4T8R | 9.2 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | 0.0 |
|  |  |  |  |  | 4T8R | -3.5 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.6 |
|  |  |  |  |  | 4T8R | 5.6 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.7 |
|  |  |  |  |  | 4T8R | 9.2 |
|  | 50MHz | Type A | TDLB100-400 | 2 | 4T4R | 0.1 |
|  |  |  |  |  | 4T8R | -3.5 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.6 |
|  |  |  |  |  | 4T8R | 5.5 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 18.5 |
|  |  |  |  |  | 4T8R | 9.6 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | 0.1 |
|  |  |  |  |  | 4T8R | -3.5 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.6 |
|  |  |  |  |  | 4T8R | 5.6 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 18.5 |
|  |  |  |  |  | 4T8R | 9.6 |
| 30kHz | 10MHz | Type A | TDLB100-400 | 2 | 4T4R | -0.7 |
|  |  |  |  |  | 4T8R | -4.2 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.1 |
|  |  |  |  |  | 4T8R | 5.3 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 16.8 |
|  |  |  |  |  | 4T8R | 9.0 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | -0.7 |
|  |  |  |  |  | 4T8R | -4.1 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.1 |
|  |  |  |  |  | 4T8R | 5.3 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 16.7 |
|  |  |  |  |  | 4T8R | 9.0 |
|  | 40MHz | Type A | TDLB100-400 | 2 | 4T4R | -0.4 |
|  |  |  |  |  | 4T8R | -3.8 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.3 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.2 |
|  |  |  |  |  | 4T8R | 9.0 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | -0.4 |
|  |  |  |  |  | 4T8R | -3.7 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.4 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.3 |
|  |  |  |  |  | 4T8R | 9.0 |
|  | 100MHz | Type A | TDLB100-400 | 2 | 4T4R | -0.4 |
|  |  |  |  |  | 4T8R | -3.7 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.4 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 18.3 |
|  |  |  |  |  | 4T8R | 9.5 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | -0.3 |
|  |  |  |  |  | 4T8R | -3.6 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.5 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 18.4 |
|  |  |  |  |  | 4T8R | 9.5 |

Discussions

One issue is bandwidth selection, candidate options are shown as follows:

Bandwidth selection

Agreement: - For 15kHz: - Introduce requirements for 5MHz and 50MHz, FFS on 20MHz - For 30kHz: - Introduce requirements for 10MHz and 100MHz, FFS on 40MHz

For MCS2 and 12, the performance difference is negligible between minimum bandwidth and maximum bandwidth. However, for MCS17, there is 1.6~1.7dB difference between minimum bandwidth and maximum bandwidth for 4Rx and 0.5~0.6 dB for 8Rx. Considering the performance difference for MCS17, we can compromise to introduce performance requirements for medium bandwidth for MCS17 only. I.e. 20MHz for 15kHz and 40MHz for 30kHz. Meanwhile, legacy applicability rules for supporting different bandwidth should be reused.

Proposal 1: Introduce requirements for 5MHz, 20MHz and 50MHz for 15kHz and 10MHz, 40MHz and 100MHz for 30kHz for MCS17 only. Meanwhile, legacy applicability rules for supporting different bandwidth should be reused.

Applicability rule

One issue is applicability rule, candidate options are shown as follows:

Applicability rule Way forward: - Option 1: Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the 4 connectors and the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. - Option 2: Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

Option 2 was agreed in February meeting. In the email discussion of May meeting, some companies concern that there is large performance difference between 4Rx and 8Rx. However, our understanding is that compared to 4Rx, 8Rx needs higher requirements for implementation, so there is no necessity to test 4Rx again. Our another concern is that it may be unrealistic for BS to use 4Rx to receive PUSCH with 4MIMO layers if supporting 8Rx due to the high SNR range for 4Rx.

Proposal 2: Use Option 2 as applicability rules for supporting different Rx.

Conclusion

In this paper, we provide our views on 4Tx PUSCH requirements. The proposals and observations are:

Proposal 1: Introduce requirements for 5MHz, 20MHz and 50MHz for 15kHz and 10MHz, 40MHz and 100MHz for 30kHz for MCS17 only. Meanwhile, legacy applicability rules for supporting different bandwidth should be reused.

Proposal 2: Use Option 2 as applicability rules for supporting different Rx.

- 4   Reference

- R4-2305890 WF on FR1 4Tx demodulation requirement. Ericsson