3GPP TSG-RAN WG4 Meeting # 107	R4-2308875

# Incheon, KR, May 22 – May 26, 2023

Title: 	Simulation results on 4TX PUSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.5.3.2

Document for:	Discussion

Background

In this paper, we will provide our simulation results for 4Tx BS demodulation requirements as per [1].

Simulation results

The simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions for 4Tx PUSCH requirements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

The simulation results are captured in Table 2-2:

Table 2-2: Summary of simulation results for 4Tx PUSCH requirements

| SCS | Bandwidth | PUSCH mapping type | Propagation conditions | MCS | Antenna | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 5MHz | Type A | TDLB100-400 | 2 | 4T4R | -0.3 |
|  |  |  |  |  | 4T8R | -3.7 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.2 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.0 |
|  |  |  |  |  | 4T8R | 9.0 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | -0.3 |
|  |  |  |  |  | 4T8R | -3.7 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.3 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 16.9 |
|  |  |  |  |  | 4T8R | 9.0 |
|  | 20MHz | Type A | TDLB100-400 | 2 | 4T4R | 0.1 |
|  |  |  |  |  | 4T8R | -3.4 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.5 |
|  |  |  |  |  | 4T8R | 5.5 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.7 |
|  |  |  |  |  | 4T8R | 9.2 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | 0.0 |
|  |  |  |  |  | 4T8R | -3.5 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.6 |
|  |  |  |  |  | 4T8R | 5.6 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.7 |
|  |  |  |  |  | 4T8R | 9.2 |
|  | 50MHz | Type A | TDLB100-400 | 2 | 4T4R | 0.1 |
|  |  |  |  |  | 4T8R | -3.5 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.6 |
|  |  |  |  |  | 4T8R | 5.5 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 18.5 |
|  |  |  |  |  | 4T8R | 9.6 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | 0.1 |
|  |  |  |  |  | 4T8R | -3.5 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.6 |
|  |  |  |  |  | 4T8R | 5.6 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 18.5 |
|  |  |  |  |  | 4T8R | 9.6 |
| 30kHz | 10MHz | Type A | TDLB100-400 | 2 | 4T4R | -0.7 |
|  |  |  |  |  | 4T8R | -4.2 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.1 |
|  |  |  |  |  | 4T8R | 5.3 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 16.8 |
|  |  |  |  |  | 4T8R | 9.0 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | -0.7 |
|  |  |  |  |  | 4T8R | -4.1 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.1 |
|  |  |  |  |  | 4T8R | 5.3 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 16.7 |
|  |  |  |  |  | 4T8R | 9.0 |
|  | 40MHz | Type A | TDLB100-400 | 2 | 4T4R | -0.4 |
|  |  |  |  |  | 4T8R | -3.8 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.3 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.2 |
|  |  |  |  |  | 4T8R | 9.0 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | -0.4 |
|  |  |  |  |  | 4T8R | -3.7 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.4 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 17.3 |
|  |  |  |  |  | 4T8R | 9.0 |
|  | 100MHz | Type A | TDLB100-400 | 2 | 4T4R | -0.4 |
|  |  |  |  |  | 4T8R | -3.7 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.4 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 18.3 |
|  |  |  |  |  | 4T8R | 9.5 |
|  |  | Type B | TDLB100-400 | 2 | 4T4R | -0.3 |
|  |  |  |  |  | 4T8R | -3.6 |
|  |  |  | TDLC300-100 | 12 | 4T4R | 12.5 |
|  |  |  |  |  | 4T8R | 5.4 |
|  |  |  | TDLA30-10 | 17 | 4T4R | 18.4 |
|  |  |  |  |  | 4T8R | 9.5 |

Conclusion

In this paper we provide our simulation results for 4TX BS demodulation requirements.

- 4   Reference

- R4-2305890 WF on FR1 4Tx demodulation requirement. Ericsson