3GPP TSG-RAN WG4 Meeting #107	R4-2308884

# Incheon, 22nd - 26th May, 2023

Title: 	Simulation results on NR UE ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.14.5.2

Document for:	Discussion

- Background

During RAN4#106-bis-e meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we provide our initial simulation results about NR UE ATG demodulation requirements.

- Simulation

The ideal simulation results for NR UE ATG demodulation requirements are shown as following Table 2-1.

Table 2-1 Ideal simulation results for NR UE ATG demodulation requirements

| Case number | SCS | Bandwidth | Antenna configuration | Propagation condition | Rank | SNR@70% max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MCS13 | MCS17 | MCS24(Table2) |
| 1 | 15kHz | 10MHz | 2x2 | AWGN + 220Hz Doppler | 1 | 3.71 | 6.93 | 22.55 |
| 2 | 15kHz | 10MHz | 2x4 | AWGN + 220Hz Doppler | 1 | 1.23 | 4.50 | 19.85 |
| 3 | 30kHz | 40MHz | 2x2 | AWGN + 500Hz Doppler | 1 | 3.73 | 7.04 | 24.08 |
| 4 | 30kHz | 40MHz | 2x4 | AWGN + 500Hz Doppler | 1 | 1.25 | 4.61 | 21.38 |

The maximum throughput cannot achieve the maximum throughput under 256QAM.

- Proposals

In this contribution, we provide our initial simulation results about NR UE ATG demodulation requirements. Our observations and proposals are:

- The maximum throughput cannot achieve the maximum throughput under 256QAM.

- Reference

[1]	R4-2305899, WF for ATG demodulation requirements, RAN4#106-bis-e, CMCC