3GPP TSG-RAN WG4 Meeting #107	R4-2308886

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.8.4.3

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception.

Discussion

Test parameters

Number of CSI-RS ports

- Proposals - Option 1: 8-port per TRP. - Option 2: Define CSI requirements with 4-port per TRP as a minimum. In addition to requirements with 4-port per TRP, requirements with 8-port per TRP can be included. - Option 3: 4-port per TRP. - Option 4: 2-port per TRP.

Considering that we are defining the minimum requirements, more test coverage should be ensured. If 8-port per TRP is selected, there will no test case for the UE supporting only 4-port per TRP. Also, in FR1 discussion, 4 ports per TRP is considered for the multi-TRP PMI reporting cases, it is expected that for FR2 multi-Rx 4-port per TRP configuration similarly has enough obvious performance gain for the test. Therefore, we propose to align with the FR1 multi-TRP PMI reporting cases for the number of CSI-RS ports selection for FR2 multi-Rx PMI reporting requirements for single-DCI transmission scheme.

4-port per TRP for FR2 multi-Rx PMI reporting requirements for single-DCI transmission scheme.

Whether to assume LI in report quantity for multi-Rx FR2

- Proposals - Option 1: Include LI in the report quantity to facilitate PTRS port mapping. - Option 2: No

We don’t think it is needed to report LI for FR2 multi-Rx. In existing FR2 single-TRP CSI reporting cases PTRS is also configured but RAN4 didn’t consider this report quantity. So we propose to align with the legacy CSI reporting requirements to only consider cri-RI-PMI-CQI report quantity for multi-Rx FR2.

Only consider cri-RI-PMI-CQI report quantity for multi-Rx FR2.

Proposals

In this contribution, we discuss on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- 4-port per TRP for FR2 multi-Rx PMI reporting requirements for single-DCI transmission scheme.

Only consider cri-RI-PMI-CQI report quantity for multi-Rx FR2.

Reference

R4-2306000, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#106-bis-e, Qualcomm