3GPP TSG-RAN WG4 Meeting #107	R4-2308891

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on UE CA demodulation requirements for HST FR2

Source: 	Huawei, HiSilicon

Agenda item:	8.13.5.2

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] on demodulation requirements for Rel-18 FR2 HST was approved. In this contribution, we share our views about UE CA demodulation requirements for HST FR2.

Discussion

CA configurations

Agreement - Specify CA PDSCH requirements for FR2 HST with component carrier configuration as - {50, 100, 200, 400} Hz for 120KHz SCS - Companies are encouraged to check whether existing SNR requirement for 200MHz CBW defined in single carrier can be reused for CA case with 200MHz in the next meeting pending on the collected impairment results - FFS on the carrier combinations considered for requirement

As per current TS 38.101-4, only TDD 120 kHz + TDD 120 kHz case is defined for legacy FR2 CA. Also only 120kHz SCS is considered in Rel-17 HST FR2 single carrier demodulation requirements. It is obviously that 60kHz is not typical scenario. So we prefer to only consider TDD 120 kHz + TDD 120 kHz case for FR2 HST CA scenario. Considering that we have already define 200MHz case in Rel-17, the SNR for the corresponding case can be reused. Simulation is still needed for other bandwidths, i.e. 50MHz, 100MHz and 200MHz.

Only consider TDD 120 kHz + TDD 120 kHz case for FR2 HST CA scenario.

Reuse SNR for 200MHz case from Rel-17 FR2 HST single carrier case, and run simulation for other bandwidths, i.e. 50MHz, 100MHz and 200MHz.

Others

Way forward: - FFS on the necessity of considering additional margin for CA requirement based on single carrier simulation with considering imperfection model - FFS on how to consider the imperfection model if considered

From our understanding, the CA requirements can be directly derived by single carrier simulation. In the simulation it is no need to additionally consider the impact of addition noise that is the same way we did for the FR2 normal CA requirements.

Do not consider additional margin for CA requirement based on single carrier simulation with considering imperfection model.

Proposals

In this contribution, we discuss on UE demodulation requirements for HST FR2. Our observations and proposals are:

- Only consider TDD 120 kHz + TDD 120 kHz case for FR2 HST CA scenario.

Reuse SNR for 200MHz case from Rel-17 FR2 HST single carrier case, and run simulation for other bandwidths, i.e. 50MHz, 100MHz and 200MHz.

Do not consider additional margin for CA requirement based on single carrier simulation with considering imperfection model.

Reference

R4-2305892, WF on demodulation requirements for Rel-18 FR2 HST, RAN4#106-bis-e, Samsung