3GPP TSG-RAN WG4 Meeting #107	R4-2308922

Incheon, Republic of Korea, 22-26 May, 2023

Source:	Ericsson

Title:	On timing requirements in FR2 multi-RX

Agenda item:	8.8.3.6

Document for:	Approval

# 1	Introduction

In WF [1]:

… Issue 3-1-1: Whether to consider RTD larger than CP in multi-RX WI Agreed to postpone the discussion of MRTD > CP until RAN4#107. Issue 3-1-2: Others Way forward: Please bring further analysis on the below proposal. - RAN4 to study impact of receive timing differences from mTRPs on beam pair selection for group-based beam reporting Issue 3-1-3: MRTD for UE not supporting RTD>CP Agreed to postpone the discussion of MRTD > CP until RAN4#107. …

# 2	Discussion

In general we have:

MRTD> TAE+ ∆RF_propagation_difference +∆Channel_dispersion

Where:

- TAE is time alignment error.

- ∆RF_propagation_difference is time of arrival difference of DL signals.

- ∆Channel_dispersionis the radio channel delay dispersion of signals

In [2] LS reply to RAN1 from RAN4 in MIMO Evolution WI have agreed large MRTD and MTTS for a capable UE:

RAN4 thanks RAN1 for the LS on maximum uplink timing difference between the two TAs for multi-DCI multi-TRP with two TAs. After RAN4 further discussion, following values are agreed as MTTD values.

For a UE capable of supporting Receive Time Difference (RTD) > CP, MRTD/MTTD value for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs.

Observation 1: LS reply to RAN1 from RAN4 in MIMO Evolution WI have agreed large MRTD and MTTS for a capable UE for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs.

In FR2-1 we have SCS = 60 kHz and  SCS = 120 kHz in UL/DL and SCS = 240 kHz for SSB DL. This means that CP goes from 36 Ts and 18 Ts down to 9 Ts. This corresponds to differences of 350 m to 88 m of TRP displacement if margin for radio channel dispersion is completely ignored.

Observation 2: FR2 intra band with MRTD < CP will limit MRTD deployment.

In Figure 1 a typical FR2 Multi RF use case is shown. The UE has separate panels. It is reasonable to assume that a UE with separate panels has separate timing and FFT.

Figure 1: Multi panel UE example [1]

Observation 3: The UE has separate panels. It is reasonable to assume that a UE with separate panels has separate timing and FFT.

Proposal 1: For intra cell multi–RX FR2 MRTD > CP, assume MRTD or 8 µs and MTTD or 8.5 µs, for a capable UE.

# 3	Summary

Observation 1: LS reply to RAN1 from RAN4 in MIMO Evolution WI have agreed large MRTD and MTTS for a capable UE for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs.

Observation 2: FR2 intra band with MRTD < CP will limit MRTD deployment.

Observation 3: The UE has separate panels. It is reasonable to assume that a UE with separate panels has separate timing and FFT.

Proposal 1: For intra cell multi–RX FR2 MRTD > CP, assume MRTD or 8 µs and MTTD or 8.5 µs, for a capable UE.

# References

[1] 		R4- 2306319, WF on FR2_multiRx_part2, Ericsson.

[2]		R4-2217279, Reply LS on MTTD for multi-DCI multi-TRP with two Tas, Ericsson

[3]		R4-2300455, Discussion on receive timing difference between different directions, Samsung.