3GPP TSG RAN WG4 Meeting #107		 R4-2308929

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.24.2.1

Source: 	MediaTek Inc.

Title: 	Discussion on resolving ambiguity issue on Tx switching

Document for:	Discussion

1. Introduction

In RAN4 #106, a WF[1] has been agreed for R18 Tx switching for single-TAG. There are still some remaining issues requiring continue discussion in this RAN4 meeting. RAN4 also discover the DL interruption issue due to mandatory simultaneous Rx/Tx operation for many band combos. In this contribution we mainly focus on the case 2 Tx chains are switched between two different band pairs for one Tx switching instance.

2. Discussion

2.1 Single-TAG

Background

RAN4 Agreements captured in the LS R4-2220548 (RAN4 #105):

For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, and it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}, RAN4 agreed that:

- As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}.

Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

In RAN4#106-bis-e, it was agreed to study the benefit to resolve the switching pattern ambiguity issue for two TX concurrent switching operation in WF[1] as a starting point. The switching period depends on UE implementation so it is possible that UE may have different switching period for different switching band pair. In this contribution we will focus on the case 2 Tx chains are switched between two different band pairs for one Tx switching instance. For bands A/B/C/D for one Tx switching instance from {1T,1T,0,0} to {0,0,1T,1T}, considering  it is not clear whether UE performs Tx switching either {from band A to C + B to D} or {from band A to D + B to C}, both TX chains would not be able to transmit during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. When UE reports different switch periods between max{Tswitch_A-C, Tswitch_B-D} and max{Tswitch_A-D, Tswitch_B-C}, the network could only reserve a gap with longer switching period whichever been reported.

Observation 1: While keeping previous agreement, neither of the two Tx chains is expected to be used for transmission during the switching periods, network can only reserve a gap with max{Tswitch_A-C, Tswitch_B-D} or max{Tswitch_A-D, Tswitch_B-C} whichever is longer if ambiguity issue is not resolved.

Considering 2 Tx chains are switched between two different band pairs for one Tx switching instance, if the switching configuration can be explicitly indicated when there are different switching periods reported by UE, the uplink gap may be saved by ∆2-1, ∆3-1 or ∆3-2 as illustrated in Figure 1. For example, if UE reports max{Tswitch_A-C, Tswitch_B-D}=SW1,

max{Tswitch_A-D, Tswitch_B-C}=SW2, where SW2 > SW1

it can earn ∆2-1 for uplink transmission utilization improvement.

Figure 1. Potential improvement on UL transmission switching gap

Observation 2: There may be uplink transmission utilization improvement, ∆2-1, ∆3-1 or ∆3-2, if switching band pairs can be explicitly indicated without impact on RAN1 requirements.

If the switching configuration can be represented in order of switching bands for example, a band pair “Band B+A” is configured, this implicitly indicate Band B is at Tx chain #1, Band A is at Tx chain #2. Under such representation, if Tx switching is configured from “B+A” to “C+D”, it means exactly that in Tx chain #1, Band B uplink is switched to Band C, and in Tx chain #2, Band A is switched to Band D. The ambiguity is resolved without any additional signaling overhead with such representation. Blanking max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C} would not be required.

Proposal 1:

Enhancements for Rel-18 Tx switching

o	Resolving the switching pattern ambiguity issue if it is determined that it is possible that 2 Tx chains are switched between two different band pairs for one Tx switching instance

      - By resolving the switching pattern ambiguity issue, the switching period can be the  switching capability of switched band pairs.

o	RAN4 agrees UE indication capability to resolve the ambiguity issue of the exact switching band pairs when concurrent switching occurs

An example signaling design is provided for reference:

- UE capability reporting

BandCombination-UplinkTxSwitch-v1800 ::= SEQUENCE {

bandCombination-v1800                     BandCombination-v1800                OPTIONAL,

supportedBandQuadListNR-v1800 SEQUENCE(SIZE(1..maxULTxSwitchingBandQuads)) OF ULTxSwitchingBandQuad-r18 OPTIONAL,

uplinkTxswitchingWithoutAmbiguity-r18, ENUMERATED, OPTIONAL,

…

}

ULTxSwitchingBandQuad-r18 ::= SEQUENCE {

bandIndexUL1-r18 INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r18 INTEGER(1..maxSimultaneousBands),

bandIndexUL3-r18 INTEGER(1..maxSimultaneousBands),

bandIndexUL4-r18 INTEGER(1..maxSimultaneousBands),

preferredBandPairsTxChain1 SEQUENCE(SIZE(1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPairPrefer-r18 OPTIONAL,

preferredBandPairsTxChain2 SEQUENCE(SIZE(1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPairPrefer-r18 OPTIONAL,

…

}

ULTxSwitchingBandPairPrefer-r18 ::={

bandIndexUL1-switch-from-r18 INTEGER(1..4),

bandIndexUL1-switch-to-r18   INTEGER(1..4),

uplinkTxSwitchingPeriod-r18  ENUMERATED {nXus, nYus, nZus},

}

- Network configuration signaling

uplinkTxSwitchingBandQuadEnable-r18 ::= SEQUENCE {

bandIndexUL1-r18 INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r18 INTEGER(1..maxSimultaneousBands),

bandIndexUL3-r18 INTEGER(1..maxSimultaneousBands),

bandIndexUL4-r18 INTEGER(1..maxSimultaneousBands),

…

}

The order of bandIndexUL1-r18, bandIndexUL2-r18, bandIndexUL3-r18 and bandIndexUL4-r18 implicitly represents the relationship of switch-from and switch-to for each Tx chain. In particular, when UE receives the command uplinkTxSwitchingBandQuadEnable-r18, UE needs to interpret the switching as

- At Tx Chain #1, switching from bandIndexUL1-r18 to bandIndexUL2-r18

- At Tx Chain #2, switching from bandIndexUL3-r18 to bandIndexUL4-r18

For example, if a UE is configured with 4 bands {n1, n3, n5, n7}, and the UE transmits n1 and n3 simultaneously, UE may report an optional preference on preferredBandPairsTxChain1 = n1-n5, preferredBandPairsTxChain2 = n3-n7 as reference information.

And if network scheduler decides to configure the UE from transmitting n1 and n3 to n5 and n7, i.e., from UL CA_n1-n3 to UL CA_n5-n7, two different switching pattern could be configured:

Scenario 1

Network may configure UL switch as below,

bandIndexUL1-r18 = Band n1

bandIndexUL2-r18 = Band n5

bandIndexUL3-r18 = Band n3

bandIndexUL4-r18 = Band n7

gNB

UE

BandCombination-v1800

supportedBandQuadListNR-v1800

preferredBandPairsTxChain1

preferredBandPairsTxChain2

uplinkTxSwitchingPeriod-r18

…

UE report Tx switching capability information including band combo, supported UL band pairs, preferred switching band pair on each Tx chain …etc and switching periods

Network config UE for switching:

bandIndexUL1-r18=n1

bandIndexUL2-r18=n5

bandIndexUL3-r18=n3

bandIndexUL4-r18=n7

UE interpret the sequence implies:

UE interpret the switching configuration as:

Scenario 2

If network configures the UL switching as another alternative:

bandIndexUL1-r18 = Band n1

bandIndexUL2-r18 = Band n7

bandIndexUL3-r18 = Band n3

bandIndexUL4-r18 = Band n5

UE needs to interpret the switching configuration as:

Of course, network scheduler can take into account the preferred band pairs for Tx switching for each Tx chain reported by the UE that UE does not expect to be configured as scenario 2.

Observation 3: Associating the ordering of bands for defining switch-from and switch-to pairs in switching configuration commands may resolve the ambiguity issue without additional cost.

Proposal 2: RAN4 to agree on the band ordering based approach to resolve the ambiguity issue.

Proposal 3: Send an LS to RAN1/2 for the approach to resolve ambiguity issue for UE enhancement.

3. Conclusion

Observation 1: While keeping previous agreement, neither of the two Tx chains is expected to be used for transmission during the switching periods, network can only reserve a gap with max{Tswitch_A-C, Tswitch_B-D} or max{Tswitch_A-D, Tswitch_B-C} whichever is longer if ambiguity issue is not resolved.

Observation 2: There may be uplink transmission utilization improvement, ∆2-1, ∆3-1 or ∆3-2, if switching band pairs can be explicitly indicated without impact on RAN1 requirements.

Proposal 1:

Enhancements for Rel-18 Tx switching

o	Resolving the switching pattern ambiguity issue if it is determined that it is possible that 2 Tx chains are switched between two different band pairs for one Tx switching instance

      - By resolving the switching pattern ambiguity issue, the switching period can be the  switching capability of switched band pairs.

o	RAN4 agrees UE indication capability to resolve the ambiguity issue of the exact switching band pairs when concurrent switching occurs

Observation 3: Associating the ordering of bands for defining switch-from and switch-to pairs in switching configuration commands may resolve the ambiguity issue without additional cost.

Proposal 2: RAN4 to agree on the band ordering based approach to resolve the ambiguity issue.

Proposal 3: Send an LS to RAN1/2 for the approach to resolve ambiguity issue for UE enhancement.

Reference

- R4-2306660 WF on Tx switching ambiguity issue, RAN4#106-bis-e

- R4-2218928 Continue discussion on R18 Tx switching – Single TAG, RAN4#105

- R4-2301713 Discussion on multi-carrier enhancement for single-TAG Tx switching, RAN4#106

- R4-2303508 WF on ambiguity issue when two Tx chains are switched between different band pairs,