3GPP TSG-RAN WG4 Meeting # 107	R4-2308938

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.5.3.1.2

Source:	Ericsson

Title:	Simulation results for 8 Rx PDSCH requirements in FR1

Document for:	Information

# 1	Introduction

In this paper, we present the simulations results for 8Rx PDSCH in FR1. Based on last meeting discussions [1], we carried out simulations for PDSCH considering the agreed parameters.

# 2	PDSCH Simulation Results

## 2.1 Simulation setup

According to WF [1], we will use as general parameters:

| Assumptions | Value |
| --- | --- |
| Duplex mode | TDD |
| TDD UL-DL pattern | 7D1S2U with S=6D:4G:4U |
| Carrier Frequency [GHz] | 3.5 GHz |
| Subcarrier Spacing [KHz] | 30 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) and TDLC (300 ns delay spread) |
| Antenna configuration | 2x8 ULA Medium B, 4x8 and 8x8 ULA Low |
| Velocity | 3 km/h (Doppler spread = 10 Hz), 30 km/h (Doppler spread = 100 Hz) |
| PA Model | None |
| Pre-loaded Tx EVM | 6% (QPSK/16QAM/64QAM) and 3% (256QAM) |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4 and Rank 8 |
| DMRS Configuration | 2+2 Type 1 DMRS symbols at (2,3,10,11) symbol indexes for Rank 8 1+1 Type 1 DMRS symbols at (2, 10) symbol indexes for Rank 2 and Rank 4 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). From MCS Table 2 (TS38.214): Up to MCS 20 (256QAM) |
| NOH | 0 |
| Number of HARQ Processes | 8 |
| Maximum HARQ transmissions | 4 |

| Parameter |  | Value |
| --- | --- | --- |
| SCS (kHz)/Bandwidth (MHz) |  | 30/40 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for Rank ≤ 4 2 for Rank > 4 |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions, with different channel models, different MCSs but up to 8 MIMO layer (=< Rank 8). | 1-1, 1-2, 1-3, 2-1, 2-2, 2-3, 2-4 3-1, 3-2, 3-3, 3-4, 3-5 3-6, 3-7 |

## 2.2 Simulation results

In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in the Appendix.

Test cases for Rank 8:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 1.4 |
| 1-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 13.6 |
| 1-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| MCS were defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

Observation 1: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 8 DL MIMO Layers in FR1 using MCS17 under TDLA30-10 propagation environment. The SNR value is at 19.3 dB, thus 5 dB below the maximum SNR levels for conducted requirements.

Test cases for Rank = 4:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | -3.3 |
| 2-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 5.3 |
| 2-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 20 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 11.4 |
| 2-4 | 3.5 | 40 / 30 (106) | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 18.5 |
| MCS were defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

Observation 2: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 4 DL MIMO Layers in FR1 using MCS26 under TDLA30-10 propagation environment. The SNR value is at 18.5 dB, thus almost 6 dB below the maximum SNR levels for conducted requirements.

Test cases for Rank = 2:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 2 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | -4.5 |
| 3-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 7 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 4.4 |
| 3-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 13 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.4 |
| 3-4 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 13 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | 9.3 |
| 3-5 | 3.5 | 40 / 30 (106) | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 18.5 |
| 3-6 | 3.5 | 40 / 30 (106) | 256QAM, 0.67 MCS 20 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | 17.1 |
| 3-7 | 3.5 | 40 / 30 (106) | 256QAM, 0.89 MCS 26 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | 23.5 |
| MCS were defined based on TS 38.101-4 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

Observation 3: We can note that 8 Rx UE can support 2 DL MIMO Layers in FR1 using MCS20 (256QAM) under TDLC300-100 with ULA Medium B propagation environment. Furthermore, we can further boost the achievable MCS level to MCS26 (256QAM) for 8 Rx UE with Rank 2 under TDLA30-10 with ULA Medium B where an SNR margin of 6.5 dB below the maximum SNR level is enough to count for possible RF impairments (at 3% of Tx EVM).

# 3	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 8 DL MIMO Layers in FR1 using MCS17 under TDLA30-10 propagation environment. The SNR value is at 19.3 dB, thus 5 dB below the maximum SNR levels for conducted requirements.

Observation 2: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 4 DL MIMO Layers in FR1 using MCS26 under TDLA30-10 propagation environment. The SNR value is at 18.5 dB, thus almost 6 dB below the maximum SNR levels for conducted requirements.

Observation 3: We can note that 8 Rx UE can support 2 DL MIMO Layers in FR1 using MCS20 (256QAM) under TDLC300-100 with ULA Medium B propagation environment. Furthermore, we can further boost the achievable MCS level to MCS26 (256QAM) for 8 Rx UE with Rank 2 under TDLA30-10 with ULA Medium B where an SNR margin of 6.5 dB below the maximum SNR level is enough to count for possible RF impairments (at 3% of Tx EVM).

# References

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei.

# Appendix

## A.1 Table 5.1.3.1-1: MCS index table 1 for PDSCH (TS 38.214)

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

## A.2 Table 5.1.3.1-2: MCS index table 2 for PDSCH (TS 38.214)

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

## A.3 PDSCH Simulations

(a) Up to 64QAM, Rank 8, TDLA30-10 ULA Low      (b) Up to 64QAM, Rank 4, TDLA30-10 ULA Low

Fig. 1 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 4 and Rank 8 with up to 64QAM, using 40 MHz/30 kHz (106 RBs) and ULA Low under TDLA30-10 channels.

(a) Up to 16QAM, Rank 2, TDLC300-100               (b) Up to 256QAM, Rank 2, TDLA30-10

Fig. 2 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 2 with up to 256QAM, using 40 MHz/30 kHz (106 RBs) and ULA Medium B, under TDLC300-100 and TDLA30-10 channels.

Fig. 3 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 2 with 64QAM (MCS13) and 256QAM (MCS20), using 40 MHz/30 kHz (106 RBs) and ULA Medium B, under TDLC300-100 channels.