3GPP TSG-RAN WG4 Meeting # 107	R4-2308941

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.5.3.1.3

Source:	Ericsson

Title:	Simulation results for 8 Rx SDR in FR1

Document for:	Information

# 1	Introduction

In this paper, we present the simulations results for 8Rx PDSCH in FR1. Based on last meeting discussions [1], we carried out simulations for PDSCH considering the agreed parameters.

# 2	SDR Simulation Results

## 2.1 Simulation setup

Referring to WF [1], we summarize the agreed parameters in Tables 2-1 and 2-2.

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 for Rank <= 4 2 for Rank > 4 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2, 4 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

Table 2-2: Additional test parameters for TDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 8 |
| K1 value |  |  | Specific to FR1.30-1 |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| Note 1: PDSCH is scheduled only on full DL slots |  |  |  |

## 2.2 Simulation results

Updating Table 5.5A-5 [2] to support ‘Maximum number of PDSCH MIMO layers = 8’

- MCS index for maximum modulation format = 6 (Test case 1)

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- |
| 8 | 6 | 1 | 26 | 18.7 |
|  |  | 0.8 | 24 | 16.7 |
|  |  | 0.75 | 23 | 15.7 |
|  |  | 0.4 | 14 | 7.7 |
|  | 4 | 1 | 16 | 8.7 |
|  |  | 0.8 | 16 |  |
|  |  | 0.75 | 16 |  |
|  |  | 0.4 | 11 | 4.7 |
|  | 2 | 1 | 9 | 3.7 |
|  |  | 0.8 | 9 |  |
|  |  | 0.75 | 9 |  |
|  |  | 0.4 | 5 | -0.3 |

Observation 1: We can note that SDR results with Rank 8 can be verified for FR1 8Rx UE using 64QAM with MCS26.

- MCS index for maximum modulation format = 8 (Test case 2)

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- |
| 8RX | 8 | 8 | 1 | 24 | 23.7 |
|  |  |  | 0.8 | 22 | 20.7 |
|  |  |  | 0.75 | 21 | 19.7 |
|  |  |  | 0.4 | 12 | 11.7 |

Observation 2: We can note that SDR results with Rank 8 can be verified for FR1 8Rx UE using 256QAM with MCS24.

Updating Table 5.5A-6 [2] to support ‘Maximum number of PDSCH MIMO layers = 2’

- MCS index for maximum modulation format = 10 (Test case 3)

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- | --- | --- |
| 8RX | 4 | 10 | 1 | 24 | 27.7 |
|  |  |  | 0.8 | 21 | 22.4 |
|  |  |  | 0.75 | 19 | 20.7 |
|  |  |  | 0.4 | 9 | 10.1 |
|  | 2 | 10 | 1 | 24 | 23.8 |
|  |  |  | 0.8 | 21 | 18.7 |
|  |  |  | 0.75 | 19 | 16.8 |
|  |  |  | 0.4 | 9 | 6.7 |

Observation 3: We can note that SDR results with Rank 2 and Rank 4 can be verified for FR1 8Rx UE using 1024QAM with MCS24.

# 3	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: We can note that SDR results with Rank 8 can be verified for FR1 8Rx UE using 64QAM with MCS26.

Observation 2: We can note that SDR results with Rank 8 can be verified for FR1 8Rx UE using 256QAM with MCS24.

Observation 3: We can note that SDR results with Rank 2 and Rank 4 can be verified for FR1 8Rx UE using 1024QAM with MCS24.

# References

- R4-2305888, “WF for 8Rx UE demodulation requirements”, Huawei.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.8.0.

# Appendix

Table 5.1.3.1-1: MCS index table 1 for PDSCH (TS 38.214)

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Table 5.1.3.1-2: MCS index table 2 for PDSCH (TS 38.214)

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

## A.3 SDR simulation results

Fig. 1 - PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements when using up to 64QAM for FR1 8 Rx UE with Rank 8.

Fig. 2 - PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements when using 256QAM for FR1 8 Rx UE with Rank 8.

Fig. 3 - PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements when using 1024QAM for FR1 8 Rx UE with Rank 2.

Fig. 4 - PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements when using 1024QAM for FR1 8 Rx UE with Rank 4.