3GPP TSG-RAN WG4 Meeting # 107	R4-2308944

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.4.1

Source:	Ericsson

Title:	General aspects for FR2 multi-Rx DL chain.

Document for:	Discussion

# 1	Introduction

In this paper, we present our views on the remaining issues related to the general aspects for UE demodulation and CSI reporting requirements for FR2 multi-Rx chains. Meeting #106bis-e was the second meeting to discussion this WI where some agreements have been achieved as follow [1].

Issue 1-1-2: whether to assume correlation matrix approach for simulation and requirements derivation only. - Agreement: - Assume correlation matrix approach for simulation and requirements derivation only. Issue 1-1-3: Whether to place the test probes at offset associated with the assumed correlation for OTA setup. - Agreement: - Place the test probes at offset associated with the assumed correlation for OTA setup. Issue 1-1-4: Whether to Prioritize definition of new correlation matrix for demodulation requirement with multi-RX in FR2for OTA setup. - Agreement: - Prioritize definition of new correlation matrix for demodulation requirement with multi-RX in FR2for OTA setup Issue 1-1-5: Best beam pair selection for demodulation performance requirements. - Agreement: - Wait for agreement in RF and/or RRM session on metric for best beam pair selection and discuss if applicable and feasible for demod setup. - UE must maintain the beam from each panel fixed during each multi-Rx performance test Issue 1-1-6: Whether to study cross-talk between TRPs after Rx beamforming is achieved. - Agreement: - Consider cross-talk between TRPs captured via the correlation matrix as being discussed under Issue 1-1-1 Issue 1-1-7: Assumption on UE Antenna module. - Agreement: - Don’t explicitly include antenna module placement in demod discussion; can be considered through correlation modelling under Issue 1-1-1 (if needed). Issue 1-1-8: Whether to study applicability rule for UE panel deployment. - Agreement: - Don’t consider applicability rule for UE panel deployment. Issue 1-1-15: Whether to study isolation level between two parallel TRP-to-RX links for 2x2 independent UE processing. - Agreement: - No need to consider isolation level separately. The isolation level between two parallel TRP-to-Rx links are inherently considered as part of the correlation model discussed under Issue 1-1-1. Issue 1-1-16: PN model and PN compensation. - Agreement: - Leave PN impact and modelling to companies’ implementation, the impact can be considered into results submitted by companies and the impact on test parameters i.e., MCS selection subject to companies’ results. Issue 1-1-17: Assumption on FFT window. - Agreement: - Consider single FFT window as baseline assumption for FR2 multi-Rx demodulation requirements definition.

However, some issues, that we summarize below, still need to be discussed.

Issue 1-1-1: On new correlation matrix for FR2-1 in a Multi-TRP and Multi-Rx context for OTA demodulation performance requirements. - Proposals: - Option 1: - Use RH approach to generate OTA chamber internal H matrix for M-Rx 2 layer LLS simulations defined as: RH= 1ɳ1*ɳ1¦ɳ1¦2ɳ2*1ɳ1ɳ2*ɳ1ɳ2ɳ2*ɳ11ɳ1*¦ɳ2¦2ɳ2ɳ2*1 ⊙RH, where ⊙ is the elementwise product - Define requirements using the following configurations of AoA in the OTA chamber for 2-layer (1+1) case, with corresponding relative sidelobe antenna gains of: - AoA30: ɳ_1= ɳ_2= 0.1 - AoA90: ɳ_1= ɳ_2= 0.01 - AoA150: ɳ_1= ɳ_2= 0.03 - Option 2: - Reuse the current MIMO correlation matrix calculation methods defined in RAN4 spec to generate correlation matrix for each TRP-to-RX MIMO channel matrix Hi,j. Channel matrices from different TRPs to a Rx chain, i.e., Hi,1 vs Hi,2, are assumed to be statistically independent. - MIMO channel matrices from a TRP to two Rx chains, i.e., Hi,1 vs Hi,2, can be correlated, and the correlation coefficient αB between two Rx chains can be categorized as low (independent), medium and high which may depend on the AoA offset between two beam pairs. FFS: exact value of αB. Issue 1-1-9: Whether to consider MCS reduction for IFF method (if agreed for demodulation requirement). - Agreement: - Discuss later once achievable MCS is agreed. Issue 1-1-10: Delta AoA assumption for single DCI - Agreement: - Further evaluate the performance with the full set of AoA offset from RF session, down-selection on the values for performance requirements introduction can be carried based on evaluation results and analysis from companies. Issue 1-1-11: Delta AoA assumption for multi- DCI fully overlapping. - Agreement: - Further evaluate the performance with the full set of AoA offset from RF session, down-selection on the values for performance requirements introduction can be carried based on evaluation results and analysis from companies. Issue 1-1-12: Whether to introduce requirements for both single-DCI and multi-DCI scenarios. - Agreement: - Keep the current agreement of transmissions schemes and layer combinations for evaluation for companies to provide initial simulation results. Consider both sDCI with SDM and mDCI with fully overlapping resources, prioritize mDCI with fully overlapping. Issue 1-1-13: Performance requirement for multi- DCI non-overlapping. - Agreement: - Define at least one set of performance requirements for multi-DCI non-overlapping (if agreed to introduce requirements). Issue 1-1-14: Receiver assumption. - Proposals - MMSE-IRC receiver - Option 1: 2x2 Per TRP - Option 2: Jointly processing as 4x4 - Companies are encouraged to further evaluate the performance, UE processing complexity impact on above options.

In this draft, we provide our views on these pending issues

# 2	General aspects for UE demodulation and CSI reporting requirements

As it has been stated in [2], this WI aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier to achieve improved UE demodulation performance. To define demodulation performance requirements for enhanced FR2-1 UEs supporting up to 4 DL MIMO layers on single component (CC), we consider receptions configured with 2 active TCI states, requiring beam reception from 2 directions (2 layers/direction). Dual TCI operation can be combined with the Rel-17 mTRP framework even if the base station is being deployed as a single TRP [2]. Accordingly, we will draw our views based on the following general aspects:

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR2 DL multi-RX chain:

- TDD mode only

- Number of receive antennas per Rx chain NRx = 2

- Modulation order: Up to 64QAM

- Rank: Up to 4 DL MIMO layers (2+2).

Proposal 2: RAN4 defines the PDSCH demodulation and PMI reporting requirements with:

- FR2-1 TDD: SCS = 120 kHz, CBW = 100 MHz and 66 RBs.

- Phase noise @ 47 GHz

- Channel models: TDLA (30 ns delay spread) only

- Max Doppler frequencies: up to 300 Hz.

## 2.1 Channel correlation matrices

Consider a Multi-Rx chain UE receiving simultaneously from two TRxPs referred to as TRP1 and TRP2, respectively. Let Panel 1 and Panel 2 refer to the first and second Rx chain at the UE, respectively.

Define H11 and H21 the TRP1 – Panel1 and TRP1-Panel2 channel matrices, respectively, and H12 and H22 the TRP2 – Panel1 and TRP2-Panel2 channel matrices, respectively.

The NRx-dimensional received signal vector r1k, l of the k-th subcarrier and the l-th OFDM symbol is assumed to be expressed as a sum of the received signals H11k,ls1k,l (H21k,ls1k,l) and H12k,ls2k,l (H22k,ls2k,l) by Panel 1 (Panel 2) and the white noise n1(k,l) (n2(k,l)), given by

r1k, lr2k, l= H11k,lH21k,lH12k,lH22k,l s1k,ls2k,l +n1k,ln2k,l,

where,

sjk,l and Hijk,l, j=1, 2 represent the Nlayer,j x1 transmitted signal vector and the (NRx x Nlayer,j) channel matrix between the j-th transmit TRxP and the i-th UE Rx chain.

Observation 1: Considering the test will be in IFF conditions, we can assume far fields wave propagation conditions for AoA offset.

Fig. 1 – AoA offset considering far field assumption.

In Fig. 1, and considering far-field assumptions, we can note that both Rx chains see same AoA offset. Referring to WF [1]

Proposal 3: Considering same AoA offset at both Rx chains, assume TRxP1 – UE beam direction as a reference, while TRxP2- UE beam direction forms the AoA offset.

Proposal 4: Referring to Proposal 3, Let

- Let H1=H11H21 be the (2NRx x Nlayer,1) channel matrix between the 1st transmit TRxP, TRP1, and the UE,

- Let H2=H12H22 the (2NRx x Nlayer,2) channel matrix between the 2nd transmit TRxP, TRP2, and the UE,

- Define H11 and H21 MIMO correlation matrices as per clause B.2.3.1 or clause B.2.3.2 of TS 38.101-4,

- Define H12 and H22 MIMO correlation matrices as per clause B.2.3.1 or clause B.2.3.2 of TS 38.101-4,

- Consider the AoA offset in H12 and H22 only.

It is worth reminding that both TRxPs transmit independently to the UE.

Proposal 5: Consider that the propagation conditions applied to both TRxP1 and TRxP2 are statistically independent.

## 2.2 UE Receiver Processing

Receiving with a multi-Rx chain in DL needs to be carefully investigated in FR2. It is worth reminding that we have PN effects on the top of the RF impairments. Therefore, choosing between the two proposed options, referred to as

- Option 1: UE joint processing with 4x4,

- Option 2: UE processing with 2x2 per TRxP,

remains of great significance since each processing relies on its advantages.

However, RF session made some agreements in the meeting#106 WF stated as follow [3].

Agreement: - Define the RF requirement based on mDCI with understanding that UE supporting sDCI can also meet the requirement. - To have the unified requirement concept for UEs supporting multi-DCIs as for UEs supporting single DCI, RAN4 can focus on fully overlapping in time and in frequency, supported by UE capabilities “multiDCI-MultiTRP-r16” and “overlapPDSCHsFullyFreqTime-r16.” - No “joint detect/decode” is considered in simulation for mDCI. In other words, Process only 2 TRP-RX pairs– In this case we only consider two TRP to RX pairs - TRP1-RX1 and TRP2-RX2. The signal from TRP2 to RX1 and TRP1 to RX2 is treated as interference.

The system model is depicted by Fig. 2 where two TRxPs referred to as TRP1 and TRP2, respectively, communicate with a multi-Rx chain UE in downlink.

TRP1

TRP2

Fig. 1 – DL Multi-TRP Multi-Panel reception.

In the sequel, we provide the system equations to evaluate the performance of intra-cell inter-layer interference mitigation at UE, where the full overlapping sDCI- or mDCI-based schemes are considered.

The 2NRx-dimensional received signal vector rk, l=r1k, lr2k, l of the k-th subcarrier and the l-th OFDM symbol is assumed to be expressed as a sum of the UE received signals H1k,ls1k,l and H2k,ls2k,l by the two panels respectively, and the white noise n(k,l), given by

rk, l= H1k,ls1k,l+H2k,ls2k,l+n(k,l)

where,

sjk,l and Hjk,l, j=1, 2 represent the Nlayer,j x1 transmitted signal vector and the (2NRx x Nlayer,j) channel matrix between the j-th transmit TRxP and the UE (receiving with both panels).

Option 1: UE applying a joint processing, as 4T4R MIMO

In this case, the 2NRx-dimensional received signal vector rk, l can be rewritten as

rk, l= H1k,lH2k,l s1k,ls2k,l +n(k,l)

where both channel matrices H1 and H2 can be estimated through orthogonal DMRS. Indeed, for both cases sDCI and mDCI, UE is aware about the DMRS and antenna ports of TRP1 and TRP2 so that it can perform the channel estimates. It is worth reminding that we have NRx = 2 per Rx chain.

Consequently, eMMSE-IRC can be applied using the MMSE weight matrix R given by

R= P1H1k,lH1Hk,l+P2H2k,lH2Hk,l+1Nrek,l∈DMRS sent to UEr(k,l)r(k,l)H,

where

rk,l=rk, l-H1k,ld1k,l+ H2k,ld2k,l,

where Hjk,l and djk,l denote the estimated channel matrix and the DMRS symbols from the j-th transmit TRxP, while Nre denotes the number of DMRS resource elements. It is worth noting that Pj denotes the TRPj transmit power that is equal to Edj(k,l)2 where E. Denotes the expectation operator. The transmitted Nlayer,j x1 signal vector sj can be equalized as

sj(k,l) = H1k,lR-1rk, l

In sDCI based case, the equalizer output s1(k,l) and s2(k,l) will be vectorized to feed the LDPC decoder.

Option 2: UE applying a 2T2R MIMO processing per TRxP

In Option 2, each panel will treat a received signal from a TRxP while considering the received signal from the second TRxP as an interference. Without loss of generalities, consider the received signal at Rx chain 1,  Panel1, given by

r1k, l= H11k,lH12k,l s1k,ls2k,l +nk,l.

In this case, we apply MMSE-IRC receiver with a MMSE weight matrix given by

R= P1H11k,lH11Hk,l+1Nrek,l∈DMRS sent to UEr(k,l)r(k,l)H,

where

r1k,l=r1k, l-H11k,ld1k,l,

where H11k,l and d1k,l denote the estimated channel matrix and the DMRS symbols from TRP1.

Observation 2: It is obvious that each method has its advantages and disadvantages.

Proposal 6: Further discussion with companies on the pros and cons of each UE receiver schemes are needed:

- eMMSE-IRC for 4T4R UE joint processing.

- MMSE-IRC for 2T2R MIMO processing per TRxP.

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR2 DL multi-RX chain:

- TDD mode only

- Number of receive antennas per Rx chain NRx = 2

- Modulation order: Up to 64QAM

- Rank: Up to 4 DL MIMO layers (2+2).

Proposal 2: RAN4 defines the PDSCH demodulation and PMI reporting requirements with:

- FR2-1 TDD: SCS = 120 kHz, CBW = 100 MHz and 66 RBs.

- Phase noise @ 47 GHz

- Channel models: TDLA (30 ns delay spread) only

Max Doppler frequencies: up to 300 Hz.

Observation 1: Considering the test will be in IFF conditions, we can assume far fields wave propagation conditions for AoA offset.

Proposal 3: Considering same AoA offset at both Rx chains, assume TRxP1 – UE beam direction as a reference, while TRxP2- UE beam direction forms the AoA offset.

Proposal 4: Referring to Proposal 3, Let

- Let H1=H11H21 be the (2NRx x Nlayer,1) channel matrix between the 1st transmit TRxP, TRP1, and the UE,

- Let H2=H12H22 the (2NRx x Nlayer,2) channel matrix between the 2nd transmit TRxP, TRP2, and the UE,

- Define H11 and H21 MIMO correlation matrices as per clause B.2.3.1 or clause B.2.3.2 of TS 38.101-4,

- Define H12 and H22 MIMO correlation matrices as per clause B.2.3.1 or clause B.2.3.2 of TS 38.101-4,

- Consider the AoA offset in H12 and H22 only.

Proposal 5: Consider that the propagation conditions applied to both TRxP1 and TRxP2 are statistically independent.

Observation 2: It is obvious that each method has its advantages and disadvantages.

Proposal 6: Further discussion with companies on the pros and cons of each UE receiver schemes are needed:

- eMMSE-IRC for 4T4R UE joint processing.

- MMSE-IRC for 2T2R MIMO processing per TRxP.

# References

- R4-2306000, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm Inc.

- RP-223527, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Inc.

- R4-2303708, “WF on NR FR2 multi-Rx chain DL reception”, Samsung, Apple.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v17.8.0.