3GPP TSG-RAN WG4 Meeting # 107	R4-2308946

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.4.3

Source:	Ericsson

Title:	PMI reporting for FR2 multi-Rx DL reception

Document for:	Discussion

# 1	Introduction

Last meeting #106bis-e, it was the second meeting for this WI where discussions mainly focused on general aspects. In this meeting, we expect that we go through the PMI remaining issues stated in [1]. Below, we summarize the PMI reporting related open issues.

Issue 3-2-1: Whether to evaluate performance after defining correlation matrix to model spatial aspects. - Agreement: - Evaluate performance after defining correlation matrix to model the spatial aspects. ### Sub-topic 3-3: Simulation assumptions Issue 3-3-1: MIMO layer - Agreement: - Consider 1 MIMO layer per TRP. Issue 3-3-2: Number of CSI-RS ports - Proposals - Option 1: 8-port per TRP. - Option 2: Define CSI requirements with 4-port per TRP as a minimum. In addition to requirements with 4-port per TRP, requirements with 8-port per TRP can be included. - Option 3: 4-port per TRP. - Option 4: 2-port per TRP. Issue 3-3-3: MCS - Agreement: - Consider MCS13 for initial evaluation. Issue 3-3-4: Whether to assume LI in report quantity for multi-Rx FR2 - Proposals - Option 1: Include LI in the report quantity to facilitate PTRS port mapping. - Option 2: No Issue 3-3-5: Whether to assume time and frequency offsets. - Agreement: - Don’t consider time and frequency offsets for CSI reporting requirements. Issue 3-3-6: Simulation parameters for PMI reporting requirement for sDCI SDM scheme - Agreement: - Further discuss after relevant open issues are finalized including the TDD slot pattern.

# 2	PMI reporting requirements for FR2 Multi-Rx UE

Rel-17 has introduced per-TRP CSI reporting and RAN4 only defined s-DCI-based PMI reporting test using TypeI-SinglePanel codebook.

For sDCI, our view is to define single PMI reporting requirements using 4 ports TypeI-SinglePanel codebook and 1 MIMO layer per TRxP.

Proposal 1: Define single PMI reporting requirements for sDCI SDM based scheme using 4 ports TypeI-SinglePanel codebook and 1 MIMO layer per TRxP.

Hereafter, we provide our view on the general parameters that should be considered for PMI reporting requirements where the propagation environments, the antenna configuration, CQI/RI/PMI delay, number of retransmissions, etc. Furthermore, we consider the PDSCH Reference Channel with TDD UL-DL pattern FR2.120-2 as given by TS 38.101-4 Table A.3.2.2.5-8 [2].

Proposals 2 and 3 summarize these parameters.

Proposal 2: Consider PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-2 as given by TS 38.101-4 Table A.3.2.2.5-8, with MCS Table 64QAM and MCS index MCS13 (16QAM with CR = 0.48).

Proposal 3: Consider the following parameters to define PMI reporting requirements for sDCI SDM based scheme.

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 |  |
|  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  | TDD |  |
| Bandwidth |  | MHz | 100 |  |
| Subcarrier spacing |  | kHz | 120 |  |
| TDD DL-UL configurations |  |  | FR2.120-2 as specified in Annex A of TS 38.101-4 |  |
| Active DL BWP index |  |  | 1 |  |
| Propagation channel |  |  | TDLA30-75 |  |
| Antenna configuration per TRxP |  |  | High XP 4 x 4 (N1,N2) = (2,1) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 of TS 38.101-4 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 1.375 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.5-8.1 TDD |  |

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: Define single PMI reporting requirements for sDCI SDM based scheme using 4 ports TypeI-SinglePanel codebook and 1 MIMO layer per TRxP.

Proposal 2: Consider PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-2 as given by TS 38.101-4 Table A.3.2.2.5-8, with MCS Table 64QAM and MCS index MCS13 (16QAM with CR = 0.48).

Proposal 3: Consider the following parameters to define PMI reporting requirements for sDCI SDM based scheme.

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 |  |
|  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  | TDD |  |
| Bandwidth |  | MHz | 100 |  |
| Subcarrier spacing |  | kHz | 120 |  |
| TDD DL-UL configurations |  |  | FR2.120-2 as specified in Annex A of TS 38.101-4 |  |
| Active DL BWP index |  |  | 1 |  |
| Propagation channel |  |  | TDLA30-75 |  |
| Antenna configuration per TRxP |  |  | High XP 4 x 4 (N1,N2) = (2,1) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 of TS 38.101-4 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 1.375 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.5-8.1 TDD |  |

# References

- R4-2306000, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm Inc.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.8.0.