3GPP TSG-RAN WG4 Meeting #107                                         R4-2308947

Incheon, Korea, May 22 – 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On UE RF requirement for FR2 multi-Rx chain DL reception

Agenda Item:	8.8.2

Document for:	Approval

Introduction

In the last RAN4 meeting, fruitful discussion has been done for the Rel-18 WI for FR2 multi-Rx chain DL reception. The conclusion and remaining issues can be found in the approved WF [1].

- Data bias in agreed TE assumptions Agreement: - A TE system that uses a legacy 2-axis positioner as measurement setup for multi-Rx UE RF testing will introduce different measurement results for different UE orientations because test AoA pairs only lie along meridians of the UE centric grid. i.e., no test AoA pairs in other directions. - The work on the enhanced positioner, including motivation, conceptual solution, implementation aspects shall be recorded in the accompanying SI TR38.871 as record of having considered the technical aspects associated with the legacy positioner. - To balance pragmatic considerations against the goal of removing the different measurement results for different UE orientations, the enhanced positioner is not adopted in testing and the requirement is agreed to be framed based on the previously agreed legacy positioner. - The requirement applies for one of the FFS options below: Option 1: one or more UE-declared orientations in the positioner - FFS if UE orientation is packaged with an AoA separation angle as a pair. Option 2: one or more standard-specified UE orientations in the positioner. Option 3: the requirement applies for two fixed AoA configurations, the UE orientation can be declared (no capability needed) for each of these two directions. - Further alignment of simulation procedure Motivation: Simulation results from companies seem quite diverged. Companies are encouraged to propose further candidates for alignment, like: - procedures and assumptions on how SINR is calculated - how electromagnetic simulation data is used - simulation steps details and so on. Agreement: The following procedure can be used as a guideline for simulation: 1. For one UE implementation 2. For one UE orientation 3. Run EM simulation to obtain per-beam antenna gain patterns - Constant step size is suggested <= 5° - Performance difference between V/H element can be considered - Normalize antenna gain to align with the gain drop between peak EIS and spherical coverage in current spec (option 1 in 2.6) - Other calibration method that described in 2.6 also can be used. 4. For one angular separation 5. For one test grid point in 3D scan - Select beam based on RSRP (or SINR) 5.1 Calculate SINR of AoA+ and AoA- respectively - SINR = P_signal/(Noise + P_interf) Where the P_signal is the power of wanted signal and the P_interf is the power of interference, Noise(dBm)= -174 +10*log10(CBW) +NF, CBW is channel bandwidth, NF =10 - If SINR>=-1, PASS, otherwise, FAIL 5.3a OR combining the results of AoA+ and AoA- 5.3b No logic combination of the results of AoA+ and AoA-, but treat them as two separate points (e.g., arithmetic mean) - Other methods for +/- offset data are not precluded - Companies are encouraging to provide analysis on the pros and cons for each “combination” method 5.4 Add weighting (sin θ or Clenshaw-Curtis Quadrature) 6. Repeat for other test grid point 7. Calculate the spherical coverage percentage 8. Repeat for other angular separation 9. Repeat for other UE orientations 10. Repeat for other UE implementations

In this contribution, we would like to share more simulation results for this topic based on the above instructive simulation setup.

Discussion

Simulation results and analysis

For our simulation, we choose the functionality verification methodology i.e., for each TRP (corresponding to one of the two paired test grids), using the same DL power and check whether the SINR can be larger than -1dB, which is pretty aligned with the agreed simulation procedure. Before showing the simulation results, it is necessary to clarify other details.

- UE antenna module gain & location

Since it has been agreed that company is allowed to consider bias, antenna module location, antenna gain and other possible imperfections for UE RF architecture, we use EM simulation result for each 1x4 antenna module gain pattern instead of the 3GPP standards in TR 38.803. Specifically, those modules may not have identical beam gain pattern. Furthermore, the beam peak direction of one antenna module may not coincide with the normal direction of the physical plane of each module due to realistic consideration as to the surrounding material. Regarding the antenna module location, we have considered opposite faces (left and right) and adjacent faces (top of the back and left/right).

- UE orientation

We consider different UE orientations that the normal direction of the UE screen coincides with the positive direction of y-axis and z-axis.

- Rx beam selection

Based on the 7 beams that formed by a single antenna module, maximum RSRP is selected as the metric for Rx beam searching.

- Other

For our simulation, frequency point 28GHz is considered. Consequently, we choose -77.4dBm as the DL power for each TRP and the thermal noise can be calculated by the following formula.

Thermal noise = -174dBm + 10log10(CBW) + NF,

where 50MHz is selected as channel bandwidth and 10dB is picked for noise figure.

- Simulation results

- PC3 UE with two 1x4 dual polarization antenna modules on opposite faces

| AoA separation (degree) | Pass ratio |  |
| --- | --- | --- |
|  | UE facing positive direction of Y-axis | UE facing positive direction of Z-axis |
| 30 | 16.85% | 12.94% |
| 60 | 26.05% | 20.77% |
| 90 | 29.33% | 26.09% |
| 120 | 34.36% | 35.14% |
| 150 | 36.11% | 38.51% |
| 180 | 35.94% | 36.14% |

- PC3 UE with two 1x4 dual polarization antenna modules on adjacent faces (top of the back and left)

| AoA separation (degree) | Pass ratio |  |
| --- | --- | --- |
|  | UE facing positive direction of Y-axis | UE facing positive direction of Z-axis |
| 30 | 18.68% | 15.86% |
| 60 | 23.89% | 18.64% |
| 90 | 23.38% | 20.65% |
| 120 | 14.7% | 22.41% |
| 150 | 15.5% | 19.23% |
| 180 | 19.16% | 18.62% |

- PC3 UE with two 1x4 dual polarization antenna modules on adjacent faces (top of the back and right)

| AoA separation (degree) | Pass ratio |  |
| --- | --- | --- |
|  | UE facing positive direction of Y-axis | UE facing positive direction of Z-axis |
| 30 | 23.28% | 19.14% |
| 60 | 25.83% | 23.02% |
| 90 | 28.43% | 24.42% |
| 120 | 22.12% | 23.11% |
| 150 | 13.06% | 16.93% |
| 180 | 11.17% | 10.63% |

- Analysis

The above simulation results for the UE has dual antenna modules on opposite faces show that [120, 150, 180] degree is the predominance set for AoA separation, while pass ratio performance can be better for the UE has dual antenna modules on adjacent faces with [60, 90, 120] degree AoA separation. We think the reason is that under such functionality verification method, the AoA separation that a UE would have best performance is somehow determined by the beam peak direction separation of the two modules.

Observation: Under the functionality verification method, the AoA separation that a UE equipped with dual antenna modules would have best performance is somehow determined by the beam peak direction separation of the two modules.

Further, we can find that for different orientation, the results for the same set of conditions can be different. Consequently, we think it is better to let UE declare one AoA separation and one specified orientation as a package for the test of the unified requirement.

Proposal 1: For Multi-Rx, UE declares one AoA separation and one specified orientation as a package for the test of the unified requirement.

As for the absolute pass ratio, we think relaxation relative to the legacy requirement (e.g., 50% for PC3 UE) is needed because imperfection factors like materials surrounding the antenna modules that would impact the actual antenna gain shall be considered. Judging from the simulation results which were proposed in the previous meeting, we think 25% can be a starting point, so we have the following proposal.

Proposal 2: Relaxation should be considered for the RF requirement of multi-Rx. Specifically, 25% can be considered as starting point for the ratio of qualified points over whole sphere. Whether it is a qualified point shall be determined by the ‘OR’ operation for two related AoA pairs’ P/F results.

Conclusion

In this contribution we provide our views on FR2 UE multi-Rx from RF requirement perspective, we have the following observation and proposals:

Observation: Under the functionality verification method, the AoA separation that a UE equipped with dual antenna modules would have best performance is somehow determined by the beam peak direction separation of the two modules.

Proposal 1: For Multi-Rx, UE declares one AoA separation and one specified orientation as a package for the test of the unified requirement.

Proposal 2: Relaxation should be considered for the RF requirement of multi-Rx. Specifically, 25% can be considered as starting point for the ratio of qualified points over whole sphere. Whether it is a qualified point shall be determined by the ‘OR’ operation for two related AoA pairs’ P/F results.

- References

- R4-2306604, “WF on FR2-1 UE RF requirements for multi-Rx”, vivo, Apple, 3GPP TSG-RAN WG4 Meeting #106bis-e, Electronic Meeting, April 17 – 26, 2023.