3GPP TSG-RAN WG4 Meeting #107                                         R4-2308951

Incheon, Korea, May 22 – 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On FR1 8Rx UE RF requirements

Agenda Item:	8.5.1.2

Document for:	Approval

Introduction

In the RAN4#106bis-e, some progress has been achieved on NR 8Rx RF requirements. The WF [1] captures all agreements and especially controversial issues that require further discussion:

Issue: Whether to remove ΔPpowerclass for PCMAX_H,f,c <Way forward/Agreement>: Further discuss whether to remove the ΔPPowerClass=3dB applied for PCMAX_H,f,c for a PC2 capable UE with the support of TxD, if it further indicates the support of 1TxR AS-SRS. Reference for the future discussion: - Background - In current spec, ΔPPowerClass = 3dB applies to PCMAX_H,f,c for PC2 UE supporting TxD for 1TxR cases. - The reason of proposal: - If UE supports PC2 with 23+26dBm or 26+26dBm PA configuration, UE can transmit with full power PA for 1TxR. So, applying ΔPPowerClass to PCMAX_H,f,c is not necessary for such Ues. - The reason of objection: - If UE supports PC2 and indicates TxD capability, UE may do antenna virtualization and transmit 23+23dBm while it shouldn’t (no matter the PA configuration). To prevent this antenna virtualization and avoid 3dB uncertainty, applying ΔPPowerClass to PCMAX_H,f,c is necessary. Issue: whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage <Way forward/Agreement>: Open issue needs further discussion - Option 1: Do not remove - Option 2: Remove Issue: Which release 8Rx can be release independent from. <Way forward/Agreement>: FFS the following approach can be agreeable in next meeting. - 8Rx without SRS-AS can be release independent from Rel-15. - 8Rx with SRS-AS can be release independent from Rel-X. - Further discuss “Rel-X” by clarifying which SRS-AS patterns were introduced from which release. - Further discuss how to describe it in TS 38.307. Issue: Other RF Rx single carrier requirements (Proposal 1). <Way forward/Agreement> Further discuss the following options: For UE equipped with 8Rx, the UE RF Rx requirements other than REFSENS are - Option 1: Verified with 4Rx - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.” - Option 2: Verified with 8Rx - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.” - Option 3: - “For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four or eight Rx ports shall be verified in operating bands where the UE is equipped with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE), and additional requirements for eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four or eight Rx antenna ports (selected based on the maximum Rx antenna port supported by the UE) and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four or eight Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.” - Option 4: Others Issue: CA requirements. <Way forward/Agreement>: For 8Rx UE RF CA requirement, encourage interested companies to bring contributions in RAN4#107 with the following aspects: - Expected clarification/revision on the objective in WID if needed considering whether to consider band combinations in this WI. - For a band supporting 8Rx in single band operation, whether the band is mandated to support 8Rx in a band combination. - For whether example CA/DC band combinations (including EN-DC) is needed or not, check if proposal 2 is agreeable in next meeting: - Proposal 2: 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC). - For whether additional MSD is needed, check if proposal 3 is agreeable in next meeting: - Proposal 3: Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1. - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

In this contribution, we would like to share our views regarding some discussion points for 8Rx RF requirements.

Discussion

On the ΔPPowerClass for PCMAX_H, f, c

As captured in the WF [1], we understand the concern so far is coming from the potential ambiguity from the removal, e.g., whether it would force UE to enable TxD for antenna switching SRS transmission. We think a case by case exercise can help for the situation. Note that PA configuration is transparent to the specification, so basically there are three types of PA configurations to realize PC2 with TxD operation:

- 26 dBm + 26 dBm

- 26 dBm + 23 dBm

- 23 dBm + 23 dBm

For the first two types, when TxD is not allowed, at least one PC2 PA is still available so it is unnecessary to artificially set upper limit on the transmission power. Regarding the third type, we think such removal is not to force the UE for achieving SRS transmission power higher than 23 dBm by TxD from specification perspective, but some benefits to both UE and network in case of such UE can obtain higher transmission power by single Tx chain from implementation perspective can be introduced. In addition, no uncertainty will be caused between UE and gNB or for example PEMAX, c in the upper limit can work as a safe guard since it is configured by the network.

Observation: Regardless of any PA configurations (transparent to the specification) for the PC2 UE capable of TxD and AS-SRS, no uncertainty but benefit will be introduced for both UE and gNB if the 3dB ΔPPowerClass in the upper limit of UE transmission power can be removed.

Furthermore, we share the same understanding that this enhancement can be considered regardless the Rx number of UE. Thus we propose our draft CR in R4-2308952 once again.

Proposal 1: Draft CR in R4-2308952 for capturing the following enhancement:

- For a PC2 capable UE with the support of TxD or a PC1.5 capable UE, if it further indicates the support of 1T8R AS-SRS, the ΔPPowerClass=3dB applied for PCMAX_H,f,c should be removed regardless of PA configuration.

On whether to remove the guard period

We understand the intention behind this proposal, as for one extreme case the whole period for t1r8 AS-SRS may be worth noticing under some certain TDD patterns considering that the guard period will play the role as scheduling limitation. But such guard period was defined since early release of NR, normally any modification of legacy requirement should be carefully considered and this would be the reason why when such modification was brought up but the specification remains unchanged eventually in the late stage of Rel-17.

Proposal 2: Do not remove the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage.

Release independent for 8Rx

For this issue, we think the most essential point is to make sure there is no impact to the equipment that already exists in the market. Besides, 8Rx AS-SRS was introduced in Rel-17 by RAN1. In conclusion we think Rel-17 is the appropriate timing for release independent for 8Rx in order to provide better performance of this feature.

Proposal 3: For NR, 8Rx can be release independent from Rel-17.

Conclusion

In this contribution we discussed on the 8Rx RF requirements for FR1 UE, we have the following observation and proposals:

Observation: Regardless of any PA configurations (transparent to the specification) for the PC2 UE capable of TxD and AS-SRS, no uncertainty but benefit will be introduced for both UE and gNB if the 3dB ΔPPowerClass in the upper limit of UE transmission power can be removed.

Proposal 1: Draft CR in R4-2308952 for capturing the following enhancement:

- For a PC2 capable UE with the support of TxD or a PC1.5 capable UE, if it further indicates the support of 1T8R AS-SRS, the ΔPPowerClass=3dB applied for PCMAX_H,f,c should be removed regardless of PA configuration.

Proposal 2: Do not remove the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage.

Proposal 3: For NR, 8Rx can be release independent from Rel-17.

- References

- R4-2306597, “WF on FR1 8Rx UE RF requirements”, NTT DOCOMO, 3GPP TSG-RAN WG4 Meeting #106bis-e, Electronic Meeting, April 17 – 26, 2023.