3GPP TSG-RAN WG4 Meeting #107	R4-2308968
Incheon, KR, May 22 – May 26, 2023

Agenda item:	7.29.2

Source:	OPPO, CMCC, China Telecom, China Unicom, CAICT

Title:	R18 low band 4Rx feasibility

Document for:	Approval

- Introduction

Rel-18 3Tx and low band 4Rx WI was approved in RAN#98e with WID in [1] and the low band 4Rx was discussed in the following RAN4 meeting especially the feasibility to support low band 4Rx.

- Enhancements for 4Rx at low frequency band (<1GHz) - Study if feasible Specify requirements for handheld UE with 4Rx antennas including delta RIB,4R and ∆TRxSRS - 4Rx is optional feature for handheld UE in low frequency bands - Signalling impact will be considered if any. - Low frequency bands considered in this WI is in table 4.1-1 with the contact information inside.

In February RAN4#106, regarding the feasibility part, it was agreed in WF [3] that the supporting 4Rx in low bands maybe feasible at least for some handheld UE but also confirmed the complexity might be high for some other UE which is UE implementation dependent issue.

Issue 2-1-1: Feasibility of supporting 4Rx in low bands (<1GHz) Agreement: (agreed in Main session) - Supporting 4Rx in low bands may be feasible at least for some handheld UE and complexity might be high or gain might be low for some other UE which is UE implementation dependent. - Supporting 4Rx in low bands is an optional feature. - Interested companies are encouraged to provide more simulation or measurement data.

With above agreement, it gives UE vendors flexibility to declare whether their smartphones can support the low band 4Rx. However, in April RAN4#106bis-e, the feasibility issue was raised again, and some companies have concern on the complexity while other companies believe it is feasible since there are already UEs on the market with low band 4Rx capability. Therefore, RAN4 decided to close the feasibility study part in the May meeting [4].

Feasibility of low band 4Rx - Issue 1-1-1: Feasibility of other low bands (<1GHz) Agreement: - Once the feasibility of low band 4Rx for handheld UE is confirmed, the conclusion of feasibility can be applied to all the requested low bands and the feasibility does not preclude the introduction of the new low bands in the WID. - RAN4 plans to close the feasibility study in May meeting. - Issue 1-2-1: How to specify requirements for low band 4Rx (<1GHz) Agreement: - Requirements is needed for 4Rx handheld UE, if the feasibility is confirmed - FFS whether the new requirements will be specified or the existing requirements will be reused.

In this paper, the feasibility of low band 4Rx will be discussed and closed.

- Commercial smartphones with Low band 4Rx

Low band (n28) 4Rx was firstly commercialized by Realme smartphones in year 2022, and afterwards it was supported by other commercial smartphones as one of the important features to improve the throughputs. With less than a year, there already 5 smartphones supporting low band 4Rx on the market. It is obvious that all of them have passed the certifications no matter RF tests or OTA tests. Therefore, we see no reason to say low band 4Rx is infeasible since this is the fact.

In the past meetings, the comments from companies are mainly on the complexity, and no justification of infeasible. Although we understand the concerns of complexity in supporting low band 4Rx, in our view it is another issue instead of feasibility. The performance and complexity can be discussed further in requirement definition instead of using it as an argument to block confirm the feasibility and the facts.

Observation 1:   There are already 5 commercial smartphones on the market supporting low band 4Rx and all of them have passed the entrance certification tests including OTA tests.

Observation 2:   There is no justification of low band 4Rx is infeasible, instead the concerns are coming from the complexity which can be discussed in the requirement definition.

Proposal 1:         Confirm the feasibility of supporting low band 4Rx in handheld UE:

        - Supporting 4Rx in low bands is feasible.

        - Supporting 4Rx in low bands is an optional feature.

Proposal 2:         The complexity in supporting low band 4Rx will be considered in requirement definition.

- Performance of low band 4Rx

In RAN#98e, the low band 4Rx in handheld UE was proposed and approved. In the motivation paper [2], the antenna and field test results of some commercial smartphones with 4Rx have been shown which justified the feasibility of 4Rx to be supported in handheld UE. For better visibility, some of the contents in [2] are reproduced below.

- Antenna efficiencies The four antenna efficiencies in n28 are tested, as shown in table 1, where two of the antennas are as good as current 2Rx UE antenna efficiencies. And the other two antennas are 2dB worse, however, thanks to the smaller RFFE IL (closer to RFIC) the two additional antenna’s OTA performance are still similar as those existing antennas. The OTA test shows TIS improvement is about 2.5dB compared with 2Rx antenna UE. Table 1 antenna efficiencies of 4Rx antennas in n28 Ant 1; Ant 2; Ant 3; Ant 4. Efficiency (dB); -6.5; -6.7; -8.3; -8.5. - Antenna correlation/isolation Antenna correlation (ECC) and isolation are also tested with this smartphone. Test results show the ECC can be kept within normal antenna design targets, i.e. ≤0.5. And the antenna isolations can be achieved with 10dB. This is also within normal antenna design targets. - Field tests To further demonstrate the gains of this 4Rx antenna smartphone compared with current 2Rx smartphone in band n28, field tests are carried out. In this test, different locations (Center/Middle/Edge/Extreme) are chosen according to the DL RSRP levels. Then throughputs are tested with 4Rx smartphone and 2Rx smartphones. The results can be seen in table 2 and also shown in figure 1. Field test shows the throughput can be improved largely especially in the cell edge and extreme conditions. Note: Field tests were carried out in the mid-night to avoid other user’s impacts. Table 2 Throughput comparison of 4Rx UE and 2Rx UE in n28 Test Location; 4Rx throughput /Mbps; 2Rx throughput /Mbps; Improvement. Center (-65dBm); 259.2; 180.3; 43.7%. Middle (-85dBm); 143.2; 86.5; 65.4%. Edge (-106dBm); 101.6; 42.8; 137.4%. Extreme (-114dBm); 67.8; 14.9; 353.8%. Figure 1 Throughput comparison of 4Rx UE and 2Rx UE in n28 field test

From above test results of commercial smartphones with 4Rx on the market, the 4Rx can not only be supported by handheld UE in low bands but also can achieve good performance.

Observation 3:   Commercial smartphone tests show the performance of low band 4Rx can be achieved with good quality.

In [5], one simulation results from another company were provided as below in figure 2. We appreciate the inputs on the performance simulations of n28 4Rx and respect the views on the gain. However, this just reflects what the agreement made in previous meeting, i.e. “it is UE implementation dependent”. We see no reason to block other UEs to implement it with good performance just because of one UE cannot reach good performance.

Figure 2 Simulation results of antenna efficiency for n28 4Rx

Observation 4:   One simulation result shows the antenna efficiency is not good enough to support low band 4Rx, however, as it was already agreed this is just UE implementation dependent issue which cannot be used as the reason to block other UEs to achieve good performance.

- Requirement of low band 4Rx

There was comment in previous meetings that RAN4 only define low band 4Rx requirements for CPE/FWA and it will not prevent handheld UEs to support this feature.

In our understanding, this will put UE basic quality out of control which might degrade the whole system performance. And according to the agreement in last meeting [4] requirements is needed for 4Rx handheld UE, if the feasibility is confirmed, so this comment can be solved.

Issue 1-2-1: How to specify requirements for low band 4Rx (<1GHz) Agreement: - Requirements is needed for 4Rx handheld UE, if the feasibility is confirmed - FFS whether the new requirements will be specified or the existing requirements will be reused

Observation 5:   It was agreed that requirements are needed for 4Rx handheld UE, therefore, RAN4 cannot only define requirements for CPE/FWA and then allow handheld UEs to support it without quality control.

- Conclusions

In this contribution, the feasibility of low band 4Rx for handheld UE is analysed. Based on the above analysis, we got the following observations and proposals.

Observation 1:   There are already 5 commercial smartphones on the market supporting low band 4Rx and all of them have passed the entrance certification tests including OTA tests.

Observation 2:   There is no justification of low band 4Rx is infeasible, instead the concerns are coming from the complexity which can be discussed in the requirement definition.

Proposal 1:         Confirm the feasibility of supporting low band 4Rx in handheld UE:

        - Supporting 4Rx in low bands is feasible.

        - Supporting 4Rx in low bands is an optional feature.

Proposal 2:         The complexity in supporting low band 4Rx will be considered in requirement definition.

Observation 3:   Commercial smartphone tests show the performance of low band 4Rx can be achieved with good quality.

Observation 4:   One simulation result shows the antenna efficiency is not good enough to support low band 4Rx, however, as it was already agreed this is just UE implementation dependent issue which cannot be used as the reason to block other UEs to achieve good performance.

Observation 5:   It was agreed that requirements are needed for 4Rx handheld UE, therefore, RAN4 cannot only define requirements for CPE/FWA and then allow handheld UEs to support it without quality control.

- References

- RP-223517, Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC, OPPO, China Telecom, China Unicom, CMCC, NTT DoCoMo, Orange, Telecom Italia, T-Mobile USA, Verizon, Vodafone, RAN#98-e meeting, Dec 2022.

- RP-223026, Motivation of low band enhancement for handheld UE, OPPO, RAN#98-e meeting, Dec 2022.

- R4-2303522, WF on 4Rx requirements for low operating bands, OPPO, RAN4#106, Feb 2023

- R4-2306553, WF on low band 4Rx, OPPO, RAN4#106bis-e, April 2023

- R4-2305084, Further discussion on enhancements for 4Rx at low frequency band, vivo, RAN4#106bis-e, April 2023