3GPP TSG-RAN WG4 Meeting #107	R4-2308971
Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.24.2.1

Source:	OPPO

Title:	R18 Tx switching across 3 and 4 bands

Document for:	Approval

- Introduction

The Tx switching WF [1] for single TAG and WF [2] for clarification of switch period were approved in last meeting, and the open issues are as below. This paper will discuss further on these open issues.

Sub-topic 1-4: Length of switching time for certain scenarios Issue 1-4-1: Switching case across four bands, i.e., {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} Way forward after round 2 discussion: - Further discuss in the next meeting: - Option A: As optional UE behaviour, total switching period can be extended if UE is not capable for concurrent TX switching on the two TX chains. - Option A1: add new values {70, 175} usec in addition to the agreed set of {35 us, 140 us, 210 us} - Option A2: Sum of two switching periods - When UE is scheduled to switch two TX chains in such way that switching periods may overlap, the switching period is extended for both band pairs and total switching time is the sum of possible switching periods for the band pairs involved. - When UE is scheduled for transmissions so that the switching is from two bands with one TX each to another two bands one TX each, denoted for example A+B to C+D, and it cannot be determined if UE switches TX chains from A to C or D or from B to C or D, the switching time is sum of max{Tswitch_A-C, Tswitch_A-D,} and max{Tswitch_B-C, Tswitch_B-D}. - Include clearly the aspect that when two TX chains are switched with different lengths of the switching periods, none of the TX chains are expected to be used for transmissions. - Option B: Keep the previous agreements Issue 1-4-2: Switching case across three bands, i.e., {1T, 1T, 0T} to {0T, 0T, 2T} Way forward after round 2 discussion: - Further discuss in the next meeting: - Option A: As optional UE behaviour, total switching period can be extended if UE is not capable for concurrent TX switching on the two TX chains. - Option A1: add new values {70, 175} usec in addition to the agreed set of {35 us, 140 us, 210 us} - Option A2: Sum of two switching periods - When UE is scheduled to switch two TX chains in such way that switching periods may overlap, the switching period is extended for both band pairs and total switching time is the sum of possible switching periods for the band pairs involved. - Option B: Keep the previous agreements

Agreement - Encourage companies to study the benefit to resolve the switching pattern ambiguity issue when 2 Tx chains are switched between two different band pairs for one Tx switching instance - At least for single TAG FFS whether it applies to dual TAG

- Length of switching time

New length of switching times were proposed in last meeting considering the case of serial switching of two band pairs during a switching occasion. And the proposed new values include 70us and 175us which were the SUM of two original switching periods, i.e. 35+35 or 35+140 correspondingly. However, there also questions on why the new value only considers these two SUM values instead of like 140+140, 35+210, 140+210, 210+210, etc. if the serial switching is considered as so important. There is no answer to this question in last meeting.

Observation 1:   It is unclear why only 70us and 175us are needed but 140+140, 35+210, 140+210, 210+210 are not needed.

Another option proposed in last meeting was the SUM of the reported switching periods, like if UE report 35us for one band pair, and 140us for another band pair, then 175us will be used for the switching of these two band pairs. In this approach all the switching period combinations are included, however, it is not clear whether it is really necessary to introduce such large switching periods above 210us which will defeat the usefulness of this Tx switching feature.

Observation 2:   Large switch period will make the Tx switching feature no useful.

From the Tx switching capability report perspective, it is UE choice which value among 35us, 140us and 210us will be applied, which means if UE cannot do parallel switching of two band pairs then it is free to choose a larger value to cover the whole switching periods, and whether the switching is parallel or serial actually is unknown (not cared) by NW. From this perspective, there is no need to define new values nor to use the SUM of two switching periods, otherwise, new UE capability will be needed to differentiate which approach it applies. This is apparently unnecessary and not interested.

Observation 3:   It is UE choice which value among 35us, 140us and 210us will be reported, and if UE cannot do parallel switching of two band pairs then it is free to choose a larger value to cover the whole switching periods.

Proposal 1:         Reuse the existing switching periods {35us, 140us, 210us} for the Tx switching among 3 or 4 band cases.

- Switching pattern ambiguity issue

The switching period among 4 bands when switching from 1T+1T on band A+B to 1T+1T on band C+D was already agreed in meetings ago, and also send LS [3] to RAN1 and RAN2 as below. This is reconfirmed in WF [2] as baseline assumption.

Issue 2: Ambiguity issue when two Tx chains are switched between two different band pairs For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, and it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}, RAN4 agreed that: - As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D, A&D and B&C, respectively.

Observation 4:   The switching period among 4 bands was already agreed as max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. And further solve the switching patter ambiguity is not something necessary to enable this Tx switching feature.

Then in the past two meetings, there is proposal to resolve this ambiguity by implicit capability indication [4] and example solution as below table which means the switching band pairs on each Tx chain are determined by the NW scheduling. And concerns were raised on the impact to UE implementation because it is UE implementation dependent on which band switch to which band in each Tx chain. And this is no way to leave this determined by NW, if it is then how UE knows in implementation that NW will schedule??? This finally will mandate UE to implement with all Tx switching band pair band combinations which is not acceptable in UE design.

Example solution: The order of switching scheduling either via downlink control information (DCI) or RRC commands represents the mapping of Tx switching bands. An example is illustrated below, where, a scheduling of band A and band C, band B and band D is done in such manner that it implies switching from “A+B” to “C+D”, that in this way, switching pattern ambiguity is resolved without any additional signaling overhead.

Observation 5:   It is UE implementation dependent on the switched band pairs on each Tx chain. And if it is decided by NW scheduling then it will mandate UE to support all Tx switching band pair cases since UE cannot know how NW will schedule the switched band pairs beforehand. This is not acceptable in UE design.

Proposal 2:         The switched band pairs in each Tx chain is determined by UE implementation rather than decided by NW scheduling, otherwise it will mandate UE to implement with all band pair switch cases in each Tx chain.

Then if there is still interest to solve this no harm “ambiguity”, then it will lead to new UE capabilities to tell NW how the switching happens inside UE. And what will be the gain of this ambiguity is FFS according to the WF [2].

To understand better on how much benefit will derive if solve this no harm “ambiguity”, we need to recall how the values {35us, 140us, 210u} were defined. The switching period mainly depends on whether the PLL and PAs will be changed for the switching band pair. If no change of PLL then UE most likely can do it with 35us, and if PLL is changed then 140us will be needed, and further if whole Tx chain need reload then 210us. For the four bands switching case, it is obvious that the PLL will be changed in both Tx chains, and the exact switching period is as below:

- If no reload of the whole Tx chain then 140us will be applied for the two band pairs. It means, the largest value is same as the switching periods of each band pair. Then no benefit shows.

- And if both Tx chain will be reloaded, then both Tx chain will be 210us, in this case there is also no benefit of this explicit switching band pair indication.

- Only if one of the Tx chain is reloaded, then it will be 210us, and the other not reloaded Tx chain will be 140us. The benefit is 70us. However, in implementation, this is a concern case in our understanding.

From above, it can be seen that there is no clear benefit of solving this no harm “ambiguity”.

Observation 6:   For the switching from A+B to C+D, the PLL freq of each Tx chain will be changed which leads to the switching period in implementation at least 140us. And whether 210us will be applied depends on whether the whole Tx chain needs to be reloaded.

Observation 7:   There is no benefit when the two Tx chain switching periods are same. The benefit only shows when one of the Tx chain needs to be reloaded for all components, i.e. 210us applied for this chain and 140us apply for the other chain. And this is corner case.

Proposal 3:         There is no clear benefit to solve this no harm “ambiguity”.

For the solutions, if considered, should be UE implementation dependent, and this means new UE capability need to be defined. And below are some potential approaches.

- Approach 1: Introduce new Tx switching period capability for the switched case from A+B to C+D and other band group pairs before and after switching.

  - The new UE capability can be reported for all the possible band groups combinations before and after the Tx switching. For the band combination of A+B+C+D, it can report cases like below, and the switching period is still based on the {35, 140, 210}.

    - From A+B to C+D -> Band group Tx switching period

    - From A+C to B+D -> Band group Tx switching period

    - From A+D to B+C -> Band group Tx switching period

    - From C+D to A+B -> Band group Tx switching period

    - From B+D to A+C -> Band group Tx switching period

    - From B+C to A+D -> Band group Tx switching period

- Approach 2: Introduce new per Tx chain-based Tx switching capability

  - The new Tx switch period capability can be reported for each Tx chain with the detailed bands before and after Tx switching in each chain. In this approach the ambiguity will be solved totally. However, the reporting complexity will be high. To further solve it, UE indicate the per Tx chain switch period based on the RRC band combination configuration message can be applied.

Proposal 4:         Consider following two UE indication-based approaches if there is still interests to solve this no harm “ambiguity”.

        - Approach 1: Introduce new Tx switching period capability for the switched case from A+B to C+D and other band group pairs before and after switching.

        - Approach 2: Introduce new per Tx chain-based Tx switching capability

- Conclusions

In this contribution, the Tx switching is analysed. Based on the above analysis, we got the following observations and proposals.

Observation 1:   It is unclear why only 70us and 175us are needed but 140+140, 35+210, 140+210, 210+210 are not needed.

Observation 2:   Large switch period will make the Tx switching feature no useful.

Observation 3:   It is UE choice which value among 35us, 140us and 210us will be reported, and if UE cannot do parallel switching of two band pairs then it is free to choose a larger value to cover the whole switching periods.

Proposal 1:         Reuse the existing switching periods {35us, 140us, 210us} for the Tx switching among 3 or 4 band cases.

Observation 4:   The switching period among 4 bands was already agreed as max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. And further solve the switching patter ambiguity is not something necessary to enable this Tx switching feature.

Observation 5:   It is UE implementation dependent on the switched band pairs on each Tx chain. And if it is decided by NW scheduling then it will mandate UE to support all Tx switching band pair cases since UE cannot know how NW will schedule the switched band pairs beforehand. This is not acceptable in UE design.

Proposal 2:         The switched band pairs in each Tx chain is determined by UE implementation rather than decided by NW scheduling, otherwise it will mandate UE to implement with all band pair switch cases in each Tx chain.

Observation 6:   For the switching from A+B to C+D, the PLL freq of each Tx chain will be changed which leads to the switching period in implementation at least 140us. And whether 210us will be applied depends on whether the whole Tx chain needs to be reloaded.

Observation 7:   There is no benefit when the two Tx chain switching periods are same. The benefit only shows when one of the Tx chain needs to be reloaded for all components, i.e. 210us applied for this chain and 140us apply for the other chain. And this is corner case.

Proposal 3:         There is no clear benefit to solve this no harm “ambiguity”.

Proposal 4:         Consider following two UE indication-based approaches if there is still interests to solve this no harm “ambiguity”.

        - Approach 1: Introduce new Tx switching period capability for the switched case from A+B to C+D and other band group pairs before and after switching.

        - Approach 2: Introduce new per Tx chain-based Tx switching capability

- References

- R4-2306621, WF on Rel-18 Tx switching across 3/4 bands, China Telecom, RAN4#106bis-e, April 2023

- R4-2306660, WF on ambiguity issue when two Tx chains are switched between different band pairs, MTK, RAN4#106bis-e, April 2023

- R4-2220548, LS on Rel-18 UL Tx switching, RAN4->R1/R2, RAN4#105, Nov 2022