3GPP TSG-RAN WG4 Meeting #107	R4-2308978

Incheon, KR, May 22 – May 26, 2023

Title:	On AoA separation and UE orientation

Source:	OPPO

Agenda item:	8.8.2.2

Document for:	Approval

- Introduction

In the last RAN4 #106-bis-e meeting, the RF requirement for FR2 multi Rx DL reception was warmly discussed. There was no consensus on the AoA separation for specifying the requirement [1].

Besides, the contributions showed their analysis that different UE orientations would produce different measurement results. And how to specify the UE orientations is for further study with some options.

This contribution presents our views on AoA separations and UE orientations for specifying the RF requirement of FR2 multi Rx DL reception.

- Discussion

Regarding the cause of different UE orientations produce different measurement results, the main reason is that the AoA pairs only lie along the meridians of the UE centric grid. So when the UE orientation modified, the meridians on the UE centric grid changed correspondingly, which means totally different AoA pairs on the spherical surface are measured. Also, the measurement results is highly dependent on the UE’s FR2 panels implementations. Considering the UE usage scenarios, it is difficult to specify the UE orientation, especially for FR2 PC3 devices. With the above understanding, it is proposed to apply the UE-declaration manner to determine the UE orientations in the positioner.

Proposal 1: Apply the UE-declaration manner to determine the UE orientations in the positioner.

Regarding the AoA separation, one of the arguments is that the AoA separation should be defined by UE declaration or by specification. There are currently six AoA separations discussed on the table, i.e. 30⁰, 60⁰, 90⁰, 120⁰, 150⁰, 180⁰. Technically, it is difficult to say which one or two AoA separation(s) is the proper configuration for FR2 multi Rx DL reception evaluation, especially taking the various UE implementations into account. Therefore, the UE-declaration manner would be the most appropriate and efficient way to determine the AoA separation to be measured.

Proposal 2: Apply the UE-declaration manner to determine the AoA separation to be measured.

Considering the number of AoA separations required, from UE capability perspective, one AoA separation is enough to demonstrate that UE can support the feature of multi Rx DL reception with two AoAs. However, it would always be better to have more satisfied AoA separations for DL reception. But it is impossible to measure all the six discussed AoA separations. As a compromise, two AoA separations, one is selected from {30⁰, 60⁰, 90⁰} and another is selected from {120⁰, 150⁰, 180⁰}, declared by UE are workable specification requirement.

Proposal 3: Two AoA separations, one is selected from {30⁰, 60⁰, 90⁰} and another is selected from {120⁰, 150⁰, 180⁰}, declared by UE are workable specification requirement.

- Conclusions

This paper presents our views on AoA separations and UE orientations for specifying the RF requirement of FR2 multi Rx DL reception.. And provide the following proposal.

Proposal 1: Apply the UE-declaration manner to determine the UE orientations in the positioner.

Proposal 2: Apply the UE-declaration manner to determine the AoA separation to be measured.

Proposal 3: Two AoA separations, one is selected from {30⁰, 60⁰, 90⁰} and another is selected from {120⁰, 150⁰, 180⁰}, declared by UE are workable specification requirement.

- References

[1] R4-2306604, WF on FR2-1 UERF requirements for FR2 multi-Rx