3GPP TSG-RAN WG4 Meeting #107	R4-2308980

Incheon, KR, May 22 – May 26, 2023

Title:	On representing the test point on measurement grid

Source:	OPPO

Agenda item:	8.3.2

Document for:	Approval

- Introduction

The test methodology for multi Rx UE with two AoAs were discussed for several meetings. And the arrangement of the two AoAs reached some consensus, for example the measurement grids with constant step size [1]. And the relative positional relationship between the two AoAs were still under discussion, including the AoA angle separation and so on.

This contribution presents our considerations on the relative positional relationship of the two AoAs, and proposes a way of representing the test point on measurement grid to eliminate the potential ambiguity.

- Discussion

In the RAN4 #106 meeting, it is agreed to use constant step size to perform 3D scan on the spherical surface.

So the test point of AoA1 on the spherical surface can be expressed as (θ, ϕ). While the test point of AoA2 has a required angle separation between AoA1. However, the direction of AoA2, which is also called as “AoA1-AoA2 DL Orientation Vector” in the last meeting’s contribution [2], may be not clear without special instructions. The situation is illustrated below.

In the RAN4 #106 meeting, it is agreed that the AoA1 and AoA2 are all placed in the xz plane. Consequently, AoA1 and AoA2 only lie along meridians of the UE centric coordination system. Therefore, when AoA1’s coordinate is given as (θ, ϕ), the coordinate of AoA2 is confirmed as (θ+AoAseperation, ϕ) or (θ-AoAseperation, ϕ).

To simplify the representation of  the test points of AoA1 and AoA2, we propose the way to express the two test points with one coordinate, which can also fit with the situation that AoA2 does not lie in the same meridian of AoA1. The coordinate of AoA1 (θ, ϕ) can be extended to (θ, ϕ, ω), where ω demonstrates AoA1-AoA2 DL Orientation Vector. In detail, ω is the angle between the AoA1-AoA2 DL Orientation Vector and the reference axis in the assistant plane. The assistant plane is orthogonal to the AoA1 direction, as shown below.

In the assistant plane, set the cross point of Z axis and the assistant plane as Z’, assume the direction from AoA1 to Z’ as the reference axis in the assistant plane, as shown below. And ω is the angle from the reference axis to the AoA1-AoA2 DL Orientation Vector in clockwise direction.

Taking an example, following current agreement, the AoA2 lies with in the same meridian with AoA1. The coordinate (θ, ϕ, 0) indicates the AoA pair of AoA1 (θ, ϕ) and AoA2 ((θ-AoAseperation, ϕ), and the coordinate  (θ, ϕ, π) indicates the AoA pair of AoA1 (θ, ϕ) and AoA2 ((θ+AoAseperation, ϕ).

The proposed representing approach can also indicate the cases when the AoA pairs do not lie along the same meridian, and eliminate the possible ambiguity on test point expression.

Therefore, it is proposed to adopt the coordinate (θ, ϕ, ω) to represent the AoA pair on the measurement grid to eliminate the test point expression ambiguity, where ω demonstrates AoA1-AoA2 DL Orientation Vector.

Proposal: It is proposed to adopt the coordinate (θ, ϕ, ω) to represent the AoA pair on the measurement grid to eliminate the test point expression ambiguity, where ω demonstrates AoA1-AoA2 DL Orientation Vector.

- Conclusions

This paper presents our considerations on the relative positional relationship of the two AoAs. And provide the following proposal.

Proposal: It is proposed to adopt the coordinate (θ, ϕ, ω) to represent the AoA pair on the measurement grid to eliminate the test point expression ambiguity, where ω demonstrates AoA1-AoA2 DL Orientation Vector.

- References

[1] R4-2302923, WF for FR2 OTA test enhancement SI

[2] R4-2302520, Testing Considerations for Multi AoA Rx Testing