3GPP TSG-RAN WG4 Meeting #107	R4-2308994

Incheon, Korea, May 22 – May 26, 2023

Title:	[Draft] Reply LS on intraBandENDC-Support

Response to:	R2-2304431(R4-2306665)

Release:	Rel-16

Work Item:	TEI16

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: Clement Huang

E-mail Address: clementhuang@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 would like to thank RAN2 for the Reply LS on intraBandENDC-Support.

RAN4 has discussed the solutions for Case 3 and Case 4 and make the decisions as below.

- In Rel-16 and Rel-17, to extend the meaning of “both” in IE intraBandENDC-Support by including “contiguous(DL)/non-contiguous(UL)” for intra-band DL/UL EN-DC configuration

- In Rel-18, to introduce IE intraBandENDC-Support-UL.

The supported intra-band DL/UL EN-DC configuration for Case 3 and Case 4 can be referred to the following table.

| Scenario# | intraBandENDC-Support | intraBandENDC-Support-UL | Reporting EN-DC bandwidth class | Supported DL/UL EN-DC configuration |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | To extend the meaning of “both” in intraBandENDC-Support | To introduce IE intraBandENDC-Support-UL |
| 1 | Absent (Contiguous) | Absent | LTE-DL: 48C/48D | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A_(n)48AA with UL DC_(n)48AA | DC_48A_(n)48AA with UL DC_(n)48AA |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 2 | Absent (Contiguous) | Non-contiguous | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 3 | Non-contiguous | Absent | LTE-DL: 48C/48D | DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A | DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A-48A_n48A with UL DC_48A_n48A | DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 4 | Both | Absent | LTE-DL: 48C/48D | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] DC_48A-48A_n48A with UL DC_48A_n48A | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 5 | Both | Non-contiguous | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_48A_n48A DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 6 | Absent (Contiguous) | Both | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
| 7 | Both | Both | LTE-DL: 48C/48D | X | DC_(n)48CA with UL DC_(n)48AA DC_(n)48DA with UL DC_(n)48AA DC_(n)48CA with UL DC_48A_n48A DC_(n)48DA with UL DC_48A_n48A [Case#3] DC_48C_n48A with UL DC_48A_n48A DC_48D_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |
|  |  |  | LTE-DL: 48A-48A | X | DC_48A_(n)48AA with UL DC_(n)48AA DC_48A_(n)48AA with UL DC_48A_n48A [Case#4] DC_48A-48A_n48A with UL DC_48A_n48A |
|  |  |  | LTE-UL: 48A |  |  |
|  |  |  | NR-DL: n48A |  |  |
|  |  |  | NR-UL: n48A |  |  |

RAN4 respectfully asks RAN2 to take the above decisions into account to update RAN2 specifications.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above decisions into account to update RAN2 specifications.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #108                  21st August – 25th August 2023    Toulouse, FR

TSG-RAN WG4 Meeting #108bis               9st October – 13th October 2023    Xiamen, CN