3GPP TSG-RAN WG4 Meeting #107	R4-2308995

Incheon, Korea, May 22 – May 26, 2023

Agenda item:	7.29.2

Source:            	Google Inc.

Title:                	Discussion on enhancement for 4Rx at low frequency band

Document for:	Approval

# Introduction

In RAN4#106bis-e meeting, enhancements for 4Rx at low frequency band has been discussed and some agreements are reached to consensus. It is agreed that no spec update in RAN4 is needed for ΔTRxSRS and no new signalling is expected to differentiate the requirements for different UE types. However, there are still some undetermined issues such as feasibility of LB 4Rx and how to specify the ΔRIB,4R requirement if the feasibility of LB 4Rx is confirmed. In this paper, we would like to share our view in the following.

# Way Forward

## Feasibility of low band 4Rx

### Issue 1-1-1: Feasibility of other low bands (<1GHz)

Agreement:

- Once the feasibility of low band 4Rx for handheld UE is confirmed, the conclusion of feasibility can be applied to all the requested low bands and the feasibility does not preclude the introduction of the new low bands in the WID.

- RAN4 plans to close the feasibility study in May meeting.

## Sub-topic 1-2 Requirements for 4Rx at low bands

### Issue 1-2-1: How to specify requirements for low band 4Rx (<1GHz)

Agreement:

- Requirements is needed for 4Rx handheld UE, if the feasibility is confirmed

  - FFS whether the new requirements will be specified or the existing requirements will be reused.

### Issue 1-2-2: ΔRIB,4R

- Proposals

  - Option 1: -2.7dB

  - Option 2: -2.2 dB

  - Option 3: Middle value between -2.7dB and -2.2dB.

WF:

- Further discuss in next meeting.

# Way Forward

## Feasibility of low band 4Rx

### Issue 1-1-1: Feasibility of other low bands (<1GHz)

Agreement:

- Once the feasibility of low band 4Rx for handheld UE is confirmed, the conclusion of feasibility can be applied to all the requested low bands and the feasibility does not preclude the introduction of the new low bands in the WID.

- RAN4 plans to close the feasibility study in May meeting.

## Sub-topic 1-2 Requirements for 4Rx at low bands

### Issue 1-2-1: How to specify requirements for low band 4Rx (<1GHz)

Agreement:

- Requirements is needed for 4Rx handheld UE, if the feasibility is confirmed

  - FFS whether the new requirements will be specified or the existing requirements will be reused.

### Issue 1-2-2: ΔRIB,4R

- Proposals

  - Option 1: -2.7dB

  - Option 2: -2.2 dB

  - Option 3: Middle value between -2.7dB and -2.2dB.

WF:

- Further discuss in next meeting.

# Discussion

According to the WF in RAN4#106bis-e, it is pointed out that the feasibility of LB 4Rx for handheld UE is not determined yet. Some companies consider it would be good to have more performance evaluation for LB 4Rx feasibility study. In our view, the key challenge for the handheld UE to support LB 4Rx is antenna implementation. It would be possible that LB 4Rx is feasible for the handheld UE but the antenna performance may not be acceptable. However, since LB 4Rx has been agreed to be an optional feature in the previous meeting, it can be left to the UE vendor to decide whether to support 4Rx and inform the network by UE capability signaling.

The other remaining issue is how to specify the requirement for LB 4Rx. It is agreed in WF that requirement is needed for LB 4Rx handheld UE if the feasibility is confirmed. In other words, the RF requirement for ΔRIB,4R is needed to determine. For ΔRIB,4R, it is reference sensitivity adjustment due to support for 4 antenna ports, which means how much the gain can be improved for REFSENS. There are three options are proposed by companies in WF last meeting. Option 1 is -2.7dB which is proposed to use LTE LB B20 as a baseline to discuss LB 4Rx requirement. Option 2 is -2.2dB which is the worst value already defined in the current specification. Option 3 is to use middle value between -2.7dB and -2.2dB as a compromised solution.

From commercial handheld UE implementation perspective, it is common to support as many frequency bands equipped with 4Rx antenna as possible. MHB(mid/high band) and UHB (ultrahigh band) are easier to support 4Rx antenna with acceptable antenna performance than LB(low band) due to natural physical constraint. For LB 4Rx antenna design, it is challenging to have lower antenna correlation than MHB and UHB. Hence, the handheld UE is often equipped with 2Rx antenna for LB. In order to support LB 4Rx antenna, one possible solution is to add two more antennas in the handheld UE and the other one is to perform antenna sharing with the legacy band. In current commercial handheld UE design, it is too crowd to accommodate two additional new LB antennas, and antenna sharing between LB and legacy band may also cause additional Rx performance degradation to both bands. No matter which method is adopted, it would highly increase the implementation cost and implementation complexity. Hence, in our view, although the requirement for ΔRIB,4R is based on the conducted assumption, considering very high implementation complexity for handheld UE equipped with LB 4Rx, we would like to have more relaxation to be included in ΔRIB,4R. So we propose to adopt the value ΔRIB,4R=-2.2 for LB 4Rx, which is also the lowest value already defined in the current specification.

Proposal 1: If the feasibility of LB 4Rx for handheld UE is confirmed by RAN4, considering very high implementation complexity for the handheld UE equipped with LB 4Rx, it is proposed to have more relaxation to define ΔRIB,4R = -2.2.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: If the feasibility of LB 4Rx for handheld UE is confirmed by RAN4, considering very high implementation complexity for the handheld UE equipped with LB 4Rx, it is proposed to have more relaxation to define ΔRIB,4R = -2.2.

# Reference

- R4-2306553, “WF on 4Rx requirements for low operating bands”, RAN4#106bis-e, Vivo

- R4-2305290, “Discussion on enhancement for 4Rx at low frequency band”, RAN4#106bis-e, Google