3GPP TSG-RAN WG4 Meeting # 107										R4-2309002

Incheon, KR, May 22 – May 26, 2023

Title: 	Discussion on general aspects for AIML

Source: 	Xiaomi

Agenda item:	8.22.3

Document for:	Approval

- Introduction

An SI on AI/ML for NR air interface was approved in RAN#94e [1]. RAN4 is tasked to testing aspects based on sufficient progress in RAN1 and RAN2. In RAN4#106bis-e meeting, some progress on general aspects of AI/ML has been achieved and one WF [1] was approved. Some issues captured in the WF still need more discussion, such as RAN4 work plan, LCM related test, interoperability, reference model, test dataset. This contribution will provide our view on such general issues.

- Discussion

2.1 Work plan

- 2.1.1 Work plan Agreement: - High level work plan is agreed as follows: - During Q2 (RAN4 #106bis, RAN4#107), general issues of AI/ML are firstly discussed in a single thread. The main objective of this stage is to achieve comprehensive understanding and align companies’ views on these issues for next step. Moreover, use case(s) specific issues should also be studied. - During Q3 (RAN4 #108), besides carrying on discussing unresolved general issues, use case specific discussion could be continued (in separate sessions depending on progress). At the end of Q2 (either RAN4 #107 or RANP #100) it could be further discussed how to proceed with the use case specific discussion in Q3 (how many threads/tracks will be used and in which RAN4 session). - During Q4 (RAN4 #108bis, RAN4 #109), besides carrying on discussing unresolved general issues if necessary, continue use case specific discussion and try to draw a set of solutions for each use case, including recommendations towards normative work. Meanwhile, the work on drafting TR can start as soon as TR-impact has been identified. - Discussion planning - Further discuss how to organize Q3 discussion in RAN4#107 or RAN#100 depending on progress - Use case specific could be handled in RRM and demod sessions, general issues can be handled in a general thread if not concluded - A recommendation should be made in RAN4#107 whether to go to discussions in separate sessions - The criteria to go to separate sessions is that, based on the outcome of RAN4#106bis and RAN4#107, distinct, non-overlapping use case specific issues are identified for discussion that can be handled efficiently in separate sessions and will not clash - Default assumption is that Beam management and positioning would be handled in the RRM session, CSI reporting in the demod session - Ultimate decision to be made by RAN4 leadership

Based on the discussion on RAN4 work plan, we almost reach the conclusion on how to organize the discussion among different RAN4 sessions. Default assumption is that beam management and positioning specific issue would be handled in the RRM session and CSI reporting in the demodulation session. Regarding the general issue that is common for each use case, a separate thread should be used. Firstly RAN4 should clearly identify which issue can be considered as general issues. In our understanding, testing framework, generalization performance, interoperability, reference model/data set, etc. can be treated as general issue. If all LCM related requirements falls into RRM domain, LCM can be handled in RRM session. Otherwise, LCM related requirements can considered as general issue. Then general issue can be handled in RF/main session in addition to RRM session and demodulation session in order to achieve a good balance between RRM expert and demodulation experts.

Observation 1: General issue can be handled in RF/main session while beam management and positioning specific issue can be handled in RRM session and CSI reporting specific issue in demodulation session.

2.2 LCM related

- 2.2.4 LCM Related requirements Agreements: - Following LCM related requirements are to be studied: - Model/Functionality select/switch/activate/deactivate/fallback - Model/Functionality monitoring - FFS if requirements for data collection (in particular for training) could/need be defined - FFS if requirements for transfer/delivery/update - NOTE: RAN4 study should be aligned with the agreements in other working groups. - Further study under LCM related tests, if they are defined. - how the framework can address the possibility of updates/activation/deactivation/switching to the functionalities/models after the deployment of the devices in the filed

Currently RAN1 is discussing the functionality-based LCM or model-ID-based LCM to be used for different use cases. The following agreements is achieved:

RAN1 #111 Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Funtionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model

Based on the current RAN1 agreement, for functionality-based and model-ID-based LCM procedure, the indication of activation/deactivation/switching/fallback has different granularity, i.e., based on individual AI/ML functionality or individual model IDs. RAN1 will continue discussion on functionality-based and model-ID-based LCM procedure as well potential RAN impact. RAN4 could waiting for RAN1 further agreements.

Currently RAN4 can discuss the relationship or difference between functionality-based and model-ID-based LCM procedure from test perspective and distinguish them for requirement definition if necessary.

Proposal 1: RAN4 can discuss the relationship or difference between functionality-based and model-ID-based LCM procedure from test perspective and distinguish them for requirement definition if necessary.

For functionality/model switch/activate/deactivate, the processing time to complete such procedure can be defined. This is similar as the delay requirement defined in RRM. The similar test methodology defined for cell switch/activate/deactivate can be reused as a starting point.

For functionality/model monitoring, RAN1 is being discussing the various approaches for achieving good performance. Relevant KPIs may vary across different model monitoring approaches. The following model monitoring KPIs are considered in RAN1:

- Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

- Overhead (e.g., signaling overhead associated with model monitoring)

- Complexity (e.g., computation and memory cost for model monitoring)

- Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

- FFS: Power consumption

- Other KPIs are not precluded.

RAN4 can wait for the further agreements on monitoring approach from RAN1 and then make decision. RAN4 does not need to test all KPI but introduce one or several of them, e.g., latency if necessary from RAN4 test perspective.

For functionality/model transfer/delivery, there are two solutions, i.e., one is to transfer/delivery structure and parameter and another is to transfer/delivery parameter only. The former may need UE compiling and the latter may not need UE compiling. RAN4 can have separate discussion for these two solutions if necessary.

For functionality/model identification, it is similar as UE capability reporting. There is no need to define test for functionality/model identification.

Proposal 2: For functionality/model switch/activate/deactivate, the processing time to complete such procedure can be defined. The similar test methodology defined for cell switch/activate/deactivate can be reused as a starting point.

Proposal 3: For functionality/model monitoring, RAN4 can wait for the further agreements on monitoring approach from RAN1 and then make decision. RAN4 does not need to test all KPI but introduce one or several of them, e.g., latency if necessary from RAN4 test perspective.

Proposal 4: For functionality/model transfer/delivery, RAN4 can have separate discussion for such two solutions if necessary, i.e., one is to transfer/delivery structure and parameter and another is to transfer/delivery parameter only.

Proposal 5: No need to define test for model identification.

2.3 Generalization performance

- 2.2.5 Generalization/scalability of requirements/tests - Further study whether it is needed/feasible to introduce some form of generalization and/or scalability related requirements for different scenarios/configurations based on RAN1 agreements - Whether this can be implicitly handled in the test case definition should be considered - Intention is to guarantee that performance will still be maintained in different environments/scenarios/configurations.

For AI-based use case, the model are trained by some certain data that is generate with certain scenario. Nevertheless, the channel conditions in real practice are different from the scenario that the training data comes from. Thus, how to test the generalization performance is important to verity the benefit of AI-based use case compared to non-AI use case. RAN4 does not have a legacy method to verify generalization performance so maybe we can first go through RAN1 methodology used in simulation.

Taking CSI compression as an example, the following cases are considered to verify the generalization performance of an AI/ML model over various scenarios/configurations in RAN1 simulation:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - FFS other cases for generalization verification, e.g., - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Case 1 is the baseline case where training dataset and test dataset are from the same scenario/configuration combination. For Case 2/2A and Case 3, the training dataset and test dataset are from different scenario/combinations. To evaluate the generalization performance, the performance of case 2/2A and case 3 is compared with case 1 by using intermediate KPI, e.g., SGCS, NMSE. Positioning and beam management have the similar methodology as CSI compression to evaluate the generalization performance. It is proposed to use the methodology in RAN1 simulation as a starting point to verify generalization performance. RAN4 need to discuss how to introduce RAN1 methodology into RAN4 test.

From the test perspective, there is trade-off between supported scenarios and generalization performance verification. RAN4 is not able to test all identified scenarios and instead should try to cover more typical scenarios as much as possible.

Proposal 6: The methodology used in RAN1 simulation can be used as a starting point to verify generalization performance. RAN4 need to discuss how to introduce RAN1 methodology into RAN4 test.

2.4 Interoperability

- 2-sided framework - RAN4 to study the following issues for the 2-sided model test framework - Common assumptions for proposals of the reference decoder / encoder (and the paired encoder/ decoder) for tester - Definition and derivation procedure of intermediate KPI for decoder evaluation and selection - Data collection/generation for decoder evaluation, and the common assumptions/environment needed for data collection/generation - How to minimize the impact of possible variations/differences in the reference decoder/ reference encoder design/implementation on UE/ gNB performance verification - The impact of reference decoder/ encoder for testing complexity to UE/gNB performance verification, and the advantage/disadvantage analysis of high/low complexity decoders. - Other aspects are not precluded, companies are invited to bring contribution detailing any other aspects that should be considered - FFS whether any reference for the encoder/ decoder needs to be considered given that the encoder/decoder performance is to be tested - Take into account RAN1 discussions and conclusions on interoperability and training for 2-sided - Reference Decoder for test implementation for 2-sided models in the UE performance tests - Following options should be studied for the reference decoder for test implementation in the UE performance tests - Option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained - Option 2: reference decoder is provided by the vendor of the decoder so that the encoder and decoder are jointly designed and trained - Option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The reference decoder(s) are partially specified and captured in RAN4 spec. - Option 5: Option 1, 2, 3 or 4 depending on the test - Option 6: Test decoder is specified and captured in RAN4 and is provided by test environment vendor. The encoder and decoder can be jointly trained. - Other options can be discussed depending on companies’ inputs - Reference decoder defined for the tester in the UE performance tests should not limit the implementation of different models at the network side - Reference Encoder for test implementation for 2-sided models in the gNB performance tests - Following options should be studied for the reference encoder for test implementation in the gNB performance tests - Option 1: reference encoder is provided by the vendor of the decoder under test so that the encoder and decoder are jointly designed and trained - Option 2: reference encoder is provided by the vendor of the encoder so that the encoder and decoder are jointly designed and trained - Option 3: The reference encoders are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The reference encoders are partially specified and captured in RAN4 spec. - Option 5: Option 1, 2, 3 or 4 depending on the test - Option 6: Test encoder is specified and captured in RAN4 and is provided by test environment vendor. The encoder and decoder can be jointly trained. - Other options can be discussed depending on companies’ inputs - Reference decoder defined for the tester in the gNB performance tests should not limit the implementation of different models at the UE side.Further discuss the difference between reference encoder/decoder and test encoder/decoder.

Based on the discussion in the last meeting, seems companies have different understanding on the intention of introducing reference decoder/encoder. So RAN4 first needs to align on the intention of introducing introduce reference encoder/decoder. Different models with different training types are defined in RAN1, e.g., fully connected, CNN and/or transformer. Based on the simulation results, it can be observed that in some cases different models would have very different performance. So some common assumption for the reference decoder/encoder for TE can be established and is helpful for simulation alignment. It would be difficult for RAN4 to define requirements without a reference decoder/encoder.

Regarding the reference decoder for test implementation for 2-sided model in the UE performance tests, we prefer option 3, i.e., the reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. Compared with option 1 and option 2, option 3 can also guarantee that encoder in DUT and decoder in TE are paired and simplify the test procedure. Option 1 and option 2 can be considered as solutions to option 3.

Proposal 7: RAN4 should align on the intention to define reference decoder/encoder for 2-sided model. Option 3 is preferred.

2.5 Test dataset

- 2.3.5 Test dataset generation Test dataset generation should be studied. Different generating methods can be used for different tests. The following candidate methods are to be considered or down-selected: - Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc. - “Legacy approach” refers legacy test in which a channel model is used - Field dataset (data collected directly from field measurements) - TE generates dataset for test based on assumptions/parameters defined by RAN4 (e.g. by defining some rules/function to generate data) - Other methods are not precluded

UE inference performance is determined by dataset and model structure. So how to collect test dataset is important to verify the performance of model inference. Currently the option 1, i.e., dataset based on TR 38.901 is used in RAN1 evaluation. This can be considered as a starting point.

The option 2 is to define test dataset that is collected from real field. It is a more physical solution that can accommodate different scenarios. But the feasibility should be evaluated.

The option 3 is to define common dataset based on some certain assumption/parameter defined by RAN4. This can largely reduce the test efforts and reduce the test time for test dataset generation. Option 3 is preferred.

Proposal 8: Regarding test data generation, option 1 can be considered as a starting point and option 3 is preferred.

- Conclusion

This contribution gives our consideration on some general aspects. The following proposals and observations are derived:

Observation 1: General issue can be handled in RF/main session while beam management and positioning specific issue can be handled in RRM session and CSI reporting specific issue in demodulation session.

Proposal 1: RAN4 can discuss the relationship or difference between functionality-based and model-ID-based LCM procedure from test perspective and distinguish them for requirement definition if necessary.

Proposal 2: For functionality/model switch/activate/deactivate, the processing time to complete such procedure can be defined. The similar test methodology defined for cell switch/activate/deactivate can be reused as a starting point.

Proposal 3: For functionality/model monitoring, RAN4 can wait for the further agreements on monitoring approach from RAN1 and then make decision. RAN4 does not need to test all KPI but introduce one or several of them, e.g., latency if necessary from RAN4 test perspective.

Proposal 4: For functionality/model transfer/delivery, RAN4 can have separate discussion for such two solutions if necessary, i.e., one is to transfer/delivery structure and parameter and another is to transfer/delivery parameter only.

Proposal 5: No need to define test for model identification.

Proposal 6: The methodology used in RAN1 simulation can be used as a starting point to verify generalization performance. RAN4 need to discuss how to introduce RAN1 methodology into RAN4 test.

Proposal 7: RAN4 should align on the intention to define reference decoder/encoder for 2-sided model. Option 3 is preferred.

Proposal 8: Regarding test data generation, option 1 can be considered as a starting point and option 3 is preferred.

- Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN#94e

- R4-2306299, WF on AI/ML RAN4 studies, Qualcomm, RAN4#106bie-6