3GPP TSG-RAN WG4 Meeting #107				                                R4-2309014

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.17.3

Source:	Xiaomi

Title:			On the FR1 MIMO OTA

Document for:	Discussion

- Introduction

The necessity and feasibility of FR1 MIMO OTA test with hand phantoms has been discussed for several meeting but no consensus. The latest progress could be found in the WF [1] from the last RAN4 meeting. In this paper, we give further discussion on this topic based on the WF.

- Discussion

The agreement of FR1 MIMO OTA testing with Hand Phantoms are captured in the WF [1] from the last RAN4 meeting as:

### Sub-topic 1-3 FR1 MIMO OTA test with Hand phantoms Issue 1-3-1: Feasibility of FR1 MIMO OTA test with hand phantom Proposals: - Proposal 1: Adjust positioning guidelines to ensure that the DUT and hand phantom can be encompassed in the 20 cm QZ in DMP. - Proposal 2: Further discuss the MU assessment and CM impact with hand phantom based on the conclusion of the QZ size issue. <Agreement>: - FFS the feasibility and the proposals. Issue 1-3-2: Necessity of FR1 MIMO OTA test with hand phantom <Agreement>: - FFS the necessity of FR1 MIMO OTA test with hand phantom at next meetings. - Focus more on the necessity than the feasibility of FR1 MIMO OTA test with hand phantom at the next meeting.

As already provided in our paper [2], the initial measurement results shows that the largest and smallest delta of free space to hand phantom is 3.4 dB and 0.13 dB respectively. It can be observed the MIMO OTA performance in hand phantom may be very different from that in free spec (which has also been verified in the paper [3]), i.e. the MIMO OTA performance in FS couldn’t accurately reflect the MIMO OTA performance with hand phantoms. From this point of view, it is necessary to investigate the hand-phantom test for MIMO OTA. However, we also agreed that the necessity also needs to consider the actual demand from operators.

Observation 1: the MIMO OTA performance in FS couldn’t accurately reflect the MIMO OTA performance with hand phantoms.

Observation 2: it is necessary to investigate the hand-phantom test for MIMO OTA for handheld UE.

Observation 3: the necessity also needs to consider the actual demand from operators.

Regarding the feasibility issue, the main concern is that the current 20cm quiet zone may not cover the hand phantom size. Based on the current discussion, it seems there are two possible directions. The first one is to reuse the current 20cm quiet zone but adjust positioning guidelines to ensure that the DUT and hand phantom can be encompassed. From our investigation, it is at least possible for DMP model. For other modes, there may not be possible if the DUT side is too big. The second one is to ensure the DUT is encompassed in the current 20cm QZ but allow the phantom partial out of the QZ range. From our perspective, option 1 is our preference if it is possible. But we also open to discuss the feasibility of the second one to have more analysis on its impact MU assessment and CM impact. Regarding the other direction like increasing QZ, we don’t think it is possible at least in this release as the impact would be very big compared with other directions and it may also impact current MIMO OTA requirement in FS.

Observation 4: changing current QZ range should not be considered at least in R18.

3	Conclusions

In this paper, we give further discussion on the FR1 MIMO OTA, the observations are captured as below:

Observation 1: the MIMO OTA performance in FS couldn’t accurately reflect the MIMO OTA performance with hand phantoms.

Observation 2: it is necessary to investigate the hand-phantom test for MIMO OTA for handheld UE.

Observation 3: the necessity also needs to consider the actual demand from operators.

Observation 4: changing current QZ range should not be considered at least in R18.

4 References

[1]R4-2302975, WF for Rel-18 MIMO OTA, CAICT

[2]R4-2301920, On the FR1 MIMO OTA, Xiaomi

[3]R4-2305122, Further views on FR1 MIMO OTA test for smartphone with hand phantom, CAICT