# 3GPP TSG-RAN WG4 Meeting # 107			R4-2309026

# Incheon, South Korea, May 22 – 26, 2023

Source:	Sony

Title:	Views on 4Rx handheld UE for low NR bands

Agenda item:	7.29.2

Document for:	Approval

- Background

Support of 4Rx and 3Tx for low operating bands was discussed in RAN4 #106 bis-e and a WF was agreed [1].

This Tdoc will discuss 4Rx requirement for bands n28, n20, n8, n5, n26 and n71 [2].

- Discussion

The feasibility was further discussed in RAN4 #106 bis-e and even though not finally agreed most companies agree 4RX for the specified bands are feasible [1]. The WF further discuss the ΔRIB,4R where the value is proposed to be in the range -2.7dB – -2.2dB. This applies to the conducted performance. However, the main difficulty in the support of 4Rx in the below-1GHz-operation is the antenna. For good TRP/TRS performance coupling between antennas shall be kept low. For smaller form factor, such as handheld devices (compared to FWA), this may be a great challenge. At those low frequencies whole chassis contribute to the antenna performance or act as the antenna. For a two-antenna-system, the two antennas are usually put in the corners of the chassis and act diagonally over the chassis and the design could therefore be somewhat orthogonal with low antenna correlation. For a four-antenna-system this strategy may not possible unless the width of the device is made large. Therefore, in tablets or smart phones with very large form factors, acceptable performance may be obtained, but it will be extremely difficult to accommodate with 4Rx within compact phones. Therefore, TRP/TRS performance of 4RX/3TX is expected to be more degraded for the below-1GHz-operation compared to operation at higher frequencies. The above discussion is, maybe not connected to conducted performance, however, it may be a good reason to keep the ΔRIB,4R in the higher end (less negative dB) to indicate the anticipated weaker TRP/TRS performance.

When it comes to the conducted performance, which is the main topic of the discussion, the REFSENS or ΔRIB,4R may also be less beneficial, compared to operation at higher frequencies, due the increased complexity and longer traces since antennas need to be physically separated to a larger extent. Maintaining good isolation between RF traces in a smaller form factor, for frequencies < 1GHz, could be challenging. The already specified ΔRIB,4R for n8, n28, n71, n105, targeting FWA [4] is a good starting point for the bands listed in the WID [2] (Table 4.1-1). However, as previously mentioned, the smaller form factor of handsets (compared to FWA) may increase the design challenge, e.g., higher coupling between RF traces. We, therefore, suggest the ΔRIB,4R for the bands in Table 4.1-1 [2] to -2.2dB.

Proposal 1	ΔRIB,4R = -2.2dB for the bands in Table 4.1-1 in the WID.

- Conclusion

In this contribution, we have discussed ΔRIB,4R for, bands n28, n20, n8, n5, n26 and n71. The following proposal is made:

Proposal 1	ΔRIB,4R = -2.2dB for the bands in Table 4.1-1 in the WID.

- References

R4-2306553		“WF on low band 4Rx”, vivo

RP-230161		“Revised WID for Low NR band 4Rx and 3Tx”, OPPO

RP-230160		“Status report for WI 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC”, rapporteur: OPPO

TS 30.101-2		Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R