3GPP RAN WG4 Meeting #107		R4-2309045

Seoul, South Korea, May 22 – 26th, 2023

Agenda item:	7.32.2

Source:	Apple, Globalstar

Title:	System parameters for the NTN L-/S-band

WI/SI:	NR_NTN_LSband

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments that use mixed L-/S-band pairing, whereupon the UL part is one the L-band, while the DL part is one the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band [2].

During the RAN#106 and RAN4#106bis meeting initial discussions took place and some agreements for the system parameters were made, such as band definition, channel bandwidths, the channel and sync raster points [3]. However, not all the system related aspects were finalised, such as asymmetric band combinations and variable duplex distance. Thus, in this paper we present our further proposals for the system parameters of the new NTN band.

# 2	System parameters for the NTN L-/S-band

## 2.1	Channel bandwidth

Referring to the WI description [2], the objective is to enable the following channel bandwidths for the NTN band: 5, 10 and 15MHz. Even though the total bandwidth of the new band is 16.5MHz, the next lower standard channel bandwidth is 15MHz and that is the reason why the largest channel bandwidth for this band is assumed as 15MHz.

During the previous RAN4#106bis meeting it was agreed to introduce 5 and 10MHz channel bandwidth for the new NTN band. However, some concerns were raised for the 15MHz channel and whether the DL link budget would be sufficient to support that channel. It is worth noting that link budget analysis for the satellite bands, and S-band in particular, was already conducted by 3GPP back in Rel-16 and the corresponding results can be found in TR 38.821. In short, 3GPP considered several satellite configurations ranging from GEO 59dBW/MHz to LEO-600 28dBw/MHz, for which the corresponding CNR was estimated assuming 30MHz channel bandwidth.  Based on that analysis, the CNR could vary from approximately -5 to 7dB, whereupon for most cases it was around 1-2dB.

It also worth noting that during the TR 38.821 study item phase 3GPP did not put any restriction on the maximum transmission power of the satellite node, but rather assumed several options for the satellite EIRP dBW/MHz density. This is in line with the international radio regulations, which do not restrict the satellite node maximum transmission power either, but instead may only specify some restrictions on the power flux density as dB(W/m2).  From that perspective 15MHz channel for the NTN L-/S-band will have the same link budget as 5 or 10MHz channels.

Proposal 1:	Introduce the 15MHz channel bandwidth for the new NTN band.

## 2.2	Asymmetric band combinations and Tx-Rx separation

In addition to defining 5, 10 and 15MHz channel bandwidths as the baseline symmetric combinations, our view is that it would be beneficial to introduce also asymmetric channel bandwidth combinations to allow for more flexible system deployments. Like in the case of the TN bands, the definition of the NTN band n254 follows the regulatory framework that allows satellite operation in the whole 16.5MHz range, the actual license granted for the operator could be different and may vary from region to region. In fact, some countries may even grant licenses for different UL and DL channel bandwidths for which different UL and DL channel bandwidths will be configured. Referring to sub-clause 5.3.6 in TS 38.101-1, RAN WG4 has already defined asymmetric combinations for FDD bands, such as n5, n8, n24, n25, n26, n66, n70, n71, n91-n94. Thus, we suggest defining asymmetric channel bandwidths following the same principles that we already have in RAN WG4 specifications for the TN bands.

Proposal 2a:	Introduce 5+10MHz, 5+15MHz, and 10+15MHz UL/DL asymmetric channel combinations for the new NTN band (following the existing principles for the asymmetric combinations).

As for the Tx-Rx separation distance, while the fixed Tx-Rx separation distance with asymmetric band combination provide a higher degree of the deployment flexibility, we foresee a need to have a possibility to enable even more flexible deployments with variable Tx-Rx separation distance. Like the design of the terrestrial bands n91-n94 that have asymmetric band combinations, a higher degree of the deployment flexibility can be achieved if the asymmetric channel combinations are accompanied with the variable Tx-Rx separation distance. Thus, we suggest defining Tx-Rx separation distance as 862 – 885 MHz. The smallest Tx-Rx separation distance is calculated assuming 5MHz channels at the inner-most edges of the UL and DSL blocks, while the largest distance is calculated assuming 5MHz channels at the outer-most edges.

Proposal 2b:	The Tx-Rx separation distance is defined as 862 – 885 MHz for the new NTN band.

The corresponding changes for the asymmetric channel bandwidth combinations and the Tx-RX separation distance will look as follows (refer to [5] for the complete changes).

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| [n254] | 5 | 10,15 | 0 |
|  | 10 | 15 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz |
| n255 | -101.5 MHz |
| [n254] | 862 – 885 MHz |

# 3	Conclusions

In this paper we have presented our further considerations on the system parameters for the new NTN band. As a summary of our paper, 5, 10, and 15MHz channel bandwidths should be defined for the new NTN band following existing WID objectives. In addition to that, to facilitate more flexible channel arrangements in the irregular 16.5MHz spectrum block of the new NTN band we suggest defining asymmetric band combinations and variable Tx-Rx separation distance.

The corresponding changes to TS 38.101-5 can be found in [5].

Proposal 1:	Introduce 15MHz channel bandwidths for the new NTN band.

Proposal 2a:	Introduce 5+10MHz, 5+15MHz, and 10+15MHz UL/DL asymmetric channel combinations for the new NTN band.

Proposal 2b:	The Tx-RX separation distance is defined as 862 – 885 MHz for the new NTN band.

# 4	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

R4-2306561, "WF on NTN L-/S-band", Apple

R4-2309046, "RF requirements for the NTN L-/S-band", Apple, Globalstar

R4-2309047, "Introduction of the NTN L-/S-band", Apple, Globalstar