3GPP RAN WG4 Meeting #107		R4-2309057

Incheon, South Korea, May 22 - 26, 2023

Agenda item:	8.16.3

Source:	Apple

Title:	TRP TRS performance phase priorities and device pool information

Release:	Rel-18

Related WI:	NR_FR1_TRP_TRS_enh-Core

Document for:	Discussion and Decision

# 1	Introduction

The Rel-17 RAN4-led work item on the Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) [1] has concluded, with requirements defined for bands n41 and n78 [2].  The work continued into Rel-18, with the work item on the Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies having been approved in RAN #97 [3] and revised in [4].  During the RAN4 #106 meeting a discussion of the overall work scope took place, with some prioritization and rescoping agreed in [5].  This contribution seeks to highlight the overly ambitious scope of the Rel-18 TRP/TRS work item and proposes to focus the work by updating several objectives.  Additionally, we review the existing agreement on device pool information collection from Rel-17 and propose to reuse this approach in Rel-18.

# 2	Discussion

## 2.1	Requirement priorities

We start by examining the objectives of the core part of [4]:

### 4.1 Objective of Core part WI The following objectives are considered in this WI: - Enhancements of TRP TRS test methodology - Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline): - UE with NR 2Tx configuration - Case 1: TxD (i.e., TxD capability supported) - Case 2: single layer UL-MIMO (i.e., codebook-based capability supported) - Study proper configuration from UE implementation and test system feasibility perspective - Define test case applicability for case 1 and case 2 - RedCap devices (focused on wearable device only) - Wrist-worn device as 1st priority - Considering UEs with antenna configurations of 1 Tx, 1 Rx, 2 Rx - Forearm phantom should be specified - Consider the coordination with CTIA on this aspect - Other phantoms are not precluded - UE with DL/UL carrier aggregation configuration (2nd priority) - Independent measurements of each CC can be the baseline approach - Limited to Inter-band CA with up to 2DL/2UL cases - Study and, if feasible, specify reverb-chamber based test methodology to support SA and EN-DC TRP TRS testing: - Define test methodology for UE type with 1Tx and 2Tx configuration - Test scenarios support: at least browsing mode, talk mode, and wrist-worn mode - Perform harmonization activity with reference test method - Define harmonization framework and pass/fail criteria - Perform harmonization test campaign according to the framework - Note: the harmonization between anechoic chamber and Reverb-chamber based method can be applied independently for the conditions between 1Tx and 2Tx, among browsing mode, talk mode, and wrist-worn mode. - If reverb-chamber based test methodology is found harmonized with the reference test method, then further discuss applicability of reverb-chamber based test methodology for requirements conformance testing - Develop the Measurement Uncertainty (MU) assessment of the above newly defined test methodologies and test configurations [RAN5]: - Measurement Uncertainty (MU) aspects will be handled by RAN5 and the conclusions can be captured in a separate section of the TR - Browsing mode, talk mode, and wrist-worn mode should be considered - Study and specify the testing time reduction methodology for TRP and TRS testing During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9 WG1, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic

These objectives broadly translate into the following tracks:

-	2Tx radiated power verification methodology

-	Wrist-worn phantom definition and test methodology for RedCap

-	Reverb chamber harmonization with anechoic chamber

-	Testing time reduction solutions

-	MU assessment for each test methodology enhancement above

-	Second priority: CA TRP/TRS

With the core part of the work item scheduled to conclude by RAN #102, there are only 5 RAN4 working group meetings remaining to conclude on the prioritized objectives.  Furthermore, the majority of the open issues associated with the prioritized objectives are only in the early phases of discussion:  discussions on the 2Tx radiated power verification metric have only just begun (see Issue 1-1-7 in [6]), definition and related parameters of the wrist-worn phantom for RedCap are not yet available, and the reverb chamber harmonization effort has just identified the key activity to define the proper coherence bandwidth of RC for NR testing, including the associated procedures (see Issues 2-1-1, 2-1-2, 2-1-3 in [6]).  An important objective of the work item is to continue to specify testing time reduction methods to manage the overall testing time of NR devices, with open issues still remaining (see Issues 3-1-5, 3-2-1 in [6]).  Finally, with no new agreements in RAN4 #106 on the CA methodology for TRP/TRS since RAN4 #105, it seems reasonable to assume that there is no strong industry need to define such a methodology.

Proposal 1:	With significant technical work remaining before the prioritized core objectives of the Rel-18 TRP/TRS work item can be concluded, and with no recent progress on CA methodology apparent, it is reasonable to remove the scope for CA methodology for TRP/TRS methodology.

Next, we consider the objectives of the performance part of [4]:

### 4.2 Objective of Performance part WI Specify the performance part framework (general for both 1Tx and 2Tx requirements work): - Lab alignment outcome and performance campaign framework in Rel-17 should be baseline with the following considerations: - Decide whether lab alignment is needed for new test bands, new test scenarios and EN-DC configuration - Handheld UE type is the first priority - Consider UE size 1 and size 2 - UE size 1 is the first priority - Consider both browsing mode and talk mode - Further prioritization not precluded - Consider Power class 2 and 1.5 for 2Tx requirements, Power class 3 and 2 for 1Tx requirements - Requirement applicability of TxD and single-layer UL MIMO of the same UE shall be specified - Consider further prioritization of test cases - Taking following as a starting point to further discuss which information needed including for information disclosure: - [Number of models tested by the labs] - Number of vendors that produced the models - percentage of tested devices per vendor - Percentage of models per production year - [Power Class of the devices] - Note 1: Size 1 (wide, width >72mm and ≤92mm), Size 2 (narrow, width ≥56mm and ≤72mm); - Note 2: Browsing mode (hand phantom only), Talk mode (head and hand phantom); - Specify TRP TRS requirements and recommended tolerance for UE with NR 2Tx for handheld UE based on enhanced reference test method and defined performance part framework - Specify the requirements and test tolerance for UE with SA mode - Band n41, n77 and n78, as the first priority - FDD bands are not precluded - SA with 1 CC is the first priority - Specify TRP TRS requirements and recommended tolerance for UE with NR 1Tx (SA and EN-DC) based on reference test method and defined performance part framework - Specify the requirements and recommended tolerance for UE - SA: n3, n5, n7, n8, and n28 with PC3, n1, n77 and n79 with PC2 and PC3 - For talk mode requirements, n41 and n78 with PC2 should also be additionally considered - Further prioritization on band list not precluded - EN-DC: NR SA bands related EN-DC band combinations are the first priority, only NR carrier requirement will be specified - Rel-17 band combination conclusions in TS 38.161 should be the basis - For each newly added NR band to the scope, at least one example band combination shall be defined

RAN4 discussed how to prioritize the performance part of the work, with the following conclusions [6]:

Issue 4-2-1: Power class Agreements: - For bands support PC2 and PC3, both requirements are needed. RAN4 can further discuss how to define PC3, e.g. measurement or offset from PC2. Issue 4-2-2: Usage scenario Agreements: - Both browsing mode and talk mode shall be covered and the requirements shall be introduced together for the new bands Issue 4-2-3: Tx capability Agreements: - FFS whether down-selection needed or not among 1Tx and 2Tx - For phase-1 lab alignment activity, focus on 1 Tx Issue 4-2-4: Operation mode (SA and EN-DC modes) Agreements: - FFS whether down-selection/prioritization needed or not for SA and EN-DC mode. Issue 4-2-5: RedCap and CA requirements Agreements: - For Rel-18 TRP TRS requirement work, remove RedCap and CA requirements. Issue 4-2-6: Band prioritization - Agreements: - Down-selection can be further decided based on feedback from operators

Additionally, the LS to TFES further clarified the relationship between browsing mode, talk mode, and bands [7]:

RAN4 has decided to work on requirements for both browsing mode (hand phantom only) and talk mode (Beside Head and Hand), and the requirements for both browsing mode and talk mode will be introduced together per band

Observation 1:	It is now clarified that both PC2 and PC3 requirements are needed in bands where PC2 is defined; however, in Rel-17 only PC2 TRP requirements were specified for bands n41 and n78.  The objective to specify PC3 TRP requirements for these bands needs to be added to the Rel-18 WID.

Observation 2:	It is now clarified that RAN4 will specify both browsing and talk mode requirements per band; however, in Rel-17 only browsing mode requirements were specified for bands n41 and n78.  The objective to specify talk mode OTA requirements for these bands needs to be added to the Rel-18 WID.

Observation 3:	Lab alignment procedures and performance requirement work for band n28 did not conclude in the Rel-17 work item, and OTA requirements for this band need to be defined in Rel-18.

Observation 4:	As recommended by RAN4, RedCap requirement work needs to be postponed to a future release of the TRP/TRS work due to the lack of availability of RedCap devices for measurement.

Considering the bands included in the scope of the Rel-17 TRP/TRS work item (n28, n41, n78), we note that Rel-18 effort is needed to conclude the partially completed performance objectives for bands n41, and n78.  Since no requirement objectives for band n28 were achieved in Rel-17, we treat n28 as an additional band in Rel-18 from the workload perspective.  The current scope of the work item spans the following bands in addition to the Rel-17 set: n1, n3, n5, n7, n8, n77, and n79.  Clearly, prioritization of the bands is an essential task for RAN to undertake this meeting.

Proposal 2:	The Rel-18 TRP/TRS work item shall aim to complete full coverage of band n28, n41, and n78 requirements, including performance objectives which were not concluded in Rel-17, and it shall also aim to define requirements for band n77.

The remaining aspect is the currently ongoing definition of the 2Tx radiated output power test methodology and the related requirements.  This is a significant and new direction for RAN4.  Given the group’s decision to perform lab alignment in two phases (phase 1 for 1Tx methodology and phase 2 for 2Tx methodology), it is reasonable to map the 2Tx radiated output power requirements to a phase 2 in the requirement development process.  Combining these proposals and observations together, it is possible to visualize the proposed rescoping of the work item graphically, as shown in Table 1 below in a color-coded format:  completed Rel-17 test methods and requirements in green, proposed rescoping of Rel-18 work in orange, and propose removal of scope in cells with no background color.

Table 1: Status of TRP/TRS objectives by release and Rel-18 rescoping proposal

| Aspect / Feature |  | TRS |  |  | 1Tx TRP PC3 |  |  | 1Tx TRP PC2 |  | 2Tx Rad Power |  | CA TRP & TRS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Browse | Talk | RedCap | Browse | Talk | RedCap | Browse | Talk | Browse | Talk |  |
| Test method and prelim MU |  | R17 | R17 | R18 | R17 | R17 | R18 | R17 | R17 | R18 | R18 | R18 2p |
| n1 | 72 < w ≤ 92 mm | R18 | R18 |  | R18 | R18 |  | R18 | R18 |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p |  | R18 2p | R18 2p |  | R18 2p | R18 2p |  |  |  |
| n3 | 72 < w ≤ 92 mm | R18 | R18 |  | R18 | R18 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p |  | R18 2p | R18 2p |  |  |  |  |  |  |
| n5 | 72 < w ≤ 92 mm | R18 | R18 |  | R18 | R18 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p |  | R18 2p | R18 2p |  |  |  |  |  |  |
| n7 | 72 < w ≤ 92 mm | R18 | R18 |  | R18 | R18 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p |  | R18 2p | R18 2p |  |  |  |  |  |  |
| n8 | 72 < w ≤ 92 mm | R18 | R18 |  | R18 | R18 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p |  | R18 2p | R18 2p |  |  |  |  |  |  |
| n28 | 72 < w ≤ 92 mm | R18 | R18 |  | R18 | R18 |  |  |  |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p |  | R18 2p | R18 2p |  |  |  |  |  |  |
| n41 | 72 < w ≤ 92 mm | R17 | R18 |  | missing | missing |  | R17 | R18 | R18 2p | R18 2p |  |
|  | 56 ≤ w ≤ 72 mm | missing | missing |  | missing | missing |  | missing | R18 2p | R18 2p | R18 2p |  |
| n77 | 72 < w ≤ 92 mm | R18 | R18 |  | R18 | R18 |  | R18 | R18 | R18 2p | R18 2p |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p |  | R18 2p | R18 2p |  | R18 2p | R18 2p | R18 2p | R18 2p |  |
| n78 | 72 < w ≤ 92 mm | R17 | R18 |  | missing | missing |  | R17 | R18 | R18 2p | R18 2p |  |
|  | 56 ≤ w ≤ 72 mm | missing | missing |  | missing | missing |  | missing | R18 2p | R18 2p | R18 2p |  |
| n79 | 72 < w ≤ 92 mm | R18 | R18 |  | R18 | R18 |  | R18 | R18 |  |  |  |
|  | 56 ≤ w ≤ 72 mm | R18 2p | R18 2p |  | R18 2p | R18 2p |  | R18 2p | R18 2p |  |  |  |

With the performance phase of the work slated to begin in May of 2023, it is important for RAN4 to reach a decision on the scope of the performance work, in order to allow the lab alignment and measurement campaigns to begin without delay.

Proposal 3:	RAN4 should reach a decision on the exact scope of the performance phase of the Rel-18 TRP/TRS activities in order to allow lab alignment and measurement campaigns to proceed without delay.

Considering the Rel-18 TRP TRS milestones call for start of Rel18 lab alignment activity after the RAN4#107 meeting in May 2023, it is critical to finalize at least the immediate scope for the same in terms of bands and in line with Proposal 3.

Proposal 4: 	Update the Rel18 TRP TRS Work Plan [4] with details on lab alignment framework and focus over next two RAN4 meetings.

--------------------------------------------------------------------------------------------------------------------------

- RAN4 #107 F2F (May 2023)

- Core part

- Anechoic chamber test methodology

  - Make progress on AC test methodology to support RedCap and DL/UL CA

- Reverberation chamber test methodology

  - Further discuss Reverberation chamber test methodology for other test cases

  - Collect harmonization measurement results

- Measurement Uncertainty (MU) assessment

  - RAN5 conclude MU for AC and RC for 1Tx configurations

    - Text Proposals on Measurement uncertainty should be finalized and sent to RAN4

  - RAN5 make progress on MU work for other test cases

- Testing time reduction

  - Make progress on Testing time reduction solutions

- Performance part

- Performance part framework

  - Discuss performance framework on FR1 TRP TRS requirement

    - Conclude framework for Rel-18 lab alignment activity (using reference method)

    - Collect list of volunteer labs for lab alignment activity

    - Start Rel-18 lab alignment activity for 1Tx Talk Mode (SA bands n41, n78, n28, n77 leveraged from n78), 1Tx Browsing Mode (SA bands n28, n77 leveraged from n78) for Size 1 devices.

    - Define further prioritization of test cases

    - Discuss UE information disclosure for performance measurement campaign

- NR 2Tx requirement

  - Collect views on 2Tx requirement discussion

- NR 1Tx requirements

  - Collect views on 1Tx requirement discussion

- RAN4 #108 F2F (Aug 2023)

- Core part

- Anechoic chamber test methodology

  - Conclude on AC test methodology to support RedCap

  - Further discuss test method for DL/UL CA

- Reverberation chamber test methodology

  - Further discuss Reverberation chamber test methodology for other test cases

  - Finalize the collection of harmonization measurement results

  - Discuss harmonization outcome

  - Discuss applicability of RC test method

- Measurement Uncertainty (MU) assessment

  - RAN5 conclude MU work for Anechoic chamber and Reverberation chamber for 2Tx

  - Make progress on MU for other test cases

- Testing time reduction

  - Make progress on Testing time reduction solutions

- Performance part started

- Performance part framework

  - Discuss performance framework on FR1 TRP TRS requirement

    - Collect Rel-18 lab alignment measurement results for 1Tx Talk Mode (SA bands n41, n78, n28, n77 leveraged from n78), Browsing Mode (SA bands n28, n77 leveraged from n78) for Size 1 devices.

    - Start Rel-18 lab alignment activity for 2Tx Talk Mode and browsing mode (SA bands n41, n78, n77 leveraged from n78) for Size 1 devices.

    - Define further prioritization of test cases

    - Discuss UE information disclosure for performance measurement campaign

- NR 2Tx requirement

  - Collect initial views on 2Tx requirement discussion

- NR 1Tx requirements

  - Collect initial views on 1Tx requirement discussion

## 2.2	Device pool information

With the agreement on the template for device pool collection in the context of the Rel-17 TRP/TRS work [8], a proposal to include yet more information in the Rel-18 TRP/TRS work was submitted to the RAN4 #106 meeting [9]:

Proposal 2: It is proposed to include the following fields in the datasheet that will be provided to the laboratories for collecting the measurement results: - Device model - Device vendor - Power Class - Supported bands - Year of production - Device certification (PTCRB, GCF, N/A) - Market level (entry, medium or high level) - Commercially available (YES or NO)

In general, our understanding of the scope of this discussion is to determine whether it is feasible for a neutral observer (such as the RAN4 Secretary) to collect information from test labs participating in the TRP/TRS and MIMO OTA performance campaigns and to provide the information to 3GPP RAN4 as a way to improve companies’ confidence in the agreed requirement values.  For the already concluded TRP/TRS and MIMO OTA requirements in Rel-17, this activity is purely informational in nature (i.e. there is no scope for revising the agreed requirements as part of this activity).  For the Rel-18 continuation of TRP/TRS and MIMO OTA work, this information can be provided as part of the performance requirement development process and can be considered together with the requirement proposals.

Proposal 5:	RAN4 should continue to use the same neutral observer for the collection of additional device pool information for the Rel-18 OTA data as it had already been agreed to do for the Rel-17 OTA data.

Disclosing the number of models tested by the labs is not feasible to implement, since it would require the disclosure of exact device model lists tested in each lab. The RAN4 TRP/TRS WID rapporteur already provided a good explanation during the course of the work item of how, in line with the planned statistical data-driven approach, the CDF curve depicted a good statistical spread and even by artificially plugging in duplicate data (as a hypothetical extreme). The conclusion reached was that the %-tiles did not shift significantly. Even if two labs tested the same device (which is quite possible as it is logistically impossible for 8 labs to be coordinating device lists) it provides a distinct set of data from separate test systems which is still useful. Unless the assumption is the same device was tested >4 or 5 times (which seems highly unlikely from a simple review of the raw measurement data from each lab and the CDF curve explained by the rapporteur), this approach does not provide any useful insights.

Observation 5:	Disclosing the number of models tested by the labs is not feasible to implement, since it would require the disclosure of exact device model lists tested in each lab.

Disclosing the number of vendors that produced the models is acceptable if vendor name/device models are not disclosed. This could work if each lab provides to the neutral party a spread of vendors whose devices were used (as “Vendor A, Vendor B, Vendor C”. Then the neutral party could summarize the number of vendors, without disclosing the vendor names, per lab in a report to 3GPP.

Observation 6:	It is feasible for the neutral party to summarize the number of vendors per lab, without disclosing the vendor names, in a report to 3GPP.

The information about tested devices per vendor can be included in the report to 3GPP.  This can result from the process in Observation 2, and the information shared must be the summary across all labs, without per-lab information.

Observation 7:	It is feasible for the neutral party to summarize the percentage of tested devices per vendor, without disclosing the device model and vendor names, in a report to 3GPP.  Only the complete list to be shared publicly (no per lab list or information).

The information about the percentage of models per production year can be included in the report to 3GPP.  This is acceptable if no device models or vendor names disclosed. Labs would share separate lists with only production year to the neutral party for devices they tested. Only the summary information, compiled by the neutral party, about the percentage of models per production year can be reported to 3GPP (i.e. no per-lab lists).

Observation 8:	It is feasible for the neutral party to summarize the percentage of models per production year, without disclosing the device model and vendor names, in a report to 3GPP.  Only the complete list to be shared publicly (no per lab list or information).

Observation 9:	Power class information is already provided in the lab reports as part of the performance requirement framework, and it is not necessary to additionally collect this information via the neutral party.

The eventual RAN4 agreement on the Rel-17 template [8] took Observations 1 through 5 into account and balanced them against operator requests.  This template tracks the following information:

A. Percentage of Models Per Calendar Year (NOTE 1) B. Number of device vendors used per Lab (NOTE 4) C. Percentage of Tested Devices per Vendor (NOTE 6) D. Number of Device Models (NOTE 9)

Proposal 6:	It is proposed to continue to use the Rel-17 Template for Device Information Collection in the development of Rel-18 OTA requirements without any further modification.

# Conclusions

This contribution seeks to highlight the overly ambitious scope of the Rel-18 TRP/TRS work item and proposes to focus the work by updating several objectives.  Additionally, we review the existing agreement on device pool information collection from Rel-17 and propose to reuse this approach in Rel-18.  The following observations and proposals are made:

Observation 1:	It is now clarified that both PC2 and PC3 requirements are needed in bands where PC2 is defined; however, in Rel-17 only PC2 TRP requirements were specified for bands n41 and n78.  The objective to specify PC3 TRP requirements for these bands needs to be added to the Rel-18 WID.

Observation 2:	It is now clarified that RAN4 will specify both browsing and talk mode requirements per band; however, in Rel-17 only browsing mode requirements were specified for bands n41 and n78.  The objective to specify talk mode OTA requirements for these bands needs to be added to the Rel-18 WID.

Observation 3:	Lab alignment procedures and performance requirement work for band n28 did not conclude in the Rel-17 work item, and OTA requirements for this band need to be defined in Rel-18.

Observation 4:	As recommended by RAN4, RedCap requirement work needs to be postponed to a future release of the TRP/TRS work due to the lack of availability of RedCap devices for measurement.

Observation 5:	Disclosing the number of models tested by the labs is not feasible to implement, since it would require the disclosure of exact device model lists tested in each lab.

Observation 6:	It is feasible for the neutral party to summarize the number of vendors per lab, without disclosing the vendor names, in a report to 3GPP.

Observation 7:	It is feasible for the neutral party to summarize the percentage of tested devices per vendor, without disclosing the device model and vendor names, in a report to 3GPP.  Only the complete list to be shared publicly (no per lab list or information).

Observation 8:	It is feasible for the neutral party to summarize the percentage of models per production year, without disclosing the device model and vendor names, in a report to 3GPP.  Only the complete list to be shared publicly (no per lab list or information).

Observation 9:	Power class information is already provided in the lab reports as part of the performance requirement framework, and it is not necessary to additionally collect this information via the neutral party.

Proposal 1:	With significant technical work remaining before the prioritized core objectives of the Rel-18 TRP/TRS work item can be concluded, and with no recent progress on CA methodology apparent, it is reasonable to remove the scope for CA methodology for TRP/TRS methodology.

Proposal 2:	The Rel-18 TRP/TRS work item shall aim to complete full coverage of band n28, n41, and n78 requirements, including performance objectives which were not concluded in Rel-17, and it shall also aim to define requirements for band n77.

Proposal 3:	RAN4 should reach a decision on the exact scope of the performance phase of the Rel-18 TRP/TRS activities in order to allow lab alignment and measurement campaigns to proceed without delay.

Proposal 4: 	Update the Rel18 TRP TRS Work Plan [4] with details on lab alignment framework and focus over next two RAN4 meetings.

Proposal 5:	RAN4 should continue to use the same neutral observer for the collection of additional device pool information for the Rel-18 OTA data as it had already been agreed to do for the Rel-17 OTA data.

Proposal 6:	It is proposed to continue to use the Rel-17 Template for Device Information Collection in the development of Rel-18 OTA requirements without any further modification.

# 4	References

- RP-211158, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, 3GPP RAN #92, June 2021

- RP-222152, "Status report for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC); rapporteur: vivo," 3GPP RAN #97, September 2022

- RP-222669, “New WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies,” vivo, 3GPP RAN #97, September 2022

- RP-223112, “Revised WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC),” vivo, 3GPP RAN #98, December 2022

- R4-2302920, “Working procedure for Rel-18 TRP TRS requirements,” vivo, 3GPP RAN4 #106, February 2023

- R4-2302917, “WF for Rel-18 TRP/TRS requirements,” vivo, 3GPP RAN4 #106, February 2023

- R4-2302981, “Reply LS to ETSI TC MSG/TFES on NR TRP and TRS requirements,” 3GPP RAN4 #106, February 2023

- R4-2302919, “Template for Device Information Collection,” Apple, 3GPP RAN4 #106

- R4-2300669, “Device’s information disclosure for FR1 TRP-TRS and MIMO OTA (Rel-18),” Telecom Italia, 3GPP RAN4 #106, February 2023