3GPP TSG-RAN4 Meeting #107		R4-2309058

Incheon, South Korea, May 22nd – 26th, 2023

Agenda item:	8.15.3

Source:	Apple Inc.

Title:	On 3MHz CBW A-MPR for NS_12, NS_13 and NS_15

WI/SI:	NR_FR1_lessthan_5MHz_BW-Core

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

A way forward was agreed in RAN4#106-bis-e [1] with agreement on SEM for channel bandwidth of 3MHz and request for further input on A-MPR for NS_12, NS_13 and NS_15. This paper presents simulations results and has proposals for A-MPR values and regions.

# 2  Discussion

In the following proposals for A-MPR values are provided. The emission requirements and signalling flags are not provided here but can be found in [2].

The following assumptions for the simulations were used:

- Power Class 3

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

###### 2.1 A-MPR for NS_12

The A-MPR for NS_12 was simulated by deploying the 3MHz channel at lowest possible carrier frequency of 815.5MHz. The results for QPSK with DFT-s-OFMD and CP-OFDM are shown below.

Figure 1: NS_12 A-MPR for QPSK

According to simulation results the A-MPR for NS_12 and 3MHz channel bandwidth is proposed in Table 1 and Table 2.

Table 1: Contiguous allocation A-MPR for NS_12

| Channel BW (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 3 | ≤ 0.9 | > 0 | A2 |

Table 2: A-MPR for modulation and waveform type for NS_12

| Waveform | Modulation | A2 |
| --- | --- | --- |
|  |  | Outer / Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 4.5 |
|  | QPSK | ≤ 5.0 |
|  | 16QAM | ≤ 5.0 |
|  | 64QAM | ≤ 5.0 |
|  | 256QAM | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 6.0 |
|  | 16QAM | ≤ 6.0 |
|  | 64QAM | ≤ 6.0 |
|  | 256QAM | ≤ 6.5 |

Proposal 1: Consider Table 2 when specifying A-MPR for NR_12.

###### 2.2 A-MPR for NS_13

The A-MPR for NS_13 was simulated by deploying the 3MHz channel at lowest possible carrier frequency of 818.5MHz. The results for QPSK with DFT-s-OFMD and CP-OFDM are shown below.

Figure 1: NS_13 A-MPR for QPSK

Table 3: Contiguous allocation A-MPR for NS_13

| Channel BW (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 3 | ≤ 0.54 | > 0 | A4 |

Table 4: A-MPR for modulation and waveform type for NS_13

| Waveform | Modulation | A4 |
| --- | --- | --- |
|  |  | Outer / Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3.5 |
|  | QPSK | ≤ 4.0 |
|  | 16QAM | ≤ 4.0 |
|  | 64QAM | ≤ 4.0 |
|  | 256QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 5.5 |
|  | 16QAM | ≤ 5.5 |
|  | 64QAM | ≤ 5.5 |
|  | 256QAM | ≤ 6.5 |

Proposal 2: Consider Table 4 when specifying A-MPR for NR_12.

###### 2.2 A-MPR for NS_15

The A-MPR for NS_15 was simulated by deploying the 3MHz channel at lowest possible carrier frequency of 847.5MHz. The results for QPSK with DFT-s-OFMD and CP-OFDM are shown below.

Figure 1: NS_15 A-MPR for QPSK

Table 5: Contiguous allocation A-MPR for NS_15

| Channel BW (MHz) | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 3 | ≤ 1.62 | > 0 | A5 |

Table 6: A-MPR for modulation and waveform type for NS_15

| Waveform | Modulation | A5 |
| --- | --- | --- |
|  |  | Outer / Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3.5 |
|  | QPSK | ≤ 4.0 |
|  | 16QAM | ≤ 4.0 |
|  | 64QAM | ≤ 4.0 |
|  | 256QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 5.5 |
|  | 16QAM | ≤ 5.5 |
|  | 64QAM | ≤ 5.5 |
|  | 256QAM | ≤ 6.5 |

Proposal 2: Consider Table 6 when specifying A-MPR for NR_15.

# 4	Conclusions

This paper presents simulations results and has proposals for A-MPR values and regions. The following proposals for 3MHz CBW and NS_12, NS_13, NS_15 are made:

Proposal 1: Consider Table 2 when specifying A-MPR for NR_12.

Proposal 2: Consider Table 4 when specifying A-MPR for NR_13.

Proposal 3: Consider Table 6 when specifying A-MPR for NR_15.

# 4	References

- R4-2306616, “WF on UE RF requirements“, Nokia, 3GPP TSG-WG4 Meeting RAN#106bis-e.