3GPP TSG-RAN4 Meeting #107		R4-2309060

Incheon, South Korea, May 22nd – 26th, 2023

Agenda item:	7.22.2

Source:	Apple Inc.

Title:	On PC2 A-MPR Proposal for NS_18

WI/SI:	HPUE_NR_FR1_FDD_R18

Release:	Rel-18

Document for:	Approval

# 1	Introduction

Introduction of PC2 A-MPR for NS_18 was discussed during last RAN4 meeting, RAN4#106-bis-e. Extensive evaluation and simulation results were provided in [1]. This contribution provides further considerations on the initial proposals.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions for the simulations are used:

- Power Class 2

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

The simulations are done for NS_18. The spurious co-existence is set according to clause 6.5.3.3.3 which is copied below.

###### 2.2 Simulation results

Simulations have been conducted for all channel bandwidth from 5MHz, 10MHz, 15MHz, 20MHz, 25MHz and 30MHz. The A-MPR regions proposed in [1] were used as starting point. Selected simulation results can be found in Table 1 where allocations for DFT-s-OFDM and CP-OFDM for QPSK are provided.

Table 1: A-MPR simulation results for NS_43

###### 2.3 On 5MHz CBW

Based on the proposal made in [1] and the discussion from RAN4#106-bis-e it is proposed to apply the A-MPR values according to [1]. The values are copied for reference and can be found in table 2.

Table 2: A-MPR proposal for 5MHz channel bandwidth

| Modulation/Waveform |  | A1 (dB) |
| --- | --- | --- |
|  |  | Outer |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [3.5] |
|  | QPSK | ≤ [3.5] |
|  | 16 QAM | ≤ [4.0] |
|  | 64 QAM | ≤ [4.5] |
|  | 256 QAM | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ [5.5] |
|  | 16 QAM | ≤ [5.5] |
|  | 64 QAM | ≤ [6.0] |
|  | 256 QAM | ≤ [9.0] |

Proposal 1: For 5MHz channel bandwidth apply the A-MPR provided in Table 2.

###### 2.4 On 10MHz, 15MHz and 20MHz CBW

The proposal from [1] is fine. Our simulations support adding 3dB power back-off to PC3 A-MPR values.

Table 3: A-MPR proposal for 10MHz, 15MHz and 20MHz channel bandwidth

| Modulation/Waveform |  | A2 (dB) |
| --- | --- | --- |
|  |  | Inner/Outer |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [5+3] |
|  | QPSK | ≤ [5+3] |
|  | 16 QAM | ≤ [6+3] |
|  | 64 QAM | ≤ [7+3] |
|  | 256 QAM | ≤ [9+2] |
| CP-OFDM | QPSK | ≤ [6.5+3] |
|  | 16 QAM | ≤ [7+3] |
|  | 64 QAM | ≤ [8.5+3] |
|  | 256 QAM | ≤ [11.5] |

Proposal 2: Apply A-MPR regions and MPR from Table 3 for 10MHz, 15MHz and 20MHz channel bandwidth.

###### 2.5 On 25MHz and 30MHz CBW

The contribution [1] proposed PC2 A-MPR re-using the same regions defined for PC3. Due to the out-of-band emission limit there exist a multitude of allocations in need of increased power back-off. All of those allocations which exceed MPR are covered with the given regions. With power increase from PC2 there exist a several allocations which exceed MPR and require a new region to cover their power back-off need. In case of 30MHz CBW certain outer allocations outside of any region can reach up to 2dB while inner can have up to 1.5dB for DFT-s-QPSK. In case of CP-OFDM the outer allocations can reach up to 4dB while inner can reach 3dB. Therefore, it is proposed to consider an additional region covering inner and outer allocations. The new region uses the label A6. As basis the PC3 inner MPR is used, and the delta is indicated to show that no allocations obtains more than 3dB additional power back-off.

Table 5: PC2 region definition for 25MHz and 30MHz channel bandwidth

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions (PC2) |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

The proposed A-MPR for 15MHz channel bandwidth in [1] is agreeable from our point of view and it is proposed to specify the values as copied below in Table 6.

Table 6: A-MPR for 20MHz and 30 MHz channel bandwidth

| Modulation/Waveform |  | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | [3+0.5] | [8+2] | [3+1.5] | [0+2.0] |
|  | QPSK | [3+0.5] | [8+2] | [3+1.5] | [0+2.0] |
|  | 16 QAM | [3+0.5] | [8+3] | [3+2] | [1.0+1.0] |
|  | 64 QAM | [3+0.5] | [8+3] | [4.5+1] | [2.5] |
|  | 256 QAM | [3+0.5] | [8+3] | [5.5] | [4.5] |
| CP-OFDM | QPSK | [4.5+0.5] | [9.5+2] | [5+1] | [1.5+2.5] |
|  | 16 QAM | [4.5+0.5] | [9.5+2] | [5+1] | [2.0+2.0] |
|  | 64 QAM | [4.5+0.5] | [9.5+2] | [5.5+0.5] | [3.5+0.5] |
|  | 256 QAM | [4.5+0.5] | [9.5+2] | [7.5] | [6.5] |

In total the PC2 regions would be arranged as shown below:

Proposal 3: Apply A-MPR regions and MPR from Table 5 and Table 6 for 25MHz and 30MHz channel bandwidth.

# 4	Conclusions

This contribution provides A-MPR simulation results and proposals on NS_18 based on RAN4#106-bis-e contribution and discussion. The following proposals are made:

Proposal 1: For 5MHz channel bandwidth apply the A-MPR provided in Table 2 (same values as proposed in R4-2304519).

Proposal 2: Apply A-MPR regions and MPR from Table 3 for 10MHz, 15MHz and 20MHz channel bandwidth (same values as proposed in R4-2304519).

Proposal 3: Apply A-MPR regions and MPR from Table 5 and Table 6 for 25MHz and 30MHz channel bandwidth

# 4	References

- R4-2304519, “PC2 A-MPR for band n28 NS_18“, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #106bis-e