3GPP TSG-RAN4 Meeting #107		R4-2309073

Incheon, South Korea, May 22nd – 26th, 2023

Agenda item:	7.27.2

Source:	Apple

Title:	On simultaneous Rx/Tx for NR inter-band combinations

Release:	Rel-18

Document for:	Approval

# 1  Introduction

The WI [1] objective is to introduce requirements for simultaneous Rx-Tx for certain band combinations. The WF [2] encourages companies to further discuss the implementation challenges of CA_n40A-n41A with simultaneous Rx-Tx and MIMO operation.

# 2  Discussion

The WI [1] features a list of band combinations which are under investigation for non-synchronized scenario and specifying applicable requirements for simultaneous Rx-Tx operation. According to TS38.101-1 clause 7.2 a UE is required to feature a minimum of four Rx antenna ports for band n41. This requirement applies if n41 is configured as standalone band or if n41 is part of a band combination. Due to near proximity of n40 and n41, those bands are typically grouped together using the same antenna switch to reduce implementation complexity and number of required antennas.

To tackle the implementation challenges RAN4 discusses several options and captured them in WF[2]. For reference the options are provided below:

In case of non-simultaneous Rx-Tx it is possible to share antennas between n40 and n41 as uplink and downlink transmission never occurs at the same time for both bands. However, in case of simultaneous Rx-Tx between n40 and n41 it is challenging to share antennas for uplink and downlink as filter isolation is marginal due to the close frequency separation of the two bands. Additionally, it is challenging to optimize and achieve the required performance for all the required antennas over the entire frequency of n40 and n41. If an implementation is done for simultaneous Rx/Tx then separate antennas are needed to cover the frequency range. Integrating additional antennas for this frequency range might not be an issue for FWA/CPE type devices but it is a challenging task for handhelds due to the small form factor.

Regarding option 1c, the relaxation for carrier aggregation with simultaneous Rx-Tx between n40 and n41 can be implemented by introducing a new note to Table 7.3.2-1b. The note specifies the relaxation for handheld devices only. The proposal can be found below:

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands Operating band / SCS / Channel bandwidth / REFSENS. Operating band; SCS kHz; Channel bandwidth (MHz); REFSENS (dBm)8; Duplex Mode. …. n40; 15; 5, 10, 15, 20, 25, 30, 40, 50; -100 + 10log10(NRB/25); TDD. 30; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.5 + 10log10(NRB/11). n411,11, n901,11; 15; 5, 10, 15, 20, 25, 30, 35, 40, 45, 50; -94.8 + 10log10(NRB/52); TDD. 30; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.5 + 10log10(NRB/11). …. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. Note 11: Four Rx antenna ports are not mandatory for this band when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured. This relaxation does not apply to FWA form factor devices..

Proposal 1: Preference is option 1c from WF R4-2306547 as it would result into considerable reduced implementation complexity for CA_n40A-n41A due to the lower number of required antennas.

The <Way forward 2-2> encourages further discussion on scheduling restriction. In case antenna optimization is required for carrier aggregation with bands typically grouped together, achieving all the necessary performance requirements for both bands (in this case n40 and n41) when carrier aggregation, simultaneous Rx-Tx and additional UE features (such as SRS antenna switching) needs to be supported is challenging. It is therefore supported to discuss scheduling restrictions.

Proposal 2: Further discuss the scheduling restrictions for SRS antenna switching.

# 3  Conclusions

This contribution provides considerations on simultaneous Rx-Tx for CA_n40A-n41A. The following observations and proposal are made:

Proposal 1: Preference is option 1c from WF R4-2306547 as it would result into considerable reduced implementation complexity for CA_n40A-n41A due to the lower number of required antennas.

Proposal 2: Further discuss the scheduling restrictions for SRS antenna switching.

# 4  References

- R4-2217123, “Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18”, 3GPP TSG-RAN WG4 Meeting #104-bis-e

- R4-2306547, “WF on simultaneous Rx/Tx transmission”, Huawei, HiSilicon, Skyworks Solutions Inc., 3GPP RAN WG4 Meeting #106bis-e