3GPP TSG-RAN4 Meeting #107	R4-2309093

# Incheon, South Korea, May 22nd – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.872 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 38.872: TP for CA_n28A-n105A UE RF architectures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_sub1GHz_combo_enh |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To capture the UE RF architectures assumption for CA_n28A-n105A in TR 38.872. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding text proposal on UE RF architectures assumption for CA_n28A-n105A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_n28A-n105A UE RF architectures assumption is missing for future reference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1, 5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

### 5.5.1	General

Figure 5.5.1-1 shows the aggregated spectrum allocation for CA_n28A-n105A where no frequency range restriction has been indicated for either of the constituent bands.

Figure 5.5.1-1 CA_n28A-n105A spectrum allocation

In this study item, only single UL configuration is considered for the combination due to the concern of 2UL IMD issue as described in sub-clause 5.5.2.3.

### 5.5.2	UE RF architectures assumption

Both 2-antenna and 3-antenna implementations are considered in the study item where the corresponding UE RF architectures are further detailed in the following sub-clauses.

#### 5.5.2.1	UE RF architectures with two antennas

For CA_n28A-n105A with 2-antenna implementation, there can be two potential architecture variants, one with a triplexer and a quadplexer in the main signal path and a duplexer in the diversity signal path, as shown in Figure 5.5.2.1-1, the other is with dual triplexers in main signal path and a triplexer in diversity signal path, as shown in Figure 5.5.2.1-2.

Figure 5.5.2.1-1 CA_n28A-n105A UE RF first architecture variant with 2-antenna implementation

Figure 5.5.2.1-2 CA_n28A-n105A second architecture variant with 2-antenna implementation

For the first architecture variant, due to the relatively wide frequency range in the middle sub-band for the main signal path triplexer, it is considered rather challenging and impracticable for implementation and will not be further pursued in this study item.

For the second architecture variant, both the main antenna and diversity antenna design need to cover the entire spectrum range of 191 MHz simultaneously as shown in Figure 5.5.1-1, which is equivalent to a 27% bandwidth ratio and that would exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination likely would be compromised.

#### 5.5.2.2	UE RF architectures with three antennas

For the 3-antenna implementation, the two of the three antennas are used in the main path to aggregate the n28 and n105 signals over the air, as shown in Figure 5.5.2.2-1. As n28 and n105 signals do not need to be combined through a multiplexer, there is no additional insertion loss in both n28 and n105 main signal paths as compared to the single-band implementation.

Figure 5.5.2.2-1 CA_n28A-n105A UE RF architecture with 3-antenna implementation

Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antennas as compared to single-antenna implementation. However, the additional antenna cannot be added without occupying more phone space. Therefore, the feasibility on placing more than two low-band antennas in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor.

#### 5.5.2.3	2UL IMD issue

2UL IMD3/IM5 may fall into either n105 DL or n28 DL or both DL bands simultaneously to cause REFSENS degradation (MSD), as conceptually illustrated in Figure 5.5.2.3-1.

Figure 5.5.2.3-1 Conceptual illustration on 2UL IMD3/IMD5 impact to DL bands for CA_n28A-n105A

For both 2-antenna and 3-antenna implementations, depending on the carrier allocation and Tx filter isolation on the two UL carriers, the MSD caused by 2UL IMD3 to n105 DL carrier could potentially be higher than 30 dB when n28 UL carrier is close to n105 UL band where the n105 Tx filter does not provide any rejection to n28 UL carrier and n105 PA reverse IMD would dominate the MSD. For carrier allocations where both Tx filters can provide sufficient isolation to the cross-band UL carriers, the MSD caused by 2UL IMD3 could still be around 20 dB to either or both of the DL carriers which would be dominated by PA forward IMD.

Due to the concern of the 2UL IMD issue, only single UL configuration is considered in this study item where UL can be in either n28 or n105.

#### 5.5.2.4	Cross-band interference

Apart from the 2UL IMD concern, the combination may also be subject to cross-band UL to DL interference issue due to frequency proximity even under single UL operation where the REFSENS impact needs to be further analyzed.

<<< End of changed sections >>>