3GPP TSG-RAN WG4 Meeting #107	R4-2309127

Incheon KR, May 22 – 26, 2023

Agenda item:	8.26.2.3

Source: 	Qualcomm Incorporated

Title: 	On requirements for Rel-17 MUSIM gaps – Network A

Document for:	Discussion

# Introduction

RAN4 continued discussing requirements for Rel-17 MUSIM gaps in RAN4#106bis-e. The latest agreements and open issues were captured in a WF [1].

In this paper, we will discuss our proposals for defining requirements in Network A.

# Discussion

Measurement requirements in network A need to be updated to account for any collisions between MUSIM gaps and measurement gaps or RS resources. Fortunately, RAN4 went through a similar exercise in Rel-17 when requirements were updated for scenarios where concurrent MG are configured.

It should be straightforward to extend the requirements to include collisions with MUSIM gaps. For example, the requirements for NR intra-frequency measurements without gaps (38.133, clause 9.2.5) define a scaling factor Kp to account for collisions with measurement gaps, as shown below.

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.2B.3.

Kp = 1 when Navailable = 0.

In the above definition, two quantities would need to be modified:

- The duration of the window W should be updated to include the maximum period of configured MUSIM gaps, if any.

- Navailable should be updated to also account for collisions between SMTC and MUSIM gaps within the window W, after accounting for collisions between MUSIM gaps and measurement gaps.

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling

- Proposals

  - P1: Frameworks of LBT failure in NR-U design can be used as starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps (Apple)

  - P2: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured. (CMCC xiaomi vivo ZTE oppo Huawei Ericsson Qualcomm MTK Nokia)

  - P3: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority. (Ericsson Nokia)

Proposal 1: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured.

Proposal 2: The following parameters need to be updated to account for collisions with MUSIM gaps:

- Kp for intra-frequency and inter-frequency measurements without gaps

- Kgap for intra-frequency and inter-frequency measurements with gaps

- Kgap_EUTRA for inter-RAT measurements

- Kp_CSI-RS for CSI-RS L3 measurements

- Kp,PRS,i for NR positioning measurements

- CSSFintra for intra-frequency measurements

- CSSFinter for intra-frequency measurements

- CSSFinterRAT for intra-RAT measurements

- P scaling factor for L1-RSRP and L1-SINR measurements

Proposal 3: For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 4: For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 5: For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows:

- The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1

- Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 6: For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 7: For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 8: For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows:

- The duration of the window W is max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer

- Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 9: Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 10: Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 11: Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 12: For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows:

- The duration of the window W is max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

# Conclusions

Proposal 1: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured.

Proposal 2: The following parameters need to be updated to account for collisions with MUSIM gaps:

- Kp for intra-frequency and inter-frequency measurements without gaps

- Kgap for intra-frequency and inter-frequency measurements with gaps

- Kgap_EUTRA for inter-RAT measurements

- Kp_CSI-RS for CSI-RS L3 measurements

- Kp,PRS,i for NR positioning measurements

- CSSFintra for intra-frequency measurements

- CSSFinter for intra-frequency measurements

- CSSFinterRAT for intra-RAT measurements

- P scaling factor for L1-RSRP and L1-SINR measurements

Proposal 3: For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 4: For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 5: For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows:

- The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1

- Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 6: For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 7: For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 8: For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows:

- The duration of the window W is max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer

- Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 9: Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 10: Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 11: Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 12: For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows:

- The duration of the window W is max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

# References

[1] R4-2306357, WF on NR Dual TxRx Multi-SIM, RAN4#106bis-e