3GPP TSG-RAN WG4 Meeting#107                              R4-2309168

Incheon , KR, May 22 – May 26,2023

Source:	ZTE Corporation

Title:	TP for TS 38.876: Clause 7.1 and 7.1.1

Agenda Item:	8.14.2.1

Document for:	Approval

- Introduction

- This contribution provides a text proposal to TS38.876 [1] on sub-clause 7.1 and 7.1.1.

- Reference

- R4-230xxxx, TS 38.876 Study on Air-to-ground network for NR, CMCC, approved.

- Text Proposal

----- Start of TP -----

7	RF requirements

7.1	ATG CPE specific

7.1.1 ATG CPE power class and requirement type

Since the required power level for ATG CPE might be varying in different aircraft types and also in different frequency ranges, it’s quite difficult to focus on one specific power class for ATG CPE from 3GPP perspective, therefore 3GPP agreed to introduce the new capability for ATG UE to indicate [the minimum of maximum output power] at maximum modulation order and full PRB configurations and its capability report granularity as 1dB. The range of its power limit are defined as following:

- The lower limit of conductive MOP or TRP of ATG UE is 23dBm

- Upper limit of conductive MOP or TRP of ATG UE is  is 40dBm

In addition, considering the implementation freedom for ATG CPE (e.g. for 2GHz, to use omini antenna for ATG CPE and for 4GHz, to use antenna array for ATG CPE), therefore 3GPP RAN4 agreed to introduce two ATG CPE types to distinguish the antenna types for ATG CPE.

----- End of TP -----