3GPP TSG-RAN WG4 Meeting#107                              R4-2309170

Incheon , KR, May 22 – May 27,2023

Agenda item:	8.14.4.1

Source: 	ZTE Corporation

Title: 	Discussion on reply LS to applicability of SIB19 for NR ATG

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. In the previous RAN4 meeting, RAN4 send an LS to RAN2 for check the applicability of SBI19 for ATG system due to freq pre-compensation and timing pre-compensation in larger ISD scenario,based on the latest feedback from RAN2, more clarification questions are received from reply LS from RAN2. In this contribution, we want to share some further feedbacks.

Discussion

RAN2 would like to thank RAN4 for the LS on the applicability of the SIB19 for ATG. RAN2 understands the intention is that ATG BS provides location information to assist UE-based time and/or frequency pre-compensation.

RAN2 will address this and intends to find a solution for Rel-18 (via SIB19 or other SIB). Some companies in RAN2 think sharing exact base station’s location is sensitive. RAN2 kindly asks RAN4 to provide information on any required ATG ground station reference location accuracy (i.e., uncertainty in exact base station location).

Reply: ATG BS reference location accuracy could be 40m similar as ATG CPE GNSS accuracy. In addition, regarding the sensitivity of sharing the exact base station’s location for freq and timing pre-compensation at ATG CPE side. At least from our understanding, the security issues for sharing BS location information might not cause the security problem since the security issue usually refer to share the UE location information instead of BS location information. In addition, if there are still concerns on sharing the ATG BS location information, we think that there might have the following solutions to address it if necessary:

1) to reduce the deployment ISD of ATG BS in reality to avoid the necessity of timing pre-compensation and force the ATG CPE to the doppler shift estimation by other reference signals e.g. SSB or DMRS etc;

2) to hard-code the ATG BS location information on the flight route into the ATG CPE to reduce the risk to broadcast cell specific information SIB19;

3) request the RAN2 to change the cell specific SIB19 information into UE specific information;

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R2-2304565,Reply LS on applicability of SIB19 for NR ATG, Approved.

3GPP TSG-RAN WG4 Meeting#107                              R4-230xxxxx

Incheon , KR, May 22 – May 26, 2023

Title:	Reply LS to applicability of SIB19 for NR ATG

Response to:	R2-2304565

Release:	Rel-18

Work Item:	NR_ATG-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Fei, Xue

E-mail Address:  xue.fei25@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 would like to appreciate RAN2’s confirmation to address the SIB19 issue for ATG raised by RAN4. RAN4 also discussed the RAN2 questions in the response LS in R2-2304565 and the corresponding feedback are provided as following:

Question:

RAN2 would like to thank RAN4 for the LS on the applicability of the SIB19 for ATG. RAN2 understands the intention is that ATG BS provides location information to assist UE-based time and/or frequency pre-compensation.

RAN2 will address this and intends to find a solution for Rel-18 (via SIB19 or other SIB). Some companies in RAN2 think sharing exact base station’s location is sensitive. RAN2 kindly asks RAN4 to provide information on any required ATG ground station reference location accuracy (i.e., uncertainty in exact base station location).

Reply:

ATG BS reference location accuracy could be 40m similar as ATG CPE GNSS accuracy. In addition, regarding the sensitivity of sharing the exact base station’s location for freq and timing pre-compensation at ATG CPE side. At least from our understanding, the security issues for sharing BS location information might not cause the security problem since the security issue usually refer to share the UE location information instead of BS location information. In addition, if there are still concerns on sharing the ATG BS location information, we think that there might have the following solutions to address it if necessary:

1) to reduce the deployment ISD of ATG BS in reality to avoid the necessity of timing pre-compensation and force the ATG CPE to conduct the doppler shift estimation by other reference signals e.g. SSB or DMRS etc instead of based on SIB19 indication;

2) to hard-code the ATG BS location information on the flight route into the ATG CPE to reduce the risk to broadcast cell specific information SIB19;

3) request the RAN2 to change the cell specific SIB19 information into UE specific information;

# 2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take above RAN4 responses into consideration.

# 3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #108	21 - 25 Aug 2023             Toulouse , FR

TSG-RAN WG4 Meeting #108-bis	09 - 13 Oct 2023             Xiamen , CN