3GPP TSG-RAN WG4 Meeting#107                              R4-2309181

Incheon , KR, May 22 – May 26, 2023

Agenda item:	8.36.2

Source: 	ZTE Corporation

Title: 	Further discussion on NR based UAV

Document for:	Approval

Introduction

In RAN#99e meeting, the NR UAV work item was revised to [RP-230782] according to the incoming LS from ECC, the following RAN4 related objective are included for further discussion in RAN4. Therefore in this contribution, we want to share some further views on the NR UAV especially from its supported operating bands and its power class.

5. Based on the technical conditions defined for aerial UE usage in ECC Decision (22)07, study and specify the necessary UE types and additional OOBE requirements for aerial UEs in 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz. [RAN4]. Note: In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE Note2: Applicability of power classes for aerial UE may need to be addressed in RAN4 Note3: RAN4 to identify the supported bands for aerial UE impacted by above information

Discussion

First of all, based on the ECC decision 22(07) as following, ECC indicate the following uplink spectrum to be used for UAV UL transmission, however there are no clear indication where the DL spectrum should be. Based on the existing FR1 band definition in 3GPP specification, it seems that the spectrum mentioned in ECC decision could be mapped with the following bands highlighted in both green and yellow color. For bands highlighted in yellow, ECC specify the additional OOBE requirement, for bands highlighted in green, then the existing 3GPP OOBE limit could be apply.

Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710-1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570-2620 MHz harmonised for MFCN

- Proposal 1: to confirm whether operation bands for NR UAV is limited in the following bands in Table 1.

Table 1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |

- In addition, per operator’s input [8] as listed in the following table, band n41 could be also considered for UAV operation without any additional OOBE requirement. Since the band n38 is subset of n41, only partial spectrum for UAV operation in China could be covered by band n38. It’s understandable that the interested bands proposed by ECC are limited to the spectrum in EU, therefore the Mid-band is restricted to band n38 instead of band n41, however it should be known that this band n41 was widely used in other regions e.g. China, Japan, North America and Latin America, Middle East/Africa and India and South East Asia. Therefore from this perspective, it should be reasonable to add the band n41 for the UAV operation if possible especially considering the following coexistence study outcome from China State Radio Regulation Committee.

Table 2. Coexistence analysis from China State Radio Regulation Committee

| Frequency band | Interference service | Coexistence conclusion |
| --- | --- | --- |
| 1710-1735MHz | 1718.8-1722.2MHz radio astronomy 1690-1710MHz Meteorological satellite Earth BS | Be able to coexist |
| 1735-1765MHz | 1718.8-1722.2MHz radio astronomy 1690-1710MHz Meteorological satellite Earth BS | Be able to coexist |
| 1765-1785MHz | TDD private network system | Be able to coexist |
| 2515-2675MHz | 2483.5-2500MHz satellite system 2655-2700MHz radio astronomy 2700-2900MHz Radio navigation and positioning | Be able to coexist |

- Proposal 2: propose to consider the band 41 as one of operation bands for UAV without any additional OBUE requirement;

- In addition, there are no generic requirement discussion for NR UAV or LTE UAV in the past, therefore we need to discuss its generic RF requirement for UAV firstly. From our understanding, the UAV is quite different from existing the handheld UE or CPE device since NR UAV is supposed to support the beamforming capability similar ATG CPE. And for NR UAV, its antenna array size is supposed to be lower than that of ATG CPE (8x2x2 array size for ATG) and its output power is also expected lower than it due to its capability.

- In addition, as we discussed in the last RAN4 meeting, the applicable power class for UAV UE is not clear now, RAN4 need to further discuss this issue or send the LS to ECC to check the applicable power class from their coexistence study perspective. For instance, in some regions, the PC2 or PC1.5 might be not considered for UAV coexistence study with other existing systems. In addition, beamforming gain or antenna assumption could be also one limited factor for UAV coexistence study within the regulatory study. In short, we think that it is better to send LS to ECC or other regulatory body to check the coexistence assumptions for UAV operation.

<Agreement >: - RAN4 shall further discuss whether the existing UE power classes can be applied for UAVs or UAV specific power class(es) needs to be defined.

- Proposal 3: propose to send the LS for ECC and other regulatory body to check the coexistence assumption for UAV operation and whether check whether higher output power or antenna gain could be allowed or not for UAV operation.

Conclusions

In this contribution, we want to share some initial views on NR UAV .and proposals are made as following:

- Proposal 1: to confirm whether operation bands for NR UAV is limited in the following bands in Table 1.

Table 1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |

- Proposal 2: propose to consider the band 41 as one of operation bands for UAV without any additional OBUE requirement;

- Proposal 3: propose to send the LS for ECC and other regulatory body to check the coexistence assumption for UAV operation and whether check whether higher output power or antenna gain could be allowed or not for UAV operation.

References

[1] RP-230782, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles), Approved.

[2] ECC decision (22)07

[3] RP-230668,Discussion on UAV BRID/DAA Objective, Noted.

[4] R4-2306641,WF on NR support for UAV, Approved.

[5] R4-2306642,WF on back off study (MPR/A-MPR) for UAV, Approved.

[6] R4-2306643,Running Draft CR to TS 38.101-1 on additional OOBE requirements for aerial UEs,Endorsed.

[7] R4-2306644,Running Draft CR to TS 36.101 on additional OOBE requirements for aerial UEs,Endorsed.

[8]R4-2304211,Discussion on NR UAV requirements, CMCC, Noted