3GPP TSG-RAN WG4 Meeting#107                              R4-2309190

Incheon , KR, May 22 – May 26, 2023

Agenda item:	x.xx.x.x

Source: 	ZTE Corporation

Title: 	Discussion on RF requirements for NCR-MT

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In last RAN4 meeting, we have reached initial consensus for power control for NCR-MT, however there are lots of other issue left for further discussions. In this contribution, we want to share further analysis on RF requirement for NCR-MT.

Issue 3-1-1: Power control for NCR-MT - Option 1: to support the NCR-MT power control; - Agreement: Option 1 agreed Issue 3-1-2: bandwidth related - Proposal 1: Proposal 1: Define NCR-MT RF requirements based on carrier/NCR-MT channel bandwidth instead of passband/nominal channel bandwidth. [CATT, R4-2304446]

Discussion

## 2.1. RF requirements for NCR-MT transmitter

First of all, in the following table, we summarized the progress of discussion of NCR-MT Tx RF requirement so far, therefore in this following section, we aim to resolve the all the remaining issues for NCR-MT Tx part.

Table 2.1-1. the summary of the progress of NCR-MT RF requirement discussion

|  | Status/Progress |
| --- | --- |
| General for all Tx requirements | R4-2220255@RAN4#105 meeting - Agreement: For NCR-MT transmitter: - Baseline assumption: Legacy uplink transmitter requirements from Rel-17 RF repeater still be applicable - Further discuss the power control requirements for NCR-MT - Update on other requirements not precluded - For NCR Tx direction requirements, reuse the concept of OTA coverage range which also include OTA peak directions sets. |
| NCR-MT output power | R4-2302903： For NCR-MT, set requirements on TRP and EIRP accuracy using the BS approach (i.e., same as for IAB-MT) [Ericsson,R4-2301883] |
| NCR-MT output power dynamics | Ongoing, |
| NCR-MT power control | Ongoing, the power control of NCR-MT to be confirmed with RAN1 R4-2302903： Agreement: Further clarify WI scope on power control functionality for MT part. R4-2305931 Agreement - Option 1: to support the NCR-MT power control; |
| NCR-MT ON-OFF power | Ongoing |
| NCR-MT transition period | Not discussed yet. |
| NCR-MT frequency quality | R4-2302903： For the NCR-MT, set requirements on frequency accuracy and EVM using the IAB-MT specification as a reference. |
| NCR-MT unwanted emission | Not discussed yet. |
| NCR-MT transmitter intermodulation | Not discussed yet. |

### 2.1.1 Output power dynamics and power control

Power control requirement usually consist of generic absolute power tolerance, relative power tolerance if the transmission gap between these sub-frames is less than or equal to 20 ms and aggregate power tolerance within 21ms.

From our understanding, for the absolute power tolerance of NCR-MT first contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms, this mainly depends on the downlink RSRP measurement accuracy and its uplink power control accuracy from the implementation perspective, the total tolerance for absolute power tolerance in FR1 could be up to 9dB. However similar as beam correspondence discussion in the previous RAN4 meeting, the absolute power tolerance requirement could be indirectly tested by the power accuracy requirement of NCR-MT. From this perspective, it should be fine not to define the corresponding requirement for NCR-MT similar as Rel-16 IAB-MT.

Proposal 1: propose not to define the absolute power tolerance requirement for NCR-MT.

Regarding the relative power tolerance and aggregate power tolerance requirement, it was defined for both the normal UE and IAB-MT in the past, it might be reasonable to keep some alignment among related features, however it should be known that relative power tolerance for IAB-MT is quite limited due to the small total dynamic range requirement defined.

With the online clarifications during last Athens meeting, the limited output power dynamic requirement don’t prevent that NCR-MT achieve much better output dynamic capability, it’s okay for us to follow the either IAB-MT output power dynamic requirements or the legacy UE requirements and corresponding relative power tolerance and aggregated power tolerance requirements if necessary.

Proposal 2: for relative power tolerance and aggregated power tolerance requirements for NCR-MT, either to follow the output dynamic range requirement of IAB-MT or the legacy UE requirements and corresponding.

### 2.1.2 Transmitter ON-OFF power and transition period

For the transmitter ON-OFF power and transition period of NCR-MT, the existing BS requirement could be applicable here similar as Rel-16 IAB-MT agreement.

Proposal 3: for NCR-MT transmitter ON-OFF power and transition period requirement, to reuse the Rel-16 IAB-MT RF requirement as baseline.

### 2.1.3 Transmitter unwanted emission requirement

For the occupied channel bandwidth, since this is just to follow the ITU-R SM.328 regulation, the same principle should be also applied here fore NCR-MT.

Proposal 4: for NCR-MT occupied channel bandwidth, to follow the regulation ITU-R SM.328;

For ACLR requirement of NCR-MT, based on the previous agreement reached, the legacy uplink transmitter requirements from Rel-17 RF repeater still be applicable, however there are also other preference to reuse the IAB-MT requirement directly. In the following table, we summarized the ACLR requirement for both Rel-16 IAB-MT and Rel-17 repeater to figure out the difference among proposals. It could be found that IAB-MT ACLR requirement in Local area class are more stringent than that of Local area repeater UL, the main reason is due to the assumption of IAB-MT and IAB-DU aiming to reuse the same RF hardware as much as possible, therefore the ACLR capability of IAB-DU and IAB-MT could aligned somehow, however for NCR-MT, this device might be independent from NCR-Fwd downlink part, therefore we still propose to use Rel-17 repeater UL as baseline as we agreed before.

Proposal 5: for NCR-MT ACLR requirements, propose to follow Rel-17 repeater UL ACLR requirement without input signal of repeater UL in uplink direction.

|  | IAB-MT | Rel-17 repeater UL |
| --- | --- | --- |
| FR1 WA | 45dBc [6.6.3 in TS38.174] | 45dBc, 38dBc [6.5.2.2 in 38.106] |
| FR1 LA | 45dBc [6.6.3 in TS38.174] | 31dBc [6.5.2.2 in 38.106] |
| FR2 WA | 28, 26dBc [9.7.3.3 in TS38.174] | 28, 26dBc [7.5.2.2 in 38.106] |
| FR2 LA | 24dBc [9.7.3.4 in TS38.174] | 17, 16dBc [7.5.2.2 in 38.106] |

For OBUE requirements of NCR-MT, there are similar preference as ACLR requirement part, however indeed for both repeater UL or IAB-MT transmitter, both RAT are reusing the WA and LA BS OBUE requirements, therefore we don’t see much difference among these two preferences.

Proposal 6: for NCR-MT OBUE requirements, propose to follow Rel-17 repeater UL OBUE requirement without input signal of repeater UL in uplink direction.

For transmitter spurious emission requirements of NCR-MT, since these spurious emission requirement are either to following ITU-R regulation or based on deterministic calculation for the protections of receiver in other spectrum than DL in co-located scenario or in the same geographical scenario, therefore these legacy requirement defined for Rel-17 repeater UL or Rel-16 IAB-MT could be also reused for NCR-MT. In addition, it should be known that there are one difference between Rel-17 repeater UL and Rel-16 IAB-MT.

For Rel-16 IAB-MT, Cat A spurious emission requirement reusing from BS side are applicable for both IAB-DU and IAB-MT. However for Rel-17 repeater UL, Cat A spurious emission are reusing from Cat A requirement in UE specification instead of Cat A requirement in BS specification.

Based on the above analysis, to reusing the repeater UL transmitter spurious emission requirement for NCR-MT is more appropriate option.

Proposal 7: for NCR-MT transmitter spurious emission requirements, propose to follow Rel-17 repeater transmitter spurious emission requirements.

### 2.1.4 Transmitter transmitter intermodulation

For transmitter intermodulation requirements of NCR-MT, it might be not appropriate to follow the repeater output intermodulation requirement since the wanted signal for repeater output intermodulation requirement should fill up the whole pass-band as much as possible, however for NCR-MT might only occupy single carrier block or limited carriers if CA supported instead of the whole pass-band, therefore we believe that to follow the IAB-MT transmitter intermodulation requirement might be more appropriate option.

Proposal 8: for NCR-MT transmitter intermodulation requirement, propose to reuse the Rel-16 IAB-MT intermodulation requirements.

## 2.2. RF requirements for NCR-MT receiver

In the following table, we summarized the progress of discussion of NCR-MT Rx RF requirement so far, therefore in this following section, we aim to resolve the all the remaining issues for NCR-MT Rx part.

Table 2.2-1. the summary of the progress of NCR-MT RF requirement discussion

|  | Status/Progress |
| --- | --- |
| General for all Rx requirements | R4-2220255@RAN4#105 meeting - Agreement: - Proposal 1: For NCR-MT Rx direction requirements, it’s suggested to specify following OTA Rx direction declarations. These declarations apply for both input signal and interference signal. - A declared receiver target redirection range. This range is not needed if NCR don’t support redirection. - [five] declared sensitivity RoAoA comprising the conformance testing directions. If don’t support redirection, only one RoAoA is suggested - Receiver target reference direction - Min EIS may only need for MT part Proposal 3: for the RF requirement of NCR-MT receiver, to define the refense requirement for NCR-MT receiver. |
| NCR-MT reference sensitivity | R4-2302903： - To define the NCR-MT Refsens requirement based on IAB-MT approach; - To use the proposal 3 as baseline for calculation of power level of Refesens requirement; - Note: FFS for NF - For 30kHz and 60kHz FRC, to reuse the existing FRC from TS38.174 Annex A.1 - For 15kHz FRC, FFS |
| NCR-MT dynamic range | Not discussed yet |
| NCR-MT ACS | Ongoing |
| NCR-MT In-band blocking | Not discussed yet |
| NCR-MT Out of band blocking | Not discussed yet |
| NCR-MT receiver spurious emission | Not discussed yet |
| NCR-MT receiver intermodulation | Not discussed yet |
| NCR-MT in-channel selectivity | Not discussed yet |

### 2.2.1 REFSENS requirement

For REFSENS requirement of NCR-MT, it was agreed to follow IAB-MT approach in the past, however there are still lots of open issues left for further discussion.

Regarding the NF for NCR-MT, we propose to reuse the same NF as IAB-MT, the exact values could be found as following:

Proposal 9: for NF of IAB-MT, propose to follow the following values

- NF is assumed as 5dB for FR1 WA NCR-MT, and 13dB for FR1 LA NCR-MT;

- NF is assumed as 10dB for 30GHz and 12dB for 45GHz for FR2 NCR-MT

Regarding the FRC for 15kHz for NCR-MT, we propose to follow IAB-MT approach or BS approach, the details could be found as following.  To define 5MHz and 20MHz FRC with 15kHz and CP-OFDM symbols per slot is reduced from 12 to 9 due to the PDCCH transmission in the downlink.

Proposal 10: for 15kHz FRC of FR1 IAB-MT, propose to use the following FRC.

Table A1-1: FRC parameters for FR1 reference sensitivity level for IAB-MT.

| Reference channel | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-25 | G-FR1-A1-26 | G-FR1-A1-27 | G-FR1-A1-28 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 30 | 60 | 30 | 60 | 15 | 15 |
| Allocated resource blocks | 11 | 11 | 51 | 24 | 25 | 106 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |  |  |

### 2.2.2 Dynamic range requirement

For receiver dynamic range requirements, this requirement is only defined for BS or IAB-DU part and not defined for UE specification or IAB-MT in the past, therefore for NCR-MT, it’s reasonable to keep alignment with the legacy UE and IAB-MT RF requirement definition.

Proposal 11: propose not to define the dynamic range requirement for NCR-MT.

### 2.2.3 ACS and IBB requirement

In the previous meeting, in our contribution, we analyzed the combination of ACS requirement and ACRR requirement shown as following, however this scenario is not generic to practical scenarios indeed, therefore we are also fine to define the ACS and IBB requirement separately with NCR-Fwd link.

Figure 2.2.3-1. the illustration of ACRR requirement with NCR-MT Rx requirements

Figure 2.2.3-2. the illustration of ACRR requirement with NCR-MT Rx requirements

Similar as Tx ACLR requirement, whether to follow the UE specification or BS specification ACS requirement, this needs more discussions.

For Rel-16 IAB-MT ACS and IBB requirement, it was agreed to reuse the BS ACS and IBB requirement for it, however as we mentioned in unwanted emission requirement, this is not valid for NCR-MT part, therefore we propose to use the UE ACS requirement as 33dBc for FR1 NCR-MT and 23,22dBc for FR2 NCR-MT update the IAB-MT or BS requirement accordingly as following.

Proposal 12: propose to use the UE ACS requirement as 33dBc for FR1 NCR-MT and 23,22dBc for FR2 NCR-MT update the IAB-MT or BS requirement accordingly.

For IBB requirement and NBB requirements of FR1 NCR-MT, the interfering signal power level is usually 9dB higher than ACS interfering signal in FR1, the same principle could be also applied here for NCR-MT.

For IBB requirement for FR2 NCR-MT, keep the same power offset as IBB and ACS requirement of legacy FR2 IAB-MT or FR2 BS for FR2 NCR-MT.

Proposal 13:

For IBB requirement and NBB requirements of FR1 NCR-MT, IBB interfering signal power level should be 9dB higher than ACS requirement and NBB interfering signal power level should be 3dB higher than ACS requirement.

For IBB requirement for FR2 NCR-MT, keep the same power offset as IBB and ACS requirement of legacy FR2 IAB-MT or FR2 BS for FR2 NCR-MT.

### 2.2.4 OOBB requirement

For OOBB requirement for NCR-MT, we think that the existing IAB-MT requirement or BS approach requirement could be reused here even though we think that OOBB requirement could be further relaxed due to relaxed IBB requirement and the same OOBB requirement -15dBm/CW.

Proposal 14: for OOBB requirement for NCR-MT, propose to reuse the IAB-MT requirement.

### 2.2.5 Receiver spurious emission requirements

For receiver spurious emission requirement for NCR-MT, we think that the existing IAB-MT requirement or BS approach requirement could be reused.

Proposal 15: for receiver spurious emission requirement for NCR-MT, propose to reuse the IAB-MT requirement for it.

### 2.2.6 Receiver intermodulation requirements

For receiver intermodulation for NCR-MT, we think that the existing IAB-MT requirement or BS approach requirement could be reused, however the power level should be updated according to existing power offset between RX IMD and RX ACS requirements.

Proposal 16: for receiver intermodulation requirement for NCR-MT, propose to reuse the IAB-MT requirement and update the power level of interfering signal according to the existing power offset between RX IMD and RX ACS requirement.

### 2.2.7 Receiver in-channel selectivity requirement

For receiver in-channel selectivity requirements, this requirement is only defined for BS or IAB-DU part and not defined for UE specification or IAB-MT in the past, therefore for NCR-MT, it’s reasonable to keep alignment with the legacy UE and IAB-MT RF requirement definition.

Proposal 17: propose not to define the ICS requirement for NCR-MT.

Conclusions

In this contribution, we want to share further views on NCR-MT RF requirements and proposals are made as following:

Proposal 1: propose not to define the absolute power tolerance requirement for NCR-MT.

Proposal 2: for relative power tolerance and aggregated power tolerance requirements for NCR-MT, either to follow the output dynamic range requirement of IAB-MT or the legacy UE requirements and corresponding.

Proposal 3: for NCR-MT transmitter ON-OFF power and transition period requirement, to reuse the Rel-16 IAB-MT RF requirement as baseline.

Proposal 4: for NCR-MT occupied channel bandwidth, to follow the regulation ITU-R SM.328;

Proposal 5: for NCR-MT ACLR requirements, propose to follow Rel-17 repeater UL ACLR requirement without input signal of repeater UL in uplink direction.

Proposal 6: for NCR-MT OBUE requirements, propose to follow Rel-17 repeater UL OBUE requirement without input signal of repeater UL in uplink direction.

Proposal 7: for NCR-MT transmitter spurious emission requirements, propose to follow Rel-17 repeater transmitter spurious emission requirements.

Proposal 8: for NCR-MT transmitter intermodulation requirement, propose to reuse the Rel-16 IAB-MT intermodulation requirements.

Proposal 9: for NF of IAB-MT, propose to follow the following values

- NF is assumed as 5dB for FR1 WA NCR-MT, and 13dB for FR1 LA NCR-MT;

- NF is assumed as 10dB for 30GHz and 12dB for 45GHz for FR2 NCR-MT

Proposal 10: for 15kHz FRC of FR1 IAB-MT, propose to use the following FRC.

Table A1-1: FRC parameters for FR1 reference sensitivity level for IAB-MT.

| Reference channel | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-25 | G-FR1-A1-26 | G-FR1-A1-27 | G-FR1-A1-28 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 30 | 60 | 30 | 60 | 15 | 15 |
| Allocated resource blocks | 11 | 11 | 51 | 24 | 25 | 106 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |  |  |

Proposal 11: propose not to define the dynamic range requirement for NCR-MT.

Proposal 12: propose to use the UE ACS requirement as 33dBc for FR1 NCR-MT and 23,22dBc for FR2 NCR-MT update the IAB-MT or BS requirement accordingly.

Proposal 13:

For IBB requirement and NBB requirements of FR1 NCR-MT, IBB interfering signal power level should be 9dB higher than ACS requirement and NBB interfering signal power level should be 3dB higher than ACS requirement.

For IBB requirement for FR2 NCR-MT, keep the same power offset as IBB and ACS requirement of legacy FR2 IAB-MT or FR2 BS for FR2 NCR-MT.

Proposal 14: for OOBB requirement for NCR-MT, propose to reuse the IAB-MT requirement.

Proposal 15: for receiver spurious emission requirement for NCR-MT, propose to reuse the IAB-MT requirement for it.

Proposal 16: for receiver intermodulation requirement for NCR-MT, propose to reuse the IAB-MT requirement and update the power level of interfering signal according to the existing power offset between RX IMD and RX ACS requirement.

Proposal 17: propose not to define the ICS requirement for NCR-MT.

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- TS 38.106, NR Repeater Radio Transmission and Reception; (Release 17)

- R4-2207290, WF on repeater RF requirements, Huawei, approved.

- R4-2205030,Repeater radiated power requirements, Ericsson, Noted

- TS 38.115-2,NR;Repeater conformance testing, Part 2: Radiated conformance testing.

- TS 38.115-1,NR;Repeater conformance testing, Part 1: Conducted conformance testing.

- R4-2217263,WF on RRM requirements for NCR, ZTE, approved.

- R4-2217471,WF on NCR RF requirements, ZTE, approved.

- TR 38.867,Study on NR Network-controlled Repeaters

- R4-2217512 ,Post_2nd_round_summary_[104bis-e][314]NR_netcon_repeater, ZTE

- R4-2217506,LS to RAN1 on NCR-MT transmission and Beam correspondence, CMCC, approved.

- R4-2215488, discussion on NCR RF requirements, CMCC, Noted

- R4-2220140 Summary for [105][314] NR_netcon_repeater.

- R4-2303759, Topic summary for [106][314] NR_netcon_repeater, Noted.

- R4-2305931 WF on system parameters and RF requirements for NCR_final,ZTE, approved.