3GPP TSG-RAN WG4 Meeting#107                             R4-2309191

Incheon , KR, May 22 – May 26, 2023

Agenda item:	5.29.4

Source: 	ZTE Corporation

Title: 	Discussion on conformance testing requirement for NCR

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In last RAN4 meeting, we have some initial discussions for NCR conformance testing and we reached on agreement in WF [14]. In this contribution, we want to share some further views on conformance testing requirement for NCR in Rel-18.

Discussion

## 2.1. Test model and test configuration for NCR-MT and NCR-Fwd

Regarding the test model and test configuration for NCR-MT, we think that existing Rel-16 IAB-MT test model and test configuration could be used as baseline, however this also highly depends on the core requirement for NCR-MT. In the last RAN4 meeting, from system parameter for NCR-MT and it should refer to the UE specification also take IAB-MT approach for capability report for the supported bandwidth. Based on the agreement reached so far, we propose to Rel-16 IAB-MT as starting point.

- Agreement: - For the system parameter of NCR-MT, refer to the UE specification (TS38.101-1/2) - Take IAB-MT approach as starting point i.e. to set the channel bandwidth per operating band to be optional for NCR-MT

Proposal 1: for test model for NCR-MT, propose to use the Rel-16 IAB-MT test model as baseline.

## 2.2. Test setup

In the last RAN4 meeting, there were some initial discussions for measurement setup for NCR-Fwd part and agreement are reached as following.

Issue 1-7: Measurement system set-up WF: For NCR-Fwd, the following is agreed as the starting point. For NCR-Fwd, use the Rel-17 repeater conformance testing setup as baseline and further discuss the changes if any.

For NCR-Fwd type 1-O, since we need to consider the co-location requirements for input intermodulation requirement and output intermodulation requirements, we propose to have the following measurement setup for it.

Proposal 2: for the measurement setup of Input IMD and output IMD of NCR-Fwd type 1-O, propose to consider the following measurement setup for it.

Figure 1. measurement setup for Input IMD requirement of FR1 NCR-Fwd type 1-O

Figure 2. measurement setup for output IMD requirement of FR1 NCR-Fwd type 1-O

In addition, regarding the measurement setup for NCR-MT, especially when consider the NCR-MT and NCR-Fwd are tested separately, then the existing measurement setup for Rel-16 IAB-MT could be used as baseline for NCR-MT measurement setup. .

Proposal 3: propose to use the existing measurement setup for Rel-16 IAB-MT as baseline for NCR-MT measurement setup and further consider the joint conformance testing setup for emission related requirement if necessary.

Conclusions

In this contribution, we want to share some further views on the conformance testing requirement for NCR proposals are made as following:

Proposal 1: for test model for NCR-MT, propose to use the Rel-16 IAB-MT test model as baseline.

Proposal 2: for the measurement setup of Input IMD and output IMD of NCR-Fwd type 1-O, propose to consider the following measurement setup for it.

Figure 1. measurement setup for Input IMD requirement of FR1 NCR-Fwd type 1-O

Figure 2. measurement setup for output IMD requirement of FR1 NCR-Fwd type 1-O

Proposal 3: propose to use the existing measurement setup for Rel-16 IAB-MT as baseline for NCR-MT measurement setup and further consider the joint conformance testing setup for emission related requirement if necessary.

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- TS 38.106, NR Repeater Radio Transmission and Reception; (Release 17)

- R4-2207290, WF on repeater RF requirements, Huawei, approved.

- R4-2205030,Repeater radiated power requirements, Ericsson, Noted

- TS 38.115-2,NR;Repeater conformance testing, Part 2: Radiated conformance testing.

- TS 38.115-1,NR;Repeater conformance testing, Part 1: Conducted conformance testing.

- R4-2217263,WF on RRM requirements for NCR, ZTE, approved.

- R4-2217471,WF on NCR RF requirements, ZTE, approved.

- TR 38.867,Study on NR Network-controlled Repeaters

- R4-2217512 ,Post_2nd_round_summary_[104bis-e][314]NR_netcon_repeater, ZTE

- R4-2217506,LS to RAN1 on NCR-MT transmission and Beam correspondence, CMCC, approved.

- R4-2215488, discussion on NCR RF requirements, CMCC, Noted

- R4-2220140 Summary for [105][314] NR_netcon_repeater.

- R4-2305933,WF on NR NCR RF conformance testing, CATT, approved.